3GPP TSG-RAN WG2 Meeting #128	R2-2410411

Orlando, USA 18.11.2024-22.11.2024

Source: 	Vodafone, Deutsche Telekom, Xiaomi

Title: 		Barring and SIB1-less case 2

Document for: Decision

Agenda Item: 8.5.3

Introduction

During the last 3GPP TSG RAN Meeting #105 the updated scope for SIB1-less operation was approved:

During RAN WG 127bis meeting, Vodafone presented the T-doc R2-2408454 suggesting to clarify barring behavior in case the UE during PLMN search try to camp on NES/SIB1 less frequency without having UL-WUS configuration to request SIB1 and that this behaviour might lead to problems if no clarification is provided.

Some companies anyhow argued that even the scenario is valid, there is no need to clarify anything in the specification as it can be solved by implementation.

Discussion

The UE selects the SIB1-less cell, it can bar the cell and select Cell A which provides a valid WUS configuration. With this configuration, the UE will try to reselect to the SIB1-less cell.

If reselection from Cell A to SIB1-less cell happens within 300 seconds after SIB1-less cell was declared as barred, the SIB1 cell is still barred and reselection will not work. More likely the UE will not even try to reselect to such a cell, even it has a valid WUS configuration

Please see the following behaviour described in 38.304

Observation 1: Barring behaviour as of today would lead to not availability of reselection to SIB1-less cell within 300 seconds and reduce the usefulness of case 2. On top of that it might lead to the additional battery consumption from the UE as the UE can only understand PCI once it is synchronized to the cell.

Some companies argued in RAN WG127bis meeting, that by not changing the specification, the word “may” allows the UE to not bar the cell in this case and also does not need to exclude it from  reselection candidates and so the problem would not exist.

Although true, it does not solve the problem as the standard still allows implementations which might lead to a problem (UE bars the cell and exclude as a selection/reselection candidates for 300 seconds) and therefore the solution is still needed.

The easiest way to solve this problem is to introduce a clarification that once the UE receives a valid UL-WUS configuration and if this cell is barred and/or excluded from the cell selection/reselection candidates, the UE should unbar this cell.

Proposal: It is proposed to clarify that if a UE bars the NES/SIB1 less cell and/or excludes it as a candidate for reselection, the UE would treat this cell as if cell status is “not barred” and consider it as candidate for cell reselection once it received a UL-WUS configuration to request SIB1 for this cell.

Conclusions

Proposal: It is proposed to clarify that if a UE bars the NES/SIB1 less cell and/or excludes it as a candidate for reselection, the UE would treat this cell as if cell status is “not barred” and consider it as candidate for cell reselection once it has received a UL-WUS configuration to request SIB1 for this cell.

References:

# RP-242354: Revised WID: Enhancements of network energy savings for NR

R2-2408454: Barring and SIB1-less case 2