3GPP TSG-RAN WG2 Meeting #128	R2-2410423

Orlando, USA, 18 – 22 November 2024

Agenda item:	8.2.2

Source:	Nokia

Title:	Functionality aspects of AIoT

WID/SID:	FS_Ambient_IoT_solutions - Release 19

Document for:	Discussion and Decision

# 1	Introduction

This contribution discusses AIoT functionality aspects as well as general aspects of the SI status and closure.

# 2	Discussion

## 2.1	Capability Reporting

Given the absence of SDAP and RRC layer as well as the need for simplicity and minimal memory access within AIoT devices, it is sensible to assume that device will not be characterized by substantial capability differences other than the ones stemming from their general type (device 1, 2a or 2b).

To simplify and streamline the AIoT air interface design as well as the device implementation and signaling, it is therefore plausible not to differentiate the signalling between devices and avoid selective configuration based on their individual capabilities. In other words, device capabilities would not be reported to the reader for its further consideration.

However, as RAN1 has not yet concluded the discussion on device design and physical layer specification, some exceptions to this approach may arise in the future. Further study will identify any such exceptions and define the necessary mechanisms for handling them.

Proposal 1: RAN2 assumes no direct capability indication between device and reader. FFS other WGs conclusions.

## 2.2	Energy Status Reporting

For AIoT devices, efficient utilization of their harvested energy is crucial. Since the device has effectively no control over the requested services and the associated transmissions and other energy expenditures, it is necessary for the network to accurately account for these. The harvested energy changes over the time in the device and also as a non-linear function of its transmission activities. In other words, the ability to support service requests from the networks change over time and possibly unpredictably.

Consequently, the network (readers) must strive towards efficient usage of currently available energy of the AIoT to ensure successful completion of service requests. For example, the network must ensure that an AIoT device remains available for receiving configuration and/or command data in a multi-step inventory & command procedure, or when a sequence of segmented payload transmissions is needed.

In general, am AIoT device is characterized by

- the general ability to acquire energy (pure CW-powered device 1 vs active devices 2)

- current energy availability (charge in capacitor / battery)

As for the first aspect of energy acquisition ability, the study item description envisages multiple device types, starting from the low-cost and low-complexity type 1 devices that rely purely on CW for their energy usage, up to more complex active device types 2a and 2b that can either amplify their signals or even take advantage of a battery charge.

This device categorization will have to be reflected also during AIoT operation. Simple devices such as type 1 will for example not be able to easily perform large energy shift required for FDD operation. As discussed in [1], the widely used CMOS circuitry drains energy proportionally to the operating frequency and so double-digit MHz offsets may imply already substantially energy usage.

An example of a device-specific RAN1 agreement is as follows:

RAN1 Agreement Make the following update for device 2a diagram. - Large Frequency shifter (e.g., tens of MHz) for shifting backscattered signal from one frequency (e.g., FDD-DL frequency) to another frequency (e.g., FDD-UL frequency). - Note: this does not mean that RAN1 has concluded on the feasibility of Large Frequency shifter for device 2a

Observation 1: Devices 1 may not be able to support major frequency shifts, e.g. for operation in paired FDD spectrum.

Therefore, it is proposed that the device type is signalled to reader to ensure that such operational constraints can be considered. The device type does not necessarily need to be signalled directly by the device to the reader via an explicit D2R signalling (e.g., as part of paging response). Indirect AF/CN-driven signalling, derived from information exchange during device registration, may be sufficient.

Proposal 2: Device type is indicated to the reader. FFS signalling options via CN to avoid explicit device-reader signalling overhead over the AIoT interface.

As for the second aspect, the reporting of current energy status is relevant for determining whether a device will be able to sustain ongoing transmission activity, e.g. segmented, repetitive or long transmissions.

Therefore, it is seen important that devices have the capability to indicate their (low) energy status so that the network can reflect these For example, if low energy status is indicated and then a transmission (sequence) is interrupted due to zero charge, the network can determine that the device is active and operational, but just needs some time to recharge in order to complete the service request, as opposed to zero response and the network trying to re-engage immediately and repetitively with irresponsive device and failing the service requests (as well as possibly reporting wrongly device inactivity / absence).

Observation 2: Reporting of low energy status allows the network to distinguish active / engaged devices with need to re-charge from inactive / absent devices during communication failures and select appropriate response (e.g., wait for recharge instead of immediate re-transmission).

RAN2 already agree optional support of energy status during the RAN2#127-bis meeting.

RAN2 Agreement Capture in the TR the option: the device may include energy status indication in D2R messages (e.g. MSG1,MSG3 and Command Response message), if the device can. 1 bit indication can be captured in the TR. FFS whether it is reader controlled.

Clearly, energy status reporting is not necessary for once-off transmissions of small payload such as in the inventory use cases, it is meaningful only when long / repetitive / segmented transmissions are anticipated. In this case, the devices should support energy status reporting.

Proposal 3: Devices support reporting of energy status

- reactively upon reader request (e.g. in “inventory and command” scenarios),

- proactively by device during transmissions of segmented / repetitive / large payload.

The most natural approach to delivering the energy status report is within the Msg 1. Other message formats are possible, ranging from other “integrated” RACH messages until stand-alone MAC CE.

Proposal 4: Msg 1 contains the energy status bit for all device types. FFS other formats.

## 2.3	Buffer Status Reporting

The core network, including the AIoT function and potentially also the gNB, controls all devices participating in an AIoT session (i.e., activating an AIoT device and reading its reply). In this context, the knowledge of the amount of data to be exchanged between the reader and a device is naturally of interest when managing payload transmissions.

While in specific AIoT use cases such as inventory polling, the application data to be delivered to the reader is typically constant (or produced in discrete steps sparce in time, or requested by the reader in the read command, such that the payload most likely does not change after a paging request), the general inventory & command use case may be characterized by variable data sizes. Moreover, these variable payloads may be large enough to imply multiple subsequent transmissions, be it in repetitive, sequential or segmented form.

Especially in the latter case, reporting the amount of application data to be sent might be essential. An indication of large data volume / multiple transmissions may trigger selective resource allocation and adaptation of access type in terms of medium access contention.

Observation 3: Device data may be of variable size, especially in command or sensing use cases.

Observation 4: Short payloads may be delivered in a single transmission, but a report on application data buffer status may be needed for scheduling long / repetitive / segmented transmissions.

Several options for BSR-like reporting are possible:

- BSR-like information may be indicated by the device during an active (e.g. initial) data transmission. The indication can be either proactive (triggered by the device) or reactive (if requested by the reader).

- BSR-like information can be polled by the reader any time, even when there is no ongoing data exchange between the device and the reader that could be used for BSR-like delivery. For example, specific MAC CE-like functionality could be envisaged.

- Some AIoT device classes may not support BSR-like information. In such cases, the reader could be provided with payload data details (e.g., expected payload size) during the application-layer request by the Application Function (AF).

Proposal 5: RAN2 to choose from the following options for BSR-like reporting:

- Option 1: 	BSR-like information is indicated to reader only during active (e.g. initial) data transmission.

- Option 2: 	BSR-like information may be polled by the reader independently of any payload transmission (e.g. via a dedicated MAC-CE like mechanism).

- Option 3: 	BSR-like information is provided to the reader by the AF / CN as part of a service request.

- Option 4:	No BSR-like information is given to reader.

If applicable, FFS single “data available” bit is used, or multi-bit data size representation is use.

## 2.4	Segmentation Signaling

Based on the RAN2#127-bis agreements, the AIoT segmentation functionality will not support sequence numbers, segment numbers, or the total number of segments yet further details of a comprehensive approach to segmentation and acknowledgement feedback in both D2R (Device to Reader) and R2D (Reader to Device) communications were not agreed yet.

The absence of sequence / segment numbers effectively rules out selective acknowledgements and go-back-N schemes known from ARQ. Only a simple wait-and-stop scheme with “anonymous” data delivery is possible. This approach however requires mandatory acknowledgement feedback to ensure data integrity.

Proposal 6: D2R segmentation operates on wait-and-stop basis in D2R with mandatory acknowledgement feedback. FFS feedback details.

At the same time, R2D segmentation should not precluded. The motivation is twofold: to ensure forward compatibility as well as to account for scenarios with weak signal strength that necessitates smaller transport block sizes and thus potentially segmentation in the R2D direction (e.g., for downlink command binaries or data inputs).

If R2D segmentation is used, it should however adhere to the same design principle as used for D2R segmentation.

Proposal 7: R2D segmentation should not be precluded in the study. If used, it shall reuse the D2R design principles.

In either case, explicit acknowledgements are a mandatory part of the wait-and-stop scheme to ensure correct delivery and prevent data loss related errors. These acknowledgements also enable the safe deletion of buffered data in the device, ensuring efficient memory management and data handling.

Proposal 8: Support mandatory/explicit acknowledgement feedback for any service request involving multiple D2R transmission, i.e. at least “inventory and command”.

## 3				Study Item Status

## 3.1	Completion Aspects

RAN2#128 is the last meeting of the SI wherein the TR will be submitted to RAN plenary. We understand that the RAN2 study has progressed well, but several issues still remain open including pending LSs to other WGs. In order to close the SI, all such issues should be captured in the TR.

Proposal 9: RAN2 can declare the AIoT study item complete but the final TR should capture:

- All open RAN2 issues after RAN2#128.

- Any pending feedback from other WGs impacting RAN2 including

  - RAN1 conclusions on FDMA,

  - SA2 conclusion on max. data block size delivered from upper layers to AIoT layers,

  - SA3 security procedures and impact on RAN2 architecture such as the need for R2D segmentation.

# 4	Conclusion

Observation 1: Devices 1 may not be able to support major frequency shifts, e.g. for operation in paired FDD spectrum.

Observation 2: Reporting of low energy status allows the network to distinguish active / engaged devices with need to re-charge from inactive / absent devices during communication failures and select appropriate response (e.g., wait for recharge instead of immediate re-transmission).

Observation 3: Device data may be of variable size, especially in command or sensing use cases.

Observation 4: Short payloads may be delivered in a single transmission, but a report on application data buffer status may be needed for scheduling long / repetitive / segmented transmissions.

Proposal 1: RAN2 assumes no direct capability indication between device and reader. FFS other WGs conclusions.

Proposal 2: Device type is indicated to the reader. FFS signalling options via CN to avoid explicit device-reader signalling overhead over the AIoT interface.

Proposal 3: Devices support reporting of energy status

- reactively upon reader request (e.g. in “inventory and command” scenarios),

- proactively by device during transmissions of segmented / repetitive / large payload.

Proposal 4: Msg 1 contains the energy status bit for all device types. FFS other formats.

Proposal 5: RAN2 to choose from the following options for BSR-like reporting:

- Option 1: 	BSR-like information is indicated to reader only during active (e.g. initial) data transmission.

- Option 2: 	BSR-like information may be polled by the reader independently of any payload transmission (e.g. via a dedicated MAC-CE like mechanism).

- Option 3: 	BSR-like information is provided to the reader by the AF / CN as part of a service request.

- Option 4:	No BSR-like information is given to reader.

If applicable, FFS single “data available” bit is used, or multi-bit data size representation is use.

Proposal 6: D2R segmentation operates on wait-and-stop basis in D2R with mandatory acknowledgement feedback. FFS feedback details.

Proposal 7: R2D segmentation should not be precluded in the study. If used, it shall reuse the D2R design principles.

Proposal 8: Support mandatory/explicit acknowledgement feedback for any service request involving multiple D2R transmission, i.e. at least “inventory and command”.

Proposal 9: RAN2 can declare the AIoT study item complete but the final TR should capture:

- All open RAN2 issues after RAN2#128.

- Any pending feedback from other WGs impacting RAN2 including

  - RAN1 conclusions on FDMA,

  - SA2 conclusion on max. data block size delivered from upper layers to AIoT layers,

  - SA3 security procedures and impact on RAN2 architecture such as the need for R2D segmentation.

# 4	References

- R1-2407276, RAN1#118 FL Summary Final for 9.4.1.2 Ambient IoT Device Architecture