3GPP TSG-RAN WG2 Meeting #128	R2-2410432

Orlando, US, 18 – 22 Nov, 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on on-demand SSB SCell operation for NES

Agenda Item:	8.5.2

Document for:	Discussion and decision

Introduction

In this contribution, we discuss the remaining issues of on-demand SSB SCell operation, including MAC CE design and L3 measurements.

Discussion

2.1 MAC CE design

Both RAN1 and RAN2 have agreed to introduce a new MAC CE for indication OD-SSB transmission. RAN1 has further agreed to include SSB periodicity in the MAC CE.

Agreement For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - Periodicity of the on-demand SSB. - FFS: Any other relevant parameters.

In RAN2#127bis, RAN2 has agreed that the new MAC CE is not used for SCell activation/deactivation, and that the MAC CE should be able to indicate the OD-SSB transmission for multiple SCells.

Agreements on OD-SSB SCell

- No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.

- Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A.

- NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

On MAC CE design, there are several issues to be discussed:

Issue 1: Which SCells to be included? Only the SCells supporting OD-SSB or all the SCells?

Issue 2: How to identify an SCell, using explicit SCell indexes or the SCells are listed in an implicit order?

Issue 3: How to indicate SSB periodicity?

To save signalling overhead, we think only the SCells supporting OD-SSB should be included in the new MAC CE for OD-SSB transmission. Regarding how to inform the UEs of which SCells supporting OD-SSB, we think it can be indicated in the SCell configuration, or implicitly indicated by the initial status of OD-SSB transmission for SCell (regardless of whether the initial status of OD-SSB is transmitted or not transmitted, the presence of the field indicates the SCell supports OD-SSB).

Proposal 1: Only the SCells supporting OD-SSB will be included in the OD-SSB transmission MAC CE.

The Issue 2 above implies two different ways of MAC CE design. Considering RAN2 has agreed that the new MAC CE should be able to indicate the OD-SSB transmission for multiple SCells, the new MAC CE is of variable size. If we use the explicit serving cell ID, each serving cell ID will take up 5 bits, and is less efficient compared with implicit SCell index. Therefore, we prefer to use implicit SCell index in the MAC CE.

On Issue 3, the existing SSB periodicity contains 6 values (5ms, 10ms, 20ms, 40ms, 80ms, 160ms), and 3 bits can be used to indicate the SSB periodicity.

In the MAC CE design, we include some reserved bit in case other parameters are agreed to be included in the MAC CE (e.g. gaps between bursts, time instance A and time instance B, etc)

Example: MAC CE includes implicit SCell index and SSB periodicities (variable size)

RRC signalling can be used to configure an additional index for the SCells supporting OD-SSB (i.e. enabling the OD-SSB feature from UE perspective), and the assumption is that there will not be more than 8 SCells supporting OD-SSB simultaneously. To be specific, a new field od-SSB-SCellIndex can be added to SCellConfig with the value range “INTEGER (1..8)”. And the MAC CE can be designed as follows:

-	Oi: This field indicates the activation/deactivation status of the on-demand SSB transmission of the SCell with Od-SSB-SCellIndex i as specified in TS 38.331 [5]. The Oi field is set to 1 to indicate that the on-demand SSB transmission of the corresponding SCell is activated. The Oi field is set to 0 to indicate that the on-demand SSB transmission of the corresponding SCell is deactivated;

-	Periodicity: This field indicates the periodicity of the on-demand SSB of the SCell for which on-demand SSB transmission is activated. The length of the field is 3 bits. The Periodicy fields are included in ascending order based on the Oi set to 1.

Figure 1: On-demand SSB indication MAC CE

Table 1: SSB periodicity for On-demand SSB indication MAC CE

| Periodicity value | On-demand SSB periodicity |
| --- | --- |
| 000 | 5 ms |
| 001 | 10 ms |
| 010 | 20 ms |
| 011 | 40 ms |
| 100 | 80 ms |
| 101 | 160 ms |
| 110 | Spare |
| 111 | Spare |

Proposal 2: RRC indicates whether an SCell supports OD-SSB when configuring the SCell, MAC CE indicates the activation/deactivation status sequentially, and indicates SSB periodicity for the SCells with OD-SSB activated (variable size MAC CE).

For Case #1 (no always-on SSB), the above design is straightforward. For Case #2 (always-on SSB), the above design is based on the understanding that Case #2 means having a periodic SSB transmission as a baseline and using on-demand SSB for triggering additional SSB transmission, and MAC CE is only used to activate and indicate the SSB periodicity of the OD-SSB, it has nothing to do with the always-on SSB.

Proposal 3: For Case #2 (always-on SSB), the new MAC CE is only used to activate and indicate periodicity of OD-SSB, it is not used to modify/activate/deactivate the always-on SSB.

2.2 L3 measurements

There are several open issues on L3 measurements:

Issue 1: Whether OD-SSB (on-demand SSB) is on the same frequency with AO-SSB (always-on SSB) or not

Issue 2: Whether OD-SSB measurement is based on SMTC or OD-SSB pattern

Issue 3: For Case #2 (AO-SSB), whether AO-SSB is also used to generate serving cell measurement results

On Issue 1, RAN4 has discussed the issue and identified the following scenarios:

Agreement: - In OD-SSB Case 2, RAN4 to first discuss the deactivated SCell requirement for Scenario 1. RAN4 to further discuss whether to define the requirement for the scenario 2 and 3 if scenario 2 and 3 are supported in RAN1 based on RAN1’s agreement. - Scenario 1: OD-SSB and always-on SSB are with same SSB carrier frequency, same offset but different periodicities. - Scenario 2: OD-SSB and always-on SSB are with same SSB carrier frequency, different offsets and periodicities. - Scenario 3: OD-SSB and always-on SSB are within different SSB carrier frequencies. - Send the above information to RAN1, further discuss the text including the questions to RAN1 in the LS. - RAN4 will inform RAN1 if any further agreement.

The final decision on whether OD-SSB and AO-SSB can have different frequency (Scenario 3) is up to RAN1, and RAN4 discussion will start from Scenario 1.

From RAN2 perspective, we would also like to prioritize Scenario 1, because Scenario 3 would bring considerable RAN2 impacts:

1) Impacts on servingCellMO configuration: In existing spec, each serving cell can be configured with a servingCellMO for deriving serving cell measurements, and only one SSB frequency is configured per MO. If both OD-SSB and AO-SSB are used for serving cell measurements, either two servingCellMOs are considered, or two SSB frequencies are configured in the single servingCellMO. Also, the serving cell CSI-RS measurement is based on an associated SSB, it is also unclear whether it is based on OD-SSB or AO-SSB.

2) Impacts on neighbour cell measurements: In existing spec, whether an SSB based measurement is considered as intra-frequency or inter-frequency measurement depends on whether the neighbour cell SSB is of the same frequency with the SSB of serving cell, and whether the SCS is the same as serving cell SSB. If both OD-SSB and AO-SSB are configured for serving cell measurements, RAN2 needs to clarify whether OD-SSB or AO-SSB will be used as baseline to identify intra-frequency/inter-frequency measurements.

3) Impacts on gap configuration: In existing spec, other than the cases where UE reports gap requirement (e.g. via NeedForGapsInfoNR reporting), if any of the configured BWPs do not contain serving cell SSB (which refers to the SSB of servingCellMO) and are not configured with NCD-SSB for serving cell measurement (which was introduced in RedCap), the network should configure gaps to the UE for intra-frequency measurements. However, if the SCell can be configured with AO-SSB and an inter-frequency OD-SSB, it is unclear whether gaps are needed if a BWP contains AO-SSB but not OD-SSB.

- For SSB based intra-frequency measurement, if the measurement gap requirement information is reported by the UE, a measurement gap configuration may be provided according to the information. Otherwise, a measurement gap configuration is always provided in the following case: - Other than the initial BWP, if any of the UE configured BWPs do not contain the frequency domain resources of the SSB associated to the initial DL BWP, and are not configured with NCD-SSB for serving cell measurement.

Proposal 4: RAN2 starts discussion on L3 measurement based on Scenario 1 (OD-SSB and always-on SSB are with same SSB carrier frequency, same offset but different periodicities).

Proposal 5: If Scenario 3 (OD-SSB and always-on SSB are within different SSB carrier frequencies) is considered, the following issues needs to be addressed:

- Impacts on servingCellMO configuration (whether both OD-SSB and always-on SSB frequencies are configured)

- Impacts on neighbour cell measurements (whether intra-frequency/inter-frequency determination can be based on OD-SSB)

- Impacts on gap configuration for intra-frequency measurements (whether gaps are needed if a BWP contains always-on SSB but not OD-SSB)

On Issue 2 (Whether OD-SSB measurement is based on SMTC or OD-SSB pattern):

In legacy mechanism, L3 measurement is based on the measConfig, which uses measIds to link measObjects (which provides SMTC for UE to monitor SSB) and reportConfigs, and UE measurement reporting is also based on measIds. For OD-SSB, at least the OD-SSB pattern of the serving cell will be informed to the UE. Therefore, RAN2 needs to determine whether the L3 measurement is still based on SMTC or it is based on the OD-SSB pattern or both.

For both Case #1 (no always-on SSB) and Case #2 (always-on SSB), as long as the OD-SSB pattern is informed to the UE, UE can perform serving cell measurements. There is no need to modify the legacy SMTC or make up a new dedicated SMTC for serving cell measurement purpose. If the specification forces the NW implementation to configure an SMTC for OD-SSB measurements, this configuration is anyway aligned with the OD-SSB pattern, which is already known to the UE, and an unnecessary step is defined at the NW side.

Proposal 6: For both Case #1 (no always-on SSB) and Case #2 (always-on SSB), the OD-SSB pattern is informed to the UE for performing L3 serving cell measurements. No need to modify the legacy SMTC or make up a dedicated SMTC for serving cell measurements.

Another question is, for Case #2 (always-on SSB), there is always-on SSB within the legacy SMTC, it is unclear whether the UE should ignore these SSBs or the UE needs to measure both always-on SSB and OD-SSB and combine them to generate measurement results, as illustrated in the below picture:

Figure 3: L3 measurements on OD-SSB

Understanding 1 is simpler: since the serving cell can use OD-SSB for measurements, the NW can make sure the OD-SSB is transmitted with enough periods/occasions to satisfy the measurement need, and UE does not need to refer to the SMTC for serving cell measurements.

Understanding 2 is more flexible: the UE can utilize both OD-SSB and always-on SSB to generate measurement results, but the complexity to combine the measurement results is increased.

Proposal 7: For Case #2 (always-on SSB), discuss whether the always-on SSB is also used to generate serving cell measurement results.

The next issue is how to obtain OD-SSB pattern for serving cell measurements. The necessary parameter includes SSB start offset, duration, periodicity, which are quite similar with the SMTC parameters. Besides, a bitmap indicating the transmitted SSB indexes (similar to ssb-ToMeasure or ssb-PositionsInBurst) is needed.

RAN1 discussion used the terminology “time instance A” and “time instance B”, which describe the whole transmission duration for OD-SSB, but it does not illustrate the individual SSB burst duration.

<Excepted from RAN1#116 agreements> - Option 1: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A. - Option 1A: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A until gNB turns OFF the on demand SSB - Option 2: UE expects that on-demand SSB burst(s) is transmitted from time instance A to time instance B and not transmitted after time instance B. - Option 3: UE expects that on-demand SSB burst(s) is transmitted N times after time instance A and not transmitted after N on-demand SSB bursts are transmitted. - Option 4: UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B.

Time instance A can be viewed as the start offset of the OD-SSB pattern. The SSB duration can be determined by the UE after detecting the SSB index. Therefore, at least the SSB periodicity and the SSB bitmap need to be informed to the UE.

Proposal 8: The SSB periodicity and the ssb-PositionsInBurst of the OD-SSB need to be indicated to the UE. RAN2 further discuss whether to use RRC signalling or MAC CE for such information.

For neighbour cells measurements, we think on-demand SSB status of neighbour cells should be transparent to the UE (otherwise large signalling overhead in Uu interface and Xn interface are incurred), and it is up to gNB implementation how to handle the reported neighbour cell measurement results which could be degraded based on the intermittent SSB transmission status of neighbour cell.

Proposal 9: On L3 measurements towards neighbour cells, legacy measurement and reporting procedures are used, and the OD-SSB transmission status of neighbour cells is transparent to the UE.

2.3 Measurement reporting

In the existing spec, the report types of RRM measurements include the following:

ReportConfigNR ::=                          SEQUENCE {

reportType                                  CHOICE {

periodical                                  PeriodicalReportConfig,

eventTriggered                              EventTriggerConfig,

...,

reportCGI                                   ReportCGI,

reportSFTD                                  ReportSFTD-NR,

condTriggerConfig-r16                       CondTriggerConfig-r16,

cli-Periodical-r16                          CLI-PeriodicalReportConfig-r16,

cli-EventTriggered-r16                      CLI-EventTriggerConfig-r16,

rxTxPeriodical-r17                          RxTxPeriodical-r17,

reportOnScellActivation-r18                 ReportOnScellActivation-r18

}

}

Event triggered reporting and periodical reporting are widely used to trigger serving/neighbour cell measurement reports.

For OD-SSB based SCell measurements, considering OD-SSB is transmitted in a limited period (e.g. from time instance A to time instance B), and there could be no intra-frequency neighbour cell SSBs during the period or there are fewer SSB occasions of neighbour cells compared with serving cell OD-SSB, it is difficult to define a proper report interval.

Looking at the existing RRM measurement events, event A1 and A2 only concern serving cell, and can be used for triggering OD-SSB measurement reporting. For event A3 and A5, the neighbour cell measurements need to be compared with serving cell measurements to determine whether the event is triggered. Considering the different characteristics of serving cell OD-SSB (which is denser, in a confined period of time) and neighbour cell always-on SSB (which is sparser), it is unclear whether A3 and A5 can be triggered properly.

Proposal 10: RAN2 to discuss how periodical reporting and/or event-triggered reporting is applied to OD-SSB measurements.

Conclusion

In this contribution, we discussed on-demand SSB for intra/inter-band CA, and propose the following:

Proposal 1: Only the SCells supporting OD-SSB will be included in the OD-SSB transmission MAC CE.

Proposal 2: RRC indicates whether an SCell supports OD-SSB when configuring the SCell, MAC CE indicates the activation/deactivation status sequentially, and indicates SSB periodicity for the SCells with OD-SSB activated (variable size MAC CE).

Proposal 3: For Case #2 (always-on SSB), the new MAC CE is only used to activate and indicate periodicity of OD-SSB, it is not used to modify/activate/deactivate the always-on SSB.

Proposal 4: RAN2 starts discussion on L3 measurement based on Scenario 1 (OD-SSB and always-on SSB are with same SSB carrier frequency, same offset but different periodicities).

Proposal 5: If Scenario 3 (OD-SSB and always-on SSB are within different SSB carrier frequencies) is considered, the following issues needs to be addressed:

- Impacts on servingCellMO configuration (whether both OD-SSB and always-on SSB frequencies are configured)

- Impacts on neighbour cell measurements (whether intra-frequency/inter-frequency determination can be based on OD-SSB)

- Impacts on gap configuration for intra-frequency measurements (whether gaps are needed if a BWP contains always-on SSB but not OD-SSB)

Proposal 6: For both Case #1 (no always-on SSB) and Case #2 (always-on SSB), the OD-SSB pattern is informed to the UE for performing L3 serving cell measurements. No need to modify the legacy SMTC or make up a dedicated SMTC for serving cell measurements.

Proposal 7: For Case #2 (always-on SSB), discuss whether the always-on SSB is also used to generate serving cell measurement results.

Proposal 8: The SSB periodicity and the ssb-PositionsInBurst of the OD-SSB need to be indicated to the UE. RAN2 further discuss whether to use RRC signalling or MAC CE for such information.

Proposal 9: On L3 measurements towards neighbour cells, legacy measurement and reporting procedures are used, and the OD-SSB transmission status of neighbour cells is transparent to the UE.

Proposal 10: RAN2 to discuss how periodical reporting and/or event-triggered reporting is applied to OD-SSB measurements.

References

- RP-242354, Revised WID: Enhancements of network energy savings for NR, Ericsson

- R4-2416916, WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson