3GPP TSG-RAN WG2 Meeting #128	R2-2410433

Orlando, US, 18 – 22 Nov, 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on adaptation of common signal/channel transmissions

Agenda Item:	8.5.4

Document for:	Discussion and decision

Introduction

In this contribution, we provide our considerations on adaptation of common signal/channel transmissions.

Discussion

## 2.1 Adaptation of paging

In RAN2#127, it was agreed to use Option b (extending value ranges) as the baseline for R19 paging enhancement:

Agreements on paging adaptation

- Select option-b as baseline for R19 NES paging enhancement.

- R2 should aim at signaling overhead minimization (e.g., Ns=8 and FFS for other larger values)

- Allowing legacy and R19 UEs to co-ex in the same PF/PO is possible, based on NW configuration.

- Legacy UE is not barred.

- Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration.

In this contribution, we discuss the remaining issues, including: 1) the exact values of N/Ns considering signalling overhead minimization; 2) PEI impacts.

- Extended values of N and Ns

The issue of signalling overhead was raised in [POST127][109][NES] and also discussed in RAN2#127bis [1][2]. The concern is mainly on parameter firstPDCCH-MonitoringOccasionOfPO, which is configured per PO, and the value range depends on SCS and N (number of PFs in paging cycle T).

In RAN2#127bis meeting, it is proposed that Ns can be extended to 8. Correspondingly, the value of N should be extended to T/32, otherwise NES gain cannot be achieved. (N = T/32, Ns = 8) and (N = T/16, Ns = 4) provide the same paging capacity (same number of POs). With (N = T/32, Ns = 8), the maximum number of PDCCH monitoring occasions is doubled compared with (N = T/16, Ns = 4), therefore the value range of firstPDCCH-MonitoringOccasionOfPO is also doubled and an additional bit is needed for each PO. Similarly, if we extend the value ranges to (N = T/64, Ns = 16), the paging capacity is the same as (N = T/16, Ns = 4) but the value range of firstPDCCH-MonitoringOccasionOfPO is quadrupled and two additional bits are needed for each PO. To avoid excessive signalling overhead, we think Ns = 16 or larger values should not be supported.

Another reason for not supporting Ns = 16 is the compatibility with PEI. Considering the payload size of PEI is no larger than 41 bits and 43 bits for licensed and unlicensed spectrums respectively, it is difficult to associate more POs with one PEI.

Proposal 1: Extend the Ns value to 8 and N value to T/32. Other values are not considered in this release.

Another thought on signalling overhead reduction raised during RAN2#127bis is to use repetitive PO pattern. For instance, in the following example, there are 8 POs in the PF when Ns is extended to 8. The idea is that the latter 4 POs repeat the patterns of the former 4 POs (including the configuration of first PDCCH monitoring occasion of the PO).

Figure 1: Comparison between option b and legacy mechanism

In our understanding, the configuration of firstPDCCH-MonitoringOccasionOfPO is already optional in the existing spec. The network can choose not to configure firstPDCCH-MonitoringOccasionOfPO when signalling overhead is a problem, and in this case the POs will be placed back to back in the PF. However, if the network decides to configure firstPDCCH-MonitoringOccasionOfPO for the former 4 POs (overlapping with legacy POs to be monitored by legacy UEs), the network should also have the flexibility to place the latter POs, instead of using repeated pattern to restrict the flexibility.

Proposal 2: Parameters for PF/PO calculation other than N/Ns would follow legacy spec. No additional enhancement is introduced for repetitive PO pattern.

- PEI impacts

PEI was introduced in R17 for UE power saving. Even though it cannot increase network energy saving gain, we think it is beneficial to make the R19 NES paging adaptation compatible with PEI, to avoid unnecessary power consumption at the UE side.

Figure 2: PEI mechanism in the existing spec

According to existing spec, PEI can be associated to at most 2 PFs and at most 8 POs:

PEI-Config-r17 ::=                        SEQUENCE {

po-NumPerPEI-r17                          ENUMERATED {po1, po2, po4, po8},

payloadSizeDCI-2-7-r17                    INTEGER (1..maxDCI-2-7-Size-r17),

pei-FrameOffset-r17                       INTEGER (0..16),

subgroupConfig-r17                        SubgroupConfig-r17,

lastUsedCellOnly-r17                      ENUMERATED {true}                                                OPTIONAL,  -- Need R

...

}

po-NumPerPEI The number of PO(s) associated with one PEI monitoring occasion. It is a factor of the total PO number in a paging cycle, i.e N x Ns, as specified in TS 38.304 [20]. The maximum number of PF associated with one PEI monitoring occasion is 2. The number of PO mapping to one PEI should be multiple of Ns when po-NumPerPEI is larger than Ns.

Observation 1: In the existing spec, PEI is associated to at most 2 PFs, at most 8 POs, based on the assumption that each PF has at most 4 POs.

The current restriction on the number of PF(s)/PO(s) associated to PEI assumes that there could be at most 4 POs in a PF. The number of POs associated to PEI need to balance the PEI overhead per PO and the paging latency/ UE power consumption. The following was agreed in RAN1#107:

Agreement Support mapping one PEI to POnumPerPEI PO(s) in one or multiple PF(s) - POnumPerPEI is a factor of (total PO number in a paging cycle) and configurable via SIB for the cell with the value range of {1, 2, 4, 8} - The Maximum number of PF associated with one PEI is up to 2 - Note: Maximum number of paging indication bits in DCI format 2_7 can be kept the same for any configuration of POnumPerPEI, e.g., by applying a smaller subgroupsNumPerP and a larger POnumPerPEI. - Note: Larger value of POnumPerPEI can reduce the average PEI overhead per PO, but there can also cause potentially larger paging latency and larger UE power consumption due to longer UE wake-up time before PO monitoring, which can be significant with large value of (T/N).

Since RAN2 has agreed not to bar legacy UEs, and R19 NES UEs monitor paging according to R19 paging configuration, it needs to be discussed whether additional R19 PEI configuration is configured to R19 UEs as well. More specifically, whether R19 UEs monitor the same PEI as legacy UEs.

Figure 3: Comparison between common PEI and separate PEIs

If separate PEI is sent for legacy UEs and R19 NES UEs, the existing PEI format can be reused.

If the same PEI is monitored for both legacy UEs and R19 NES UEs (including the additional POs that are non-overlapping with legacy POs), the association between PEI and POs need to be extended, considering the extended value of Ns = 8. For instance, PEI can be associated to at most 16 POs. An alternative is to restrict that the number of PF associated to the PEI can only be 1.

PEI format is defined in TS 38.212:

DCI format 2_7 is used for notifying the paging early indication and TRS availability indication for one or more UEs. The following information is transmitted by means of the DCI format 2_7 with CRC scrambled by PEI-RNTI: - Paging indication field - NPOPEINSGPO bit(s), where - NPOPEI is the number of paging occasions configured by higher layer parameter po-NumPerPEI as defined in Clause 10.4A in [5, TS 38.213]; - NSGPOis the number of sub-groups of a paging occasion configured by higher layer parameter subgroupsNumPerPO. - Each bit in the field indicates one UE subgroup of a paging occasion. - TRS availability indication - 1, 2, 3, 4, 5, or 6 bits, where the number of bits is equal to one plus the highest value of all the indBitID(s) provided by the trs-ResourceSetConfig or the number of bits is equal to one plus the highest value of all the indBitID-r18(s) provided by the trs-ResourceSetConfig-r18 if configured if configured; 0 bits otherwise. The size of DCI format 2_7 is indicated by the higher layer parameter payloadSizeDCI-2-7, according to Clause 10.4A of [5, TS 38.213]. The number of information bits in format 2_7 shall be equal to or less than the payload size of format 2_7. If the number of information bits in format 2_7 is less than the size of format 2_7, the remaining bits are reserved.

If the same PEI is monitored for both legacy UEs and R19 NES UEs, the legacy UEs still monitor the first NPOPEINSGPO bits, and the R19 NES UEs may need to monitor either the legacy bits or the extended bits, depending on whether the calculated POs overlap with legacy POs.

Proposal 3: RAN2 to discuss whether R19 NES UEs monitor the same PEI with legacy UEs.

Proposal 4: If the same PEI is monitored by R19 NES UEs and legacy UEs, RAN2 to discuss PEI association when Ns is extended to 8:

- Option 1: One PEI monitoring occasion is associated to one PF when Ns is 8;

- Option 2: One PEI monitoring occasion is associated to at most 16 POs.

## 2.2 Adaptation of SSB

The discussion on SSB adaptation mainly took place in RAN1, and considerable progress has been made in RAN1.

Apart from the open issues identified by RAN1, we think RAN2 should discuss the impacts on L3 measurements due to dynamic adaptation of SSB transmission. It is also requested by RAN4 to discuss SMTC impacts in RAN2 [3]:

RAN4#112bis extensively discussed impact on RRM requirements from SSB adaptation. The following agreements were reached after discussion: Agreements: - RAN4 to discuss whether there are any impacts on at least the following RRM requirements due to SSB adaption: - L1-RSRP for serving cell - L1-SINR - BFD - CBD - SCell activation - TCI state switching - NR intra-frequency L3 measurement - NR inter-frequency L3 measurement However, L3 measurement requirement in RAN4 is defined based on SMTC configuration rather than SSB configuration in legacy. RAN4 would like to ask RAN2 that whether there is any SMTC impact due to SSB adaptation.

The “SSB adaptation” mentioned in the RAN4 LS refers to the adaptation of SS burst periodicity [4]:

Issue 1-1-1: Scenario clarification based on RAN1 progress Agreement: - Per RAN1 agreements, RAN4 to take following as starting point for SSB adaption discussion: - CONNECTED mode, and - SSB adaptation for SCell, and - SSB adaptation for NCD-SSB, and - SSB periodicity adaptation in time-domain Note: Above could be further updated based on further RAN1 progress (e.g. PCell, other adaptation mechanism)

For serving cell measurements, in the legacy mechanism the UE measures serving cell SSB based on SMTC configuration. In R19 SSB adaptation, the UE should adjust the SMTC periodicity based on the latest SSB periodicity.

For neighbour cell measurements, it is not feasible to ask UE to adapt SMTC periodicities based on dynamic change of neighbour cell SSB measurements, a proper way is to configure a separate SMTC for neighbour cells supporting SSB adaptation. For instance, if a neighbour cell supports dynamic change of SSB periodicity among {20ms, 40ms, 80ms}, the network can configure a dedicated SMTC of 80ms periodicity for such NES neighbour cells.

Proposal 5: A dedicated SMTC with larger periodicity can be configured to measure neighbour cells supporting dynamic SSB adaptation.

The above applies only to SSB adaptation, not to on-demand SSB, as there are differences between the two features:

- For OD-SSB, there could be on-demand SSB and always-on SSB simultaneously (it is still FFS in RAN1 whether there is time/frequency overlap between on-demand SSB and always-on SSB), while for SSB adaptation only the periodicity is changed.

- For OD-SSB, the SSB pattern is much more complex than legacy SSB pattern, as RAN1 has defined a time instance A to indicate the beginning of the fist transmitted SSB, and gNB can flexibly configure SSB time domain positions of on-demand SSB burst (which could be more compact compared with existing SSB positions in an SS burst). Still, for SSB adaptation, only the periodicity is changed.

- For neighbour cell measurements, in case of SSB adaptation, the largest periodicity of neighbour cell SSB can be predicted, therefore a SMTC window of large periodicity can be used for neighbour cell measurements. However, for on-demand SSB, it is not possible or efficient for the gNB to frequently exchange the dynamic pattern for neighbour cell OD-SSB transmission and configure the SMTC accordingly.

## 2.3 Adaptation of RACH

In RAN#104 meeting, the revised WID has already removed the PRACH adaptation in spatial domain [5].

Regarding RACH adaptation in time domain, RAN1 has agreed to introduce additional PRACH resources for NES-capable UEs in additional to PRACH resources for legacy UEs. The PRACH preamble format is the same for the legacy PRACH resources and the additional resources. Hence, only the time/frequency domains are left for discussion. The PRACH configuration index and the frequency domain resources can be same or different. It is still FFS whether additional parameters are needed if the PRACH configuration index is the same, and whether additional mechanism is needed if the PRACH configuration index is different.

Agreement For adaptation of PRACH in time-domain, the same PRACH preamble format is used for the additional RACH resources and legacy PRACH resources. Agreement For adaptation of PRACH in time-domain, support both of the following - Alt 1: The PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resources - Alt 2: The PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resources - FFS: Additional details Agreement For adaptation of PRACH in time-domain, the frequency domain resources for the additional PRACH resources and legacy PRACH resources can be same or different - FFS: applicable case(s) (i.e. case(s) from the RAN1#117 agreement). - Discuss further following options for signaling - Option 1: at least the following parameter(s) can be configured separately for the additional PRACH resources. - msg1-FrequencyStart at least for 4-step RACH - FFS: other applicable legacy frequency domain parameter(s) - Option 2: Offset to legacy frequency domain parameter(s) are configured for the additional PRACH resources - Note: Offset to legacy frequency domain parameter(s) is a new parameter - FFS: applicable legacy frequency domain parameter(s) Agreement For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain, - For Alt 1 (same PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s), - Opt 1-2a: up to N1 additional value(s) of (x, y) - FFS: value of N1(>=1) - Opt 1-2b: up to N2 additional value(s) of y - FFS: value of N2 (>=1) - Opt 1-3: Muting/masking ROs - Opt 1-4: - up to N4_1 additional timing offset(s) at frame-level - up to N4_2 additional timing offset(s) at slot-level - FFS: values of N4_1(>=1) and N4_2(>=1) - Opt 1-5: No additional mechanism is selected. - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211) - For Alt 2 (different PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s), - Opt 1-2a: up to N1 additional value(s) of (x, y) - FFS: value of N1(>=1) - Opt 1-2b: up to N2 additional value(s) of y - FFS: value of N2 (>=1) - Opt 1-3: Muting/masking ROs - Opt 1-4: - up to N4_1 additional timing offset(s) at frame-level - up to N4_2 additional timing offset(s) at slot-level - FFS: values of N4_1(>=1) and N4_2(>=1) - Opt 1-5: No additional mechanism is selected. - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211)

There are other features introducing separate RACH resources, and can be categorized as follows:

- Using RACH partitioning framework, including (e)Redcap, SDT, Coverage enhancement, RAN slicing.

The configuration of RACH resource sets is achieved by separate ROs (via additionalRACH-ConfigList in BWP-UplinkCommon), or separate preamble partitioning with shared ROs (via featureCombinationPreamblesList in RACH-ConfigCommon and RACH-ConfigCommonTwoStepRA).

- Not using RACH partitioning framework, including LTM and twoTA.

The additional RACH configuration for LTM is provided via EarlyUL-SyncConfig in dedicated RRC signalling. The early sync in LTM is RACH towards LTM candidate cell, not serving cell.

The additional RACH configuration for twoTA is provided via additionalRACH-perPCI-ToAddModList and additionalRACH-perPCI-ToReleaseList in BWP-UplinkCommon.

Since RAN1 has agreed in 116bis meeting that the PRACH adaptation is applicable for UEs all RRC states, the additional RACH resources should be provided in SIB1. Additionally, the P-RNTI is assumed to be used to indicate the PRACH adaptation. After receiving the DCI, UE in all RRC states could use the additional PRACH resources based on the configuration in SIB1.

Agreement Support adaptation mechanisms of PRACH in time-domain for following: - UE in idle/inactive mode - UE in connected mode Working Assumption For DCI-based adaptation for additional PRACH resources, at least DCI format 1_0 can carry the adaptation indication for UEs in idle/inactive and connected mode. - P-RNTI is used

Proposal 6: The additional PRACH resources for NES-capable UEs are provided in SIB1.

On the detailed signalling, we think the first thing to clarify is that whether we incorporate the additional PRACH resources of NES into the RACH partitioning framework. There are two things to be considered: 1) whether there is a need to enable the early identification of NES feature on the network side; 2) whether other RACH-related features like Redcap/SDT/CE can be used jointly with NES.

Take Redcap as an instance, we think it is possible for NES cells to support the access of Redcap UEs, there is no need to exclude the combination of NES and Redcap. RAN2 has discussed whether to prevent legacy UEs from accessing R19 NES cell, and the “legacy UEs” usually refer to the UEs from previous releases not supporting the R19 NES feature. RAN2 can also consider whether UEs of other features (e.g. Redcap UEs) can implement the R19 NES feature and whether these kinds of UEs should be considered when designing the R19 framework.

Proposal 7: RAN2 to discuss whether to incorporate the additional PRACH resources of NES into the RACH partitioning framework, based on two factors: 1) whether there is a need to enable the early identification of NES feature on the network side; 2) whether other RACH-related features like Redcap/SDT/CE can be used jointly with NES.

On CBRA vs CFRA, we think RAN2 should start from CBRA, because UEs in Idle and Inactive states need to be considered and the objective is restricted to common signals. The discussion on CFRA can wait until the mechanism on CBRA is finalized. On 2-step RA vs 4-step RA, we do not see the need to preclude 2-step RA at the current stage.

Proposal 8: CBRA is prioritized for RACH adaptation in time domain.

Proposal 9: Both 4-step RA and 2-step RA are considered for RACH adaptation in time domain.

Conclusion

In this contribution, we discussed the adaptation of common signal/channel transmissions, and made the following proposals:

Proposal 1: Extend the Ns value to 8 and N value to T/32. Other values are not considered in this release.

Proposal 2: Parameters for PF/PO calculation other than N/Ns would follow legacy spec. No additional enhancement is introduced for repetitive PO pattern.

Observation 1: In the existing spec, PEI is associated to at most 2 PFs, at most 8 POs, based on the assumption that each PF has at most 4 POs.

Proposal 3: RAN2 to discuss whether R19 NES UEs monitor the same PEI with legacy UEs.

Proposal 4: If the same PEI is monitored by R19 NES UEs and legacy UEs, RAN2 to discuss PEI association when Ns is extended to 8:

- Option 1: One PEI monitoring occasion is associated to one PF when Ns is 8;

- Option 2: One PEI monitoring occasion is associated to at most 16 POs.

Proposal 5: A dedicated SMTC with larger periodicity can be configured to measure neighbour cells supporting dynamic SSB adaptation.

Proposal 6: The additional PRACH resources for NES-capable UEs are provided in SIB1.

Proposal 7: RAN2 to discuss whether to incorporate the additional PRACH resources of NES into the RACH partitioning framework, based on two factors: 1) whether there is a need to enable the early identification of NES feature on the network side; 2) whether other RACH-related features like Redcap/SDT/CE can be used jointly with NES.

Proposal 8: CBRA is prioritized for RACH adaptation in time domain.

Proposal 9: Both 4-step RA and 2-step RA are considered for RACH adaptation in time domain.

References

- R2-2407974, Report for [POST127][109][NES] (OPPO), OPPO

- R2-2409211, Report from session on V2X/SL, R19 NES and MOB, Vice Chairman (Samsung)

- R4-2416911, LS on SSB adaptation, RAN4 (contact company: Apple)

- R4-2416860, WF on RRM requirements for Netw_Energy_NR_enh_Part2, Huawei

- RP-241650, Revised WID: Enhancements of network energy savings for NR, Ericsson