3GPP TSG-RAN WG2 Meeting #128	R2-2410435

Orlando, US, Nov 18th – 22th, 2024

Agenda item:	8.7.4.1

Source:	China Telecom

Title:	Discussion on Leftover Issues for Additional LCH Priority

Document for:	Discussion and Decision

- Introduction

In the RAN2#127bis meeting, the following agreements were made for LCP prioritization.

Agreements on LCP enhancements As a baseline, additional LCH priority is applied for an LCH in both 1st and 2nd Rounds of resource allocation procedure in LCP, as long as the LCH has delay-critical data available for transmission when starting the 1st Round. FFS if we can still change the priority for the 2nd round to ensure fairness, but we need to consider tight timeline of LCP procedure and UE complexity. Companies can also check whether we can leave this to UE implementation Introduce an independent per-LCH remaining time threshold for applying delay-critical priority. We do not introduce any setting restrictions of this new remaining time threshold with relation to DSR triggering threshold.

The intention of this paper is to discuss the remaining issues regarding to additional LCH priority in LCP prioritization.

2 Discussion

Per TS38.321, the legacy UL resource allocation is shown as below. The UE maintains a variable Bj for each logical channel j. In step 1 (i.e. 1st round), the UL grant resources are allocated to the logical channels in a decreasing order with Bj>0. In step 2, the Bj subtracts the total size of MAC SDUs used in the MAC PDU for logical channel j in step 1. In step 3, if there are still UL resources remaining after the 1st round of resource allocation, they will be allocated to logical channel in a strict decreasing order, without comparing the size of Bj. In other words, the low priority logical channels can receive services only after all data from high priority logical channels has been transmitted, which means that the UE will maximize the data transmission of high priority logical channels in the 2nd round of resource allocation.

Observation: In legacy, the UE will maximize the data transmission of high priority logical channels in the 2nd round of resource allocation.

- 5.4.3.1.3 Allocation of resources Before the successful completion of the Random Access procedure initiated for DAPS handover, the target MAC entity shall not select the logical channel(s) corresponding to non-DAPS DRB(s) for the uplink grant received in a Random Access Response or the uplink grant for the transmission of the MSGA payload. The source MAC entity shall select only the logical channel(s) corresponding to DAPS DRB(s) during DAPS handover. The MAC entity shall, when a new transmission is performed: 1> allocate resources to the logical channels as follows: 2> logical channels selected in clause 5.4.3.1.2 for the UL grant with Bj > 0 are allocated resources in a decreasing priority order. If the PBR of a logical channel is set to infinity, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s); 2> decrement Bj by the total size of MAC SDUs served to logical channel j above; 2> if any resources remain, all the logical channels selected in clause 5.4.3.1.2 are served in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first. Logical channels configured with equal priority should be served equally. NOTE 1: The value of Bj can be negative.

To prioritize the transmission of data with low remaining time, an additional priority is introduced for delay-critical data. In the last meeting, it has agreed that additional LCH priority is applied for LCH in both 1st and 2nd rounds of resource allocation procedure in LCP as a baseline. In other words, the additional priority will be used for LCH all the time even though there is no delay-critical data during the 2nd round of resource allocation. However, it may be unfair for other LCHs without delay-critical data whose priority remains the same all the time.

For instance, it is assumed that the legacy priority of LCH1 and LCH2 are 5 and 3, respectively. In case there is delay-critical data in LCH1, its priority changes to 2. Based on the LCH priority, the delay-critical data in LCH1 will be assembled first. In case all the delay-critical data is assembled during the 1st round of resource allocation, it is more suitable for LCH1 to use the legacy priority 5 during 2nd round of resource allocation procedure. Otherwise, the data in LCH1 will be always prioritized even though there is no delay-critical data during the 2nd round of resource allocation procedure. Therefore, if all the delay-critical data in LCH1 is assembled during 1st round, the priority of LCH1 is changed to legacy 5 before 2nd round of resource allocation procedure to ensure fairness.

Proposal 1: If all delay-critical data is assembled during 1st round, the LCH priority is changed to legacy one before 2nd round of resource allocation to ensure fairness.

In the last meeting, it has agreed that an independent per-LCH remaining time threshold is introduced for applying delay-critical priority, indicating that the existence of delay-critical data may depend on the remaining time of PDCP discard timer. So, the MAC needs to compare the configured remainingTimeThreshold for LCH and the remaining values of discard timer in PDCP before changing the logical channel priority, which may introduce extra latency and specification complexity. To reduce specification impact, how to change the priority before the 2nd round of resource allocation can be left to UE implementation.

Proposal 2: How to change the priority before the 2nd round of resource allocation can be left to UE implementation.

In addition, it is undetermined how to configure the additional priority of LCH with delay-critical data. In legacy, the LCH priority is configured by RRC signalling. To reduce the specification impact, the additional priority of LCH with delay-critical data could also be configured by RRC signalling. In this way, the network may have more flexibility in resource allocation procedure.

Proposal 3: Additional priority of LCH with delay-critical data is configured by RRC signalling.

- Conclusions

Based on the above analysis, we have the following observation and proposals:

Observation: In legacy, the UE will maximize the data transmission of high priority logical channels in the 2nd round of resource allocation.

Proposal 1: If all delay-critical data is assembled during 1st round, the LCH priority is changed to legacy one before 2nd round of resource allocation to ensure fairness.

Proposal 2: How to change the priority before the 2nd round of resource allocation can be left to UE implementation.

Proposal 3: Additional priority of LCH with delay-critical data is configured by RRC signalling.

- References

- R2-24095xx, Report of 3GPP TSG RAN WG2 meeting #127bis, Hefei, China