3GPP TSG-RAN WG2 Meeting #128	R2-2410437

Orlando, US, Nov 18th – 22th, 2024

Agenda item:	8.7.5

Source:	China Telecom

Title:	Discussion on RLC AM Enhancements

Document for:	Discussion and Decision

- Introduction

The topic of RLC enhancements were widely discussed in the RAN2#127bis meeting, and the following agreements were achieved.

Agreements on RLC timely retransmissions - RAN2 confirm that existing mechanisms are insufficient to resolve the timely RLC retransmission problem and RLC enhancements for timely RLC retransmission are investigated in Rel-19. - Exclude enhanced status reporting. - Focus the discussion on autonomous retransmission and polling enhancements, e.g. we need to understand how each option affects the capacity and packet delay Agreements on avoiding unnecessary retransmissions - RAN2 confirm the previous baseline assumption: the RLC receiving window always advances to any given RLC SN before the transmitting window does. - RAN2 will adopt a “combined” approach for avoiding unnecessary RLC retransmissions, i.e. - TX side stops transmissions of an outdated SDU - RX side abandons the SDU based on a local timer - Rx informs Tx side about the abandoned SDUs, as a baseline we assume existing SR can be reused unless issues are identified - FFS if some C-PDU handling is needed to avoid C-PDU discard - FFS if some indication is sent from Tx to Rx. The assumption is this is not a full status report, but something simple (if needed)

This paper continue to discuss remaining issues on unnecessary retransmission avoidance and timely RLC retransmission.

- Discussion

2.1 Timely RLC retransmissions

Per TS 38.322, the UE can use polling mechanism to request the network to trigger status report:

- The amount of AMD PDUs/data bytes sent since the most recent poll bit was transmitted is larger than a threshold (i.e., PDU_WITHOUT_POLL >= pollPDU; or BYTE_WITHOUT_POLL >= pollByte), where the “pollPDU” indicates that the transmitter trigger a poll for every pollPDU PDUs, and “pollByte” indicates that the transmitter trigger a poll for every pollByte bytes;

- Both the transmission buffer and the retransmission buffer becomes empty (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) after the transmission of the AMD PDU;

- No new RLC SDU can be transmitted after the transmission of the AMD PDU (e.g. due to window stalling);

It can be observed that the above configured variables, namely PDU_WITHOUT_POLL, BYTE_WITHOUT_POLL can be used to trigger a poll of AMD PDU. For delay-critical RLC SDUs, one efficient way to trigger status reports more quickly is to configure the parameters of PDU_WITHOUT_POLL/BYTE_WITHOUT_POLL as smaller values. For instance, to enable timely RLC retransmission for time-sensitive UL XR traffic, the network could configure smaller pollPDU or pollByte values for UE by RRC message RLC-Config. In other words, frequent polling can trigger faster status report. Compared to introduce an extra timer to trigger polling, to specify new values in polling has minimal impact on RLC layer.

In addition, the status report will only be triggered upon the expiration of t-StatusProhibit timer, which means that the frequent polling may need to work together with a shorter t-StatusProhibit timer. Therefore, if shorter pollPDU or pollByte values are configured, the network could also configure a shorter t-StatusProhibit timer value.

Proposal 1: For UL traffic, the network can configure shorter pollPDU or pollByte values to trigger the polling for timely RLC retransmission.

In XR, a PDU set containing one or more PDUs is introduced to carry the payload of one unit of information generated at the application level. Per TS38.322, the UE polls the gNB after a configured number of PDUs or bytes. It is most likely that the UE may trigger a poll in the middle of a PDU set. As a result, the status report may only contain the reception status of a part of PDUs in a PDU set. If the other part of the PDUs has not been retransmitted promptly, the PDU set containing the previously successfully transmitted PDUs will also be discarded, resulting in resource waste.

Therefore, for XR traffic, the polling mechanism may need to be enhanced to enable the UE to trigger the poll based on the PDU set. For instance, the polling bit can be set based on the PDU set traffic pattern. In this way, the UE can receive the status report of one or more PDU sets.

Proposal 2: Polling enhancement is needed to enable the UE to poll the RLC status based on the PDU set.

2.2 Avoid unnecessary retransmission

At the last RAN2 meeting, it was agreed that the combined Tx and Rx approach can be used to avoid unnecessary retransmission. In the combined approach, Tx stops (re)transmission of an outdated SDU based on PDCP layer discard indication or a number of retransmissions and Rx abandons the obsolete SDUs based on a local timer. To reduce the specification impact and maintain the synchronization between the transmitting window and receiving window, the Rx entity sends the status report to the Tx entity to help it update the transmitting window.

Since the Tx RLC entity knows the remaining time of the RLC SDU, it can avoid the unnecessary retransmission of the outdated SDUs more accurately. Therefore, to prevent the Rx entity from triggering and sending unnecessary status reports for outdated SDUs, it is beneficial to support the Tx entity in sending a notification to Rx entity to indicate the obsolete RLC SDUs. However, per TS38.322, legacy status report with the STATUS PDU with 12/18 bit SN is too complex to be reused for this purpose. To reduce specification complexity, a simple Tx side status report which only contains the SN of the outdated SDUs can be designed to indicate the obsolete RLC SDUs. Upon this information was received, the Rx entity abandons the obsolete SDUs and update the receiving window. Then the legacy status report procedure could be triggered.

Proposal 3: A simple indication from Tx to Rx is needed to indicate the obsolete RLC SDUs.

- Conclusions

Based on the above analysis, we have the following proposals:

Proposal 1: For UL traffic, the network can configure shorter pollPDU or pollByte values to trigger the polling for timely RLC retransmission.

Proposal 2: Polling enhancement is needed to enable the UE to poll the RLC status based on the PDU set.

Proposal 3: A simple indication from Tx to Rx is needed to indicate the obsolete RLC SDUs.

- References

- R2-24095xx, Report of 3GPP TSG RAN WG2 meeting #127bis, Hefei, China