3GPP TSG-RAN WG2 Meeting #128	R2-24010438

Orlando, USA, 18th – 22nd of November 2024

Agenda item:	7.4.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Various Corrections and Open Points for Rel-18 NTN

WID/SID:	NR_NTN_enh-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

In this paper we present our view how to resolve the remaining Release 18 NTN issues.

# 2	Discussion

## 2.1	FR2-NTN

RAN1 during their RAN1#118 meeting (August 2024) has completed the work on supporting NR NTN in the FR2 frequency bands. During this discussion RAN1 has realized there are inconsistencies whether FR2-1 or FR2-2 procedures should apply to NTN’s FR2 (i.e. FR2-NTN). It is important as RAN1 specifications define separate behaviour for FR2-1 and FR2-2.

Observation 1: There are unclarities whether FR2-1 or FR2-2 procedures in various specifications should apply to the FR2-NTN.

It was also discovered that a similar issue might be applicable to the existing RAN2 RRC parameters. RAN1 LS [2] lists the following exemplary parameters: subcarrierSpacing, channelBWs-DL, configuredUL-GrantType1-v1650. These might be missing the reference to FR2-2 bands.

Observation 2: RAN1 has discovered certain RRC parameters might not have appropriate indication which FR2-x they belong to (i.e. FR2-1, FR2-2, FR-NTN).

According to [4], corresponding frequency ranges for FR2-1, FR2-2 and FR2-NTN are defined as follows:

- FR2-1: 24250 MHz – 52600 MHz

- FR2-2: 52600 MHz – 71000 MHz

- FR2-NTN: 17300 MHz – 30000 MHz

As can be noticed, FR2-NTN partly overlaps with FR2-1. However, the lower frequency region in FR2-NTN is not comprised within FR2-1.

Observation 3: FR2-NTN partly overlaps with FR2-1. Its lower frequency region is not comprised within FR2-1.

Nevertheless, it can be assumed that most of the parameters and procedures defined for FR2-1 (and not for FR2-2 which covers a completely different part of the spectrum, i.e. frequencies above 52 GHz according to [4]) can be applicable to FR2-NTN. We claim most of the parameters and procedures, as e.g. handover in FR2-NTN is much simplified when compared to FR2-1 (e.g. no beam sweeping).

Proposal 1: RAN2 assumes the requirements and parameters for FR2-1 are generally applicable to FR2-NTN, unless specified otherwise.

At RAN2#127bis the following has been agreed [5] when discussing the CR [6]:

We take this CR as a basis, apart from the fist change where we consider the proposal in R2-2408012 Come back in the next meeting (a single CR from ZTE and vivo is expected in the next meeting)

There is a large number of parameters and capabilities which might be impacted with the need to cover FR2-NTN. During RAN2#127bis discussions we suggested to capture somewhere a general statement or a pointer to TS 38.101 which would be used to clarify which requirements apply to FR2-NTN.

Proposal 2: Consider a general statement in RAN2 Technical Specifications or a pointer to TS 38.101 to reflect which requirements apply to FR2-NTN.

Alternatively, a single CR correcting all impacted fields and IEs can be approved in RAN2.

Proposal 3: If Proposal 2 is not agreeable, RAN2 agrees a single CR fixing the FR2-NTN requirements for all affected fields/IEs.

## 2.2	RACH-less HO in inter-gNB scenario

During RAN2#127bis it was attempted to clarify if RACH-less HO is applicable to inter-gNB scenario. The discussion occurred in the RAN2 main session, but also in NTN to decide specifically if RACH-less HO can be applicable to inter-gNB scenario at least for Non-Terrestrial Networks (NTN).

Observation 4: At RAN2#127bis it was attempted to clarify if RACH-less HO can be used also in inter-gNB scenario, at least for NTN.

Some (valid) concerns have been raised that RACH-less in Rel-18 has been deliberately specified as intra-gNB feature in order to avoid any impacts to RAN WG3 (as the decision to support RACH-less HO for baseline mobility has been done late during the Release 18 work and it was not clear how much effort it may require in RAN3).

Observation 5: During Release 18 the decision has been made to limit the RACH-less HO applicability to intra-gNB scenario, as it was not clear how much work might be required in RAN WG3.

The Stage-2 description of RACH-less handover, captured in section 9.2.3.6 of TS 38.300 is written in a way which does not leave any doubts – RACH-less handover in Release 18 is limited to intra-gNB scenario.

Observation 6: The description of RACH-less handover, captured in section 9.2.3.6 of TS 38.300, clearly limits the applicability to intra-gNB case.

RACH-LessHO-r18 is an IE included in ReconfigurationWithSync [7]. It comprises targetNTA and beamIndication. When it comes to inter-gNB signalling, ReconfigurationWithSync is sent in transparent RRC container. It is prepared by target cell and source cell theoretically does not need to know the details of this configuration.

Observation 7: RACH-less HO is decided and configured by target cell and included in ReconfigurationWithSync. Source cell theoretically does not need to comprehend this value and just forwards the RRC container to the UE.

Observation 8: RACH-LessHO IE comprises target NTA value and beam indication (either TCI state or SSB index).

In order to set this information properly (e.g. beam information), the target cell needs to obtain relevant measurement information from the source cell. However, that seems to be possible according to [7]. HOPreparationInformation contains a list of MeasResultNR, which provides the measurement results per SSB or CSI-RS. Thus, it can be assumed the target cell may prepare the beam indication on this basis.

Observation 9: Target cell obtains a list of SSB-based or CSI-RS-based measurement results when HO Preparation is done.

Thus, we do not think there are major blocking points to allow RACH-less handover also in inter-gNB scenario. It should be allowed if NW implementation can handle it.

Proposal 4: Support RACH-less HO also in inter-gNB scenarios for NTN.

## 2.3 SIB19 acquisition in IDLE mode

During the discussions at RAN2#127bis the following understanding has been written in the Session Chair notes [5]:

- RAN2 understands that for now there are no RAN4 requirements regarding measurements on a NTN cell while the UE is camping or being connected to a TN cell and then it’s up to UE implementation whether to perform such measurements or not. RAN2 will not further work on enhancements for TN to NTN mobility in Rel-18.

As a brief reminder, RAN2 has defined the possibility to provide SIB19 in Terrestrial Network (TN), aimed at helping the UE to measure NTN neighbours for potential reselection.

Observation 10: Release 18 NTN introduced the possibility to provide SIB19 in Terrestrial Network (TN), aimed at helping the UE to measure NTN neighbours for potential reselection.

When looking at the requirements described in [8] one can find section 4.2.2.12 “Measurements of inter-frequency NR cells with NTN carrier”. In the beginning of this section, the following is stated [8]:

[…] This clause applies for the inter-frequency cell reselection for NTN carriers in FR1 NTN, and TN carriers if configured. The requirements in clause 4.2.2.12 apply provided that network provides SIB19 and UE is configured with one or more NTN carrier. […]

Then a set of requirements , e.g. how to rank the neighbour inter-frequency NTN cells based on their SS-RSRP or SS-RSRQ is to be performed. Thus, the conclusion made at RAN2#127bis, captured above, might not be valid.

Observation 11: Section 4.2.2.12 in TS 38.133 comprises the requirements for measurements for inter-frequency reselection to NR NTN cells.

Thus, we propose the following:

Proposal 5: RAN2 reopens the discussion on whether the UE can measure an NTN cell while camping or being connected to a TN cell. RAN2 reflects RAN4 requirements in the appropriate specifications.

If Proposal 5 is not acceptable, we should align with RAN4 (at least let them know we do not respect their requirement).

Proposal 6: Align with RAN4 regarding the requirements for SIB19 acquisition in TN cell.

# 3	Conclusion

This document has made the following observations:

Observation 1: There are unclarities whether FR2-1 or FR2-2 procedures in various specifications should apply to the FR2-NTN.

Observation 2: RAN1 has discovered certain RRC parameters might not have appropriate indication which FR2-x they belong to (i.e. FR2-1, FR2-2, FR-NTN).

Observation 3: FR2-NTN partly overlaps with FR2-1. Its lower frequency region is not comprised within FR2-1.

Observation 4: At RAN2#127bis it was attempted to clarify if RACH-less HO can be used also in inter-gNB scenario, at least for NTN.

Observation 5: During Release 18 the decision has been made to limit the RACH-less HO applicability to intra-gNB scenario, as it was not clear how much work might be required in RAN WG3.

Observation 6: The description of RACH-less handover, captured in section 9.2.3.6 of TS 38.300, clearly limits the applicability to intra-gNB case.

Observation 7: RACH-less HO is decided and configured by target cell and included in ReconfigurationWithSync. Source cell theoretically does not need to comprehend this value and just forwards the RRC container to the UE.

Observation 8: RACH-LessHO IE comprises target NTA value and beam indication (either TCI state or SSB index).

Observation 9: Target cell obtains a list of SSB-based or CSI-RS-based measurement results when HO Preparation is done.

Observation 10: Release 18 NTN introduced the possibility to provide SIB19 in Terrestrial Network (TN), aimed at helping the UE to measure NTN neighbours for potential reselection.

Observation 11: Section 4.2.2.12 in TS 38.133 comprises the requirements for measurements for inter-frequency reselection to NR NTN cells.

It has proposed the following:

Proposal 1: RAN2 assumes the requirements and parameters for FR2-1 are generally applicable to FR2-NTN, unless specified otherwise.

Proposal 2: Consider a general statement in RAN2 Technical Specifications or a pointer to TS 38.101 to reflect which requirements apply to FR2-NTN.

Proposal 3: If Proposal 2 is not agreeable, RAN2 agrees a single CR fixing the FR2-NTN requirements for all affected fields/IEs.

Proposal 4: Support RACH-less HO also in inter-gNB scenarios for NTN.

Proposal 5: RAN2 reopens the discussion on whether the UE can measure an NTN cell while camping or being connected to a TN cell. RAN2 reflects RAN4 requirements in the appropriate specifications.

Proposal 6: Align with RAN4 regarding the requirements for SIB19 acquisition in TN cell.

# References

- RP-232669	Revised WID: NR NTN (Non-Terrestrial Networks) enhancements 3GPP TSG RAN Meeting #100 Taipei, Taiwan, June 12-14, 2023

- R1-2407406 LS on FR2-NTN inclusion to specifications 3GPP TSG RAN WG1#118                                               Maastricht, NL, August 19th – 23rd, 2024

- R2-2407573 Report from Break-out session on NR-NTN and IoT-NTN 3GPP TSG-RAN WG2 Meeting #127 Maastricht, Netherlands, Aug 19th – 23rd, 2024

- 3GPP TS 38.101 NR; User Equipment (UE) radio transmission and reception

- R2-2409213 Report from Break-out session on NR-NTN and IoT-NTN 3GPP TSG-RAN WG2 Meeting #127bis Hefei, China, October 14th – 18th, 2024

- R2-2408654 Miscellaneous corrections on NTN in FR2 bands 3GPP TSG-RAN WG2 Meeting #127bis Hefei, China, October 14th – 18th, 2024

- 3GPP TS 38.331 NR; Radio Resource Control (RRC); Protocol specification

- 3GPP TS 38.133 NR; Requirements for support of radio resource management