3GPP TSG-RAN WG2 Meeting #128	R2-2410439

Orlando, USA, 18th – 22nd of November 2024

Agenda item:	8.8.6

Source:	Nokia, Nokia Shanghai Bell

Title:	On LTE TN to NR NTN reselection using redirection in Release message

WID/SID:	NR_NTN_Ph3 - Release 19

Document for:	Discussion and Decision

# 1	Introduction

At the recent RAN2 meeting the following was discussed without being resolved [2]:

Proposal 3: RAN2 confirms the understanding that dedicated NR NTN frequency is not configured in the RRCConnectionRelease message, and confirms that this can be left to NW implementation (w/o Spec impact). - QC is ok with this but thinks it’s fine to clarify this somewhere - Samsung thinks we should separate the case of providing dedicated frequencies from the redirection case. Xiaomi agrees, redirection could be performed providing assistance information in the release message. LG shares similar view as Samsung but thinks the NW does not need to provide assistance info - China Telecom thinks satellite information would be needed to support redirection to NR NTN - QC thinks that either we go for proposal 3 or we need to introduce a UE capability for this. Come back in the next meeting

As can be seen, the views were split, and the topic has been postponed to next meeting. In this paper we provide our understanding on the current status and how this shall be addressed in Release 19.

# 2	Discussion

The scope of the work done in Release 19 for LTE TN to NR NTN objective was to enable the reselection between these two RATs and types of networks (i.e. TN to NTN). This is achieved due to the modifications introduced to LTE system information blocks (e.g. SIB33). The issue raised at RAN2#127bis [2] and quoted in the Introduction is whether the RRCConnectionRelease message in E-UTRAN can comprise the dedicated frequency information which would correspond to NR NTN. This is signalled using redirectedCarrierInfo which is a CHOICE type of IE and defined as follows [3]:

RedirectedCarrierInfo ::= CHOICE { eutra ARFCN-ValueEUTRA, geran CarrierFreqsGERAN, utra-FDD ARFCN-ValueUTRA, utra-TDD ARFCN-ValueUTRA, cdma2000-HRPD CarrierFreqCDMA2000, cdma2000-1xRTT CarrierFreqCDMA2000, ..., utra-TDD-r10 CarrierFreqListUTRA-TDD-r10, nr-r15 CarrierInfoNR-r15, nr-r17 CarrierInfoNR-r17 }

And the corresponding field description states the following [3]:

redirectedCarrierInfo The redirectedCarrierInfo indicates a carrier frequency (downlink for FDD) and is used to redirect the UE to an E-UTRA or an inter-RAT carrier frequency, by means of the cell selection upon leaving RRC_CONNECTED as specified in TS 36.304 [4]. The value geran can only be included after successful security activation when UE is connected to 5GC.

Section 5.2.7 in [4] specifies that: “At reception of RRCConnectionRelease message or RRCEarlyDataComplete message to move the UE into RRC_IDLE or RRC_INACTIVE, UE shall attempt to camp on a suitable cell according to redirectedCarrierInfo, if included in the RRCConnectionRelease message or RRCEarlyDataComplete message.” Thus, in principle the LTE UE that is capable of reading the redirectedCarrierInfo and reselecting to NR upon leaving the RRC_CONNECTED state in E-UTRA, shall attempt to camp on NR cell according to what is indicated in redirectedCarrierInfo.

Observation 1: E-UTRA capable UE shall attempt to camp on NR cell according to what is indicated in redirectedCarrierInfo, if this IE is provided in RRCConnectionRelease.

There is no restriction stating this applies to Terrestrial Network (TN) only.

Observation 2: Neither TS 36.331, nor TS 36.304 restricts the redirection provided in RRCConnectionRelease to TN.

As a part of Rel-19 work RAN2 has extended E-UTRA’s SIB24 and SIB33 to provide the assistance information for reselection to NR NTN, including the satellite information. This is also in line with the WID’s objective [5]:

The work item aims at specifying support for the following objectives: - idle mode mobility based on cell reselection from E-UTRA TN to NR NTN, where E-UTRA TN provides satellite information for NR NTN neighbor cells in a System Information Block. [RAN2]

Observation 3: As per WID in RP-240846, E-UTRA TN provides satellite information for NR NTN neighbor cells in a System Information Block.

Observation 3 does not mean the carrier information needs to be provided in System Information Block as well, just the satellite information (e.g. ephemeris) is sent there.

Observation 4: WID does not indicate the carrier information needs to be provided in System Information Block as well. Just the satellite information (e.g. ephemeris) is sent in SIB.

As of now, the specification is lacking the mapping between the frequency information provided in RRCConnectionRelease and the satellite information (ephemeris) sent in SIB33. Such mapping is introduced in Rel-19 between SIB33 and SIB3/SIB5.

Observation 5: The mapping between the frequency information provided in RRCConnectionRelease and the satellite ephemeris sent in SIB33 does not exist.

In our view, it would be beneficial if such possibility is also supported as a part of Release 19.

Proposal 1: Rel-19 supports the possibility to provide carrier information in redirectedCarrierInfo in RRCConnectionRelease and to map this information to satellite ephemeris signalled in SIB33.

If Proposal 1 is agreeable, RAN2 should discuss on the signalling details. One option is to consider inserting satellite identifier to RRCConnectionRelease message.

Proposal 2: RAN2 decides how to enable the mapping between frequency information in RRCConnectionRelease and ephemeris signalled in SIB33. One option to consider is to insert satellite identifier to RRCConnectionRelease message.

# 3	Conclusion

This document has made the following observations:

Observation 1: E-UTRA capable UE shall attempt to camp on NR cell according to what is indicated in redirectedCarrierInfo, if this IE is provided in RRCConnectionRelease.

Observation 2: Neither TS 36.331, nor TS 36.304 restricts the redirection provided in RRCConnectionRelease to TN.

Observation 3: As per WID in RP-240846, E-UTRA TN provides satellite information for NR NTN neighbor cells in a System Information Block.

Observation 4: WID does not indicate the carrier information needs to be provided in System Information Block as well. Just the satellite information (e.g. ephemeris) is sent in SIB.

Observation 5: The mapping between the frequency information provided in RRCConnectionRelease and the satellite ephemeris sent in SIB33 does not exist.

And proposed the following:

Proposal 1: Rel-19 supports the possibility to provide carrier information in redirectedCarrierInfo in RRCConnectionRelease and to map this information to satellite ephemeris signalled in SIB33.

Proposal 2: RAN2 decides how to enable the mapping between frequency information in RRCConnectionRelease and ephemeris signalled in SIB33. One option to consider is to insert satellite identifier to RRCConnectionRelease message.

# References

- RP-240775 Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3 3GPP TSG RAN Meeting #103 Maastricht, The Netherlands, March 18-22, 2024

- R2-2409213 Report from Break-out session on NR-NTN and IoT-NTN 3GPP TSG-RAN WG2 Meeting #127bis Hefei, China, October 14th – 18th, 2024

- 3GPP TS 36.331 E-UTRAN RRC, v18.3.1, September 2024

- 3GPP TS 36.304 User Equipment (UE) procedures in idle mode, v18.2.0, September 2024

- RP-240846 New WID: Inter RAT mobility support from E-UTRAN TN to NR-NTN 3GPP TSG RAN Meeting #103 Maastricht, The Netherlands, March 18-21, 2024