3GPP TSG-RAN WG2 Meeting #128			           			R2-2410448

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda Item:	8.6.4

Source: 	Lenovo

Title: 	Conditional LTM Scenarios and remaining points

Document for:	Discussion and decision

# Introduction

In RAN plenary in Melbourne instructed RAN2 to work on the Conditional LTM topic. The following agreements were arrived at in RAN2#127bis:

Agreements on C-LTM

- Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions.

- Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions.

- Source cell and each candidate cell provides its own execution condition for conditional LTM.

- It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used.

- RACH-less Conditional intra-CU LTM is supported.

- RACH based conditional intra-CU LTM is supported.

- UE based TA measurement mechanism is supported for conditional intra-CU LTM.

- PDCCH ordered early TA acquisition is supported for conditional LTM.

- Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM.

- For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach.

- The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

This document addresses the remaining points for the C-LTM work.

# Discussion

The Intra-CU LTM feature has been specified in Rel. 18 and some further enhancements (Inter-CU) is in progress in Rel. 19. Here UE receives and RRC configuration message allowing the UE to receive RRC Configuration of one or more candidate cells. In the due course of the procedure, UE initiates an Early TA for one or more candidate cell based on receiving PDCCH order. An Early TA may be determined and sent to the UE in LTM MAC CE, if the UE has already measured and reported at least one target cell. The C-LTM work however needs some further discussion:

Early TA delivery

RAN2 agreed in the last meeting that: “PDCCH ordered early TA acquisition is supported for conditional LTM.” But it is unclear if the early TA will be provided to the UE from source cell, like in Rel. 18, or in a RAR message from the candidate cell to the UE directly?

Here, it can be argued that UE should obtain TA of a candidate cell (for which it received PDCCH order and made the Msg1 transmission) directly from that candidate cell, instead of Rel. 18 way of receiving it from source cell. However, this has no additional advantage and in fact the UE will be attempting to receive Msg2 from the candidate cell, and therefore depending on UE capability may not be available on the source for data reception. In addition, this will be a change compared with Rel. 18 baseline without offering any benefits. Therefore, we propose:

Proposal 1: The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE).

In conditional LTM, UE will receive LTM candidate configuration and execution condition. The framework of LTM candidate configuration should be same as legacy LTM candidate configuration. Then, the question is whether LTM cell switch command MAC CE can be used to trigger LTM cell switch toward conditional LTM candidate cell. If L1 measurement report is configured, source DU can decide whether to trigger LTM cell switch via LTM cell switch command MAC CE before the condition is met.

Proposal 2: RAN2 to discuss whether LTM cell switch command MAC CE can be used to trigger LTM cell switch toward conditional LTM candidate cell.

As the next logical question, RAN2 should decide if an Early TA is signalled using a new MAC CE or the same MAC CE as in Rel. 18 i.e., “LTM Cell Switch Command MAC CE”. We think this is technically plausible and a new MAC CE is not required. Some stage-3 discussion on this will of course be necessary but for now the following proposal suffices:

Proposal 3: The Rel. 18 LTM Cell Switch Command MAC CE is reused to provide Early TA, TCI states etc. to the UE for C-LTM purpose without triggering a mobility execution directly.

Co-existence of conditional LTM and CHO/LTM

Now in Rel. 19, as part of this work, an alternative to the above LTM procedure is to be specified, where a conditional handover reconfiguration message can be used by a gNB to transmit a message (e.g., RRCReconfiguration) to the UE including both LTM candidate cell configurations as well as their corresponding Execution Condition(s). In addition, the network may include CHO configuration (with execution condition) for some of the C-LTM candidates. Whether the CHO configuration(s) for the C-LTM Cell is included in the same RRC Reconfiguration or a separate one from the one including the C-LTM configuration can be left to network implementation but RAN2 should avoid that RRCReconfiguration is provided in three places (inside conditionalReconfiguration, inside ltm-CandidateConfig and then in a new IE for C-LTM) for the same PCI.

Since for CHO we agreed in Rel. 18 these are CHO and LTM are two independent configurations. Now, to avoid that the reconfiguration contains yet another RRCReconfiguration, what we can do is to include LTM condition (LTM Event) in the LTM candidate configuration that are being configured by the network for C-LTM. The remaining LTM Candidates shall not include any conditions i.e., rel. 18 structure LTM-Candidate-r18 can be used as is.

- LTM candidate, with or without corresponding execution condition. When an execution candidate is not included then the candidate cell is considered for LTM only.

- CHO execution configuration for one or more included C-LTM candidate. This is like extending the Rel. 18 LTM-CHO possibility to C-LTM candidates.

Proposal 4: To configure conditional LTM, a Serving gNB transmits a RRCReconfiguration to the UE including Rel-18 LTM candidate configurations and Rel-19 conditional MCG LTM.

Observation 1: Both CHO and Intra-CU MCG LTM for the same candidate cell can be configured to UE in Rel-18.

Proposal 5: The serving gNB can transmit RRC reconfiguration to UE including both CHO candidate configuration and C-LTM candidate configuration to the same cell.

Co-existence of condition SCG LTM and MCG failure recovery

In Rel-18 Intra-CU SCG LTM, Intra-CU SCG LTM switch is allowed while MCG failure recovery procedure is ongoing. More specifically, UE will trigger fast MCG link recovery upon RLF on MCG. After UE transmits MCGFailureInformation message to MCG via SCG, UE starts T316 and waits for the response e.g., handover command. Intra-CU SCG LTM could be triggered while T316 is running. That means the response will be pending in SCG until UE completes SCG change triggered LTM. Regarding conditional Intra-CU SCG LTM, we can follow the principle of SCG LTM.  Therefore, we propose:

Proposal 6: Conditional Intra-CU SCG LTM switch is allowed to be triggered if the execution condition is met while MCG failure recovery procedure is ongoing as legacy Intra-CU SCG LTM.

Scenario Discussion based on timeline of Early TA reception and C-LTM Condition fulfillment

The LTM Condition fulfilment and Early TA availability are two independent events that may occur at different point of time. In the following a few scenarios are depicted and also some basic solutions are mentioned, RAN2 should discuss these.

Scenario 1

In case of conditional handover procedure, a handover may only be executed by the UE when included condition(s) for at least one candidate cell are fulfilled. The fulfilment of radio condition(s) is not dependent on acquisition of an Early TA – UE receives it from the source.

One question therefore arises: What if at the point of fulfilment of included condition(s) for a first candidate cell, the early TA acquisition of that candidate cell has not been received by the UE? In other words, if an Early TA value is not yet available with the UE but radio condition(s) for a potential candidate is fulfilled, should it initiate afresh RACH procedure towards the candidate (target) cell?

Figure 1: Condition(s) for a candidate cell C1 is fulfilled at point A, i.e., before receiving a PDCCH Order for C1

In our opinion, the UE shall start handover procedure execution immediately after Point A. To this end, the UE shall use the PRACH resource indicated in the Conditional Configuration message – which may include a Contention Free PRACH resources. The RACH Msg1 transmission shall include an implicit/ explicit indication that the UE is awaiting a timing advance in Msg2 and the same need not be sent to the source cell.

Proposal 7: If the LTM condition is fulfilled before a PDCCH order for the same target cell is received or the Msg1 transmission has not started, RACH Msg1 transmission shall include an implicit/ explicit indication that the UE is awaiting a timing advance in Msg2 and the same need not be sent to the source cell.

Scenario 2

Figure 2: Point A is after transmitting Msg1 (a PDCCH Order for C1 was received)

Scenario 2 depicted in Figure 2 similar to Scenario1 but here the UE can wait to receive a valid TA from the source cell if Msg1 has already been transmitted. Or, to have unified behaviour same solution as in Scenario 1 applied. If however, CFRA resources distinguishing if the TA is sent back to the UE (and not to the source cell) is not available, UE may initiate CBRA, if Msg1 has not yet been transmitted.

Proposal 8: RAN2 to discuss correct UE behaviour when the LTM condition is fulfilled, a PDCCH order for the same target cell is received and the Msg1 transmission has started, UE may wait to receive an “Early” TA from the source or may initiate CBRA transmission.

Scenario 3

UE starts condition evaluation immediately, as soon as possible based on the measurement gaps received in the configuration, of one or more candidate cells. In one example, as shown in Figure 3, the condition(s) for a candidate cell C1 is fulfilled at point A, i.e., after receiving a TA for C1 from the source cell but at which point it is unclear if the TA for C1 is still valid.

Figure 3: Condition(s) for a candidate cell C1 is fulfilled at point A, i.e., after receiving an Early TA for C1

UE starts a TA-timer, which may have been configured earlier in the Conditional Configuration message.

If Point A occurs before the expiry of this timer (Case1 in Figure 3), UE transmits handover completion. If Point A occurs after the expiry of this timer (Case2 in Figure 3), UE shall start handover procedure execution immediately after Point A. To this end, the UE shall (re)use the PRACH resource indicated in the Conditional Configuration message irrespective of if a Msg1 transmission to cell C1 has been made already. Previously received configuration may include a Contention Free PRACH resources. The RACH Msg1 transmission shall include an implicit/ explicit indication that the UE is awaiting a timing advance in Msg2 and the same need not be sent to the source cell.

Proposal 9: A TA-timer is used to check the validity of received TA value. If LTM execution is fulfilled before the expiry of the TA-timer, UE sends the HO Complete to the target cell using RACH-less LTM procedure; else, UE may start a CFRA indicating that Msg2 be sent to the UE directly (not to the source cell).

# Conclusion

This document attempted to develop a basic understanding of “conditional LTM” and proposed some first scenarios for RAN2 consideration. In summary, we have following observations and proposals:

Early TA delivery

Proposal 1: The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE).

Proposal 2: RAN2 to discuss whether LTM cell switch command MAC CE can be used to trigger LTM cell switch toward conditional LTM candidate cell.

Proposal 3: The Rel. 18 LTM Cell Switch Command MAC CE is reused to provide Early TA, TCI states etc. to the UE for C-LTM purpose without triggering a mobility execution directly.

Co-existence of conditional LTM and CHO/LTM

Proposal 4: To configure conditional LTM, a Serving gNB transmits a RRCReconfiguration to the UE including Rel-18 LTM candidate configurations and Rel-19 conditional MCG LTM.

Observation 1: Both CHO and Intra-CU MCG LTM for the same candidate cell can be configured to UE in Rel-18.

Proposal 5: The serving gNB can transmit RRC reconfiguration to UE including both CHO candidate configuration and C-LTM candidate configuration to the same cell.

Co-existence of condition SCG LTM and MCG failure recovery

Proposal 6: Conditional Intra-CU SCG LTM switch is allowed to be triggered if the execution condition is met while MCG failure recovery procedure is ongoing as legacy Intra-CU SCG LTM.

Scenario Discussion

Proposal 7: If the LTM condition is fulfilled before a PDCCH order for the same target cell is received or the Msg1 transmission has not started, RACH Msg1 transmission should be started and shall include an implicit/ explicit indication that the UE is awaiting a timing advance in Msg2 and the same need not be sent to the source cell.

Proposal 8: RAN2 to discuss correct UE behaviour when the LTM condition is fulfilled, a PDCCH order for the same target cell is received and the Msg1 transmission has started, UE may wait to receive an “Early” TA from the source or may initiate CBRA transmission.

Proposal 9: A TA-timer is used to check the validity of received TA value. If LTM execution is fulfilled before the expiry of the TA-timer, UE sends the HO Complete to the target cell using RACH-less LTM procedure; else, UE may start a CFRA indicating that Msg2 be sent to the UE directly (not to the source cell).

# References

- Chair notes for RAN2#127bis