3GPP TSG-RAN WG2 #128	R2-2410458

# Orlando, USA, 18th – 22nd November 2024

Agenda Item:	7.0.2.17

Source:	Ericsson

Title:	Coexistence of LTM with (e)RedCap

Document for:	Discussion

# 1	Introduction

In the last RAN2 meeting, the coexistence of LTM with (e)RedCap was discussed, but at the end any decision was postponed to give more time to companies to check this aspect. In this contribution we address whether is possible to configure LTM for RedCap and eRedCap UEs.

# 2	Discussion

## 2.1	L1 measurements on NCD-SSB

For RedCap it is possible to configure the UE to measure NCD-SSB which are outside of the active DL BWP. According to TS 38.331, a NCD-SSB is configured according to the following:

-------------

#### –	NonCellDefiningSSB

The IE NonCellDefiningSSB is used to configure a NCD-SSB to be used while the UE operates in an RedCap-specific initial BWP that does not contain the CD-SSB or a dedicated BWP that does not contain the CD-SSB.

NonCellDefiningSSB information element

-- ASN1START

-- TAG-NONCELLDEFININGSSB-START

NonCellDefiningSSB-r17 ::=      SEQUENCE {

absoluteFrequencySSB-r17        ARFCN-ValueNR,

ssb-Periodicity-r17             ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 }       OPTIONAL,   -- Need S

ssb-TimeOffset-r17              ENUMERATED { ms5, ms10, ms15, ms20, ms40, ms80, spare2, spare1 }      OPTIONAL,   -- Need S

...

}

-- TAG-NONCELLDEFININGSSB-STOP

-- ASN1STOP

NonCellDefiningSSB field descriptions

absoluteFrequencySSB Frequency of the NCD-SSB. The network configures this field so that the SSB is within the bandwidth of the BWP configured in BWP-DownlinkCommon.

ssb-Periodicity The periodicity of this NCD-SSB. The network configures only periodicities that are larger than the periodicity of serving cell's CD-SSB. If the field is absent, the UE applies the SSB periodicity of the CD-SSB (ssb-periodicityServingCell configured in ServingCellConfigCommon or ServingCellConfigCommonSIB).

ssb-TimeOffset The time offset between CD-SSB of the serving cell and this NCD-SSB. Value ms5 means the first burst of NCD-SSB is transmitted 5ms later than the first burst of CD-SSB transmitted after the first symbol of SFN=0 of the serving cell, value ms10 means the first burst of NCD-SSB is transmitted 10ms later than the first burst of CD-SSB transmitted after the first symbol in SFN=0 of the serving cell, and so on. If the field is absent, UE considers that the time offset between the first burst of CD-SSB transmitted in the serving cell and the first burst of this NCD-SSB transmitted is zero. For UEs in TDD cells, the network configures this time offset to be an integer multiple of the periodicity of the serving cell's CD-SSB.

-----------

According to the text in yellow, there is a dependency between the configuration of an NCD-SSB and a CD-SSB. In fact, if some of the fields for the NCD-SSB are absent the UE should use what is part of a configuration which is part of an CD-SSB.

There is a dependency between the configuration of an NCD-SSB and a CD-SSB.

A further aspect is that NCD-SSB can be configured only for the serving cell, as the serving cell is aware of the configuration of its own SSBs and BWPs. This is also reflected in some configuration aspects such as (to highlight a few):

- “The network configures only periodicities that are larger than the periodicity of serving cell's CD-SSB”.

- “If the field is absent, the UE applies the SSB periodicity of the CD-SSB (ssb-periodicityServingCell configured in ServingCellConfigCommon or ServingCellConfigCommonSIB”.

- For UEs in TDD cells, the network configures this time offset to be an integer multiple of the periodicity of the serving cell's CD-SSB.

When considering LTM, according to TS 38.331 for each LTM candidate we already indicate the SSB configuration which is used by the UE to perform LTM L1 measurements. In fact, we have:

-------------

#### –	LTM-Candidate

The IE LTM-Candidate concerns a LTM candidate configuration to add or modify.

LTM-Candidate information element

-- ASN1START

-- TAG-LTM-CANDIDATE-START

LTM-Candidate-r18 ::=     SEQUENCE {

ltm-CandidateId-r18                            LTM-CandidateId-r18,

ltm-CandidatePCI-r18                           PhysCellId                                            OPTIONAL,    -- Need M

ltm-SSB-Config-r18                             LTM-SSB-Config-r18                                    OPTIONAL,    -- Need M

ltm-CandidateConfig-r18                        OCTET STRING (CONTAINING RRCReconfiguration)          OPTIONAL,    -- Need M

ltm-ConfigComplete-r18                         ENUMERATED {true}                                     OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfig-r18                     OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)      OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfigSUL-r18                  OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)      OPTIONAL,    -- Need R

ltm-TCI-Info-r18                               LTM-TCI-Info-r18                                      OPTIONAL,    -- Need M

ltm-NoResetID-r18                              INTEGER (1..maxNrofLTM-Configs-plus1-r18)             OPTIONAL,    -- Need M

ltm-UE-MeasuredTA-ID-r18                       INTEGER (1..maxNrofLTM-Configs-plus1-r18)             OPTIONAL,    -- Need M

...

}

LTM-SSB-Config-r18 ::= SEQUENCE {

ssb-Frequency-r18                              ARFCN-ValueNR,

subcarrierSpacing-r18                          SubcarrierSpacing,

ssb-Periodicity-r18                            ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1} OPTIONAL,   -- Need R

ssb-PositionsInBurst-r18                       CHOICE {

shortBitmap                                    BIT STRING (SIZE (4)),

mediumBitmap                                   BIT STRING (SIZE (8)),

longBitmap                                     BIT STRING (SIZE (64))

}                                                                                                    OPTIONAL,   -- Need R

ss-PBCH-BlockPower-r18                         INTEGER (-60..50)                                     OPTIONAL,   -- Need R

...

}

-- TAG-LTM-CANDIDATE-STOP

-- ASN1STOP

-------------

Therefore, it could be already possible to configure the UE to perform L1 LTM measurements on an NCD-SSB, as far as there is a L3 measurement object configured for a NCD-SSB and as far as the frequency of the SSB indicated within an LTM candidate cell configuration is the same as the SSB frequency of the NCD-SSB indicated in the measurement object.

As far as the as the frequency of the SSB indicated within an LTM candidate cell configuration is the same as the SSB frequency of the NCD-SSB indicated in a L3 measurement object is the same, the UE should be able to perform LTM L1 measurements on a NCD-SSB.

## 2.2	Use of additional RACH configuration

During Rel-17 a new feature called RACH partitioning was introduced. With this feature, the network can configure multiple RACH resources to the UE (i.e., increasing thus the number of RACH preambles) and associate a set of RACH resources with feature(s) applicable to a Random Access procedure, such as for network Slicing, RedCap, SDT, and NR coverage enhancement.

In LTM, the RACH resources are used by the UE to perform the Random Access procedure upon an LTM cell switch procedure, but also to perform the Random Access procedure in case of an early UL synchronization procedure is triggered by the serving cell. In this procedure, the UE sends a RACH preamble to an LTM candidate cell and such LTM candidate cell estimates a TA value which is then sent back to the serving cell. Two points that is good to highlight is that i) only one RACH resource configuration is exchanged between the serving cell and a LTM candidate cell (for the early uplink synchronization procedure) and ii) the target cell uses the preamble index (among other fields included in the early uplink synchronization configuration) to identify to which serving cell the calculated TA value should be sent.

For LTM, only one RACH resource configuration is exchanged between one LTM candidate cell (for the early uplink synchronization procedure) and the target cell uses the preamble index (among other fields included in the early uplink synchronization configuration) to identify to which serving cell the calculated TA value should be sent.

According to this, while is it clear that a RedCap UE may use a configured additional RACH configuration to perform the Random Access procedure towards an LTM cell switch, it is also clear that the additional RACH configuration cannot be used in at least two cases: i) when the Random Access procedure is performed for the early UL synchronization procedure, and when ii) the RACH resources are indicated in the LTM cell switch MAC CE.

For LTM, it is also clear that the additional RACH configuration cannot be used when the Random Access procedure is performed for the early UL synchronization procedure, and when the RACH resources are indicated in the LTM cell switch MAC CE.

## 2.3	Support of LTM for (e)RedCap UEs

According to the analysis in section 2.1 and 2.2, it is possible to conclude that LTM can be configured for (e)RedCap UEs, but in order to allow this, few clarifications need to be made. The first one is that if the network wants to configure LTM L1 measurements on an NCD-SSB, the SSB frequency which is configured for an LTM candidate cell configuration shall be the same as the one configured within the NonCellDefiningSSB IE.

The second one is that the RACH configuration, even if configured, shall not be used by the UE when the Random Access procedure is performed for the early UL synchronization procedure, and when the RACH resources are indicated in the LTM cell switch MAC CE.

Therefore, we propose:

RAN2 to confirm that LTM can be used for (e)RedCap UEs by clarify the following:

  - If the SSB frequency within the LTM-SSB-Config IE is the same as the SSB frequency within the NonCellDefiningSSB IE, this means the SSB is a NCD-SSB.

  - If additional RACH resources are configured, the UE shall use such resources only when performing an LTM cell switch procedure, and if CFRA resources are not provided within the LTM cell switch MAC CE.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	There is a dependency between the configuration of an NCD-SSB and a CD-SSB.

Observation 2	As far as the as the frequency of the SSB indicated within an LTM candidate cell configuration is the same as the SSB frequency of the NCD-SSB indicated in a L3 measurement object is the same, the UE should be able to perform LTM L1 measurements on a NCD-SSB.

Observation 3	For LTM, only one RACH resource configuration is exchanged between one LTM candidate cell (for the early uplink synchronization procedure) and the target cell uses the preamble index (among other fields included in the early uplink synchronization configuration) to identify to which serving cell the calculated TA value should be sent.

Observation 4	For LTM, it is also clear that the additional RACH configuration cannot be used when the Random Access procedure is performed for the early UL synchronization procedure, and when the RACH resources are indicated in the LTM cell switch MAC CE.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to confirm that LTM can be used for (e)RedCap UEs by clarify the following:

a.	If the SSB frequency within the LTM-SSB-Config IE is the same as the SSB frequency within the NonCellDefiningSSB IE, this means the SSB is a NCD-SSB.

b.	If an additional RACH resources are configured, the UE shall use such resources only when performing an LTM cell switch procedure, and if CFRA resources are not provided within the LTM cell switch MAC CE.