3GPP TSG-RAN WG2 Meeting #128	R2-2410465

Orlando, USA, Nov 18th – 22nd, 2024

Agenda Item:	8.6.4

Source:	Panasonic

Title: 	Discussion on conditional intra-CU LTM

Document for:	Discussion

# Introduction

In RAN2#127bis meeting following agreements have been made for intra-CU LTM.

Agreements on C-LTM

- Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions.

- Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions.

- Source cell and each candidate cell provides its own execution condition for conditional LTM.

- It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used.

- RACH-less Conditional intra-CU LTM is supported.

- RACH based conditional intra-CU LTM is supported.

- UE based TA measurement mechanism is supported for conditional intra-CU LTM.

- PDCCH ordered early TA acquisition is supported for conditional LTM.

- Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM.

- For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach.

- The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

In this document we further discuss some of the open issues related to resource utilization during RACH-less conditional LTM, either during initial C-LTM or subsequent C-LTM, which can be configuring using CG-based or DG-based resource allocation with each of the configured C-LTM candidate cells.

# Discussion

In the previous meeting, to support RACH-less C-LTM was agreed where CG-based first UL transmission was seen suitable but for DG-based approach further studies was required. But considering that C-LTM candidate configuration is sent to UE during C-LTM preparation phase, and as per the basic principle of C-LTM procedure if UE is expected to perform initial C-LTM cell switch and followed by subsequent C-LTM cell switch without any intervention from the gNB, then eventually the CG resources for RACH-less execution with each of the C-LTM candidate cells must be allocated to one UE at an early stage of C-LTM preparation to allow for initial and subsequent C-LTM executions. This results in inefficient utilization of radio resources at the candidate cells.

Observation 1: CG-based RACH-less C-LTM results in inefficient resource utilization at the candidate cells, if the configured resources are reserved for UE to execute initial C-LTM cell switch and continued to be reserved for subsequent C-LTM cell switch.

However, if the serving cell can interpret the potential candidate cells during C-LTM evaluation performed at UE, then only a subset of candidate cells (one or more potential cells) can be indicated about the potential UE-entry to reserve resources (CG-based or DG-based) for a shorter duration of time until UE executes C-LTM cell switch to one of the identified candidate cells/beams. Such interpretation can be determined by the serving cell (for e.g. using L3/L1 measurements as reported by UE), or UE informs the potential candidate cell to UE. Here at least one of the options must be agreed by RAN2 to progress and study further about how serving cell or UE can determine the potential candidate cells.

Observation 2: To arrive conclusion about how network can interpret the potential candidate cells during C-LTM evaluation, it is important for RAN2 to progress on the aspect of who will provides the information about potential candidate cell.

Proposal 1: RAN2 to discuss how CG-based resource allocation can be utilized effectively when configured for C-LTM candidate cells, where resources are dedicated/ reserved for the potential UE-entry during initial C-LTM execution or subsequent C-LTM execution. Two options can address this issue,

Option1: Serving cell determines the potential target cell and indicates potential UE-entry to C-LTM candidate cells, during initial C-LTM or subsequent C-LTM.

Option2: UE indicates the potential target cell to serving cell and serving cell interpret such information to indicate potential UE-entry to C-LTM candidate cell.

It was also discussed in the last meeting about the proposal to determine the potential target cells/ beams either by NW estimation, based on L3/L1 measurement reporting from UE or UE determining its potential target beam during LTM event evaluation and report such information to the serving cell, but no conclusion had arrived on this aspect. For option-1 where the network can determine potential candidate cell, there could be more effective way where the serving cell’s interpretations and the UE evaluation for C-LTM execution can be more co-ordinated. One such solution is discussed below.

It is evident that such issue of inefficient resource utilization was not observed in Rel-18 LTM, because when network decides to switch to a candidate cell/ beam a cell switch command was sent to UE to trigger cell switch execution, such command acts as the trigger point from where the candidate cells also would be indicated about the UE arrival and hence the CG or DG resource allocation from the target cell was effectively utilized. Similar principle can also be applied for conditional LTM where the network can trigger the start of C-LTM evaluation with execution conditions configured at UE, for the indicated one of more C-LTM candidate cells/ beams.

Observation 3: Reusing Rel-18 LTM principle like cell switch command also for conditional LTM, to trigger the start of event evaluation with one or more indicated C-LTM candidate cells (to indicate the potential C-LTM cell switch), can improve resource allocation efficiency either for CG-based or DG-based resources configured at UE.

Figure-1: C-LTM activation command sent to UE to start evaluation for C-LTM execution conditions

One such scenario is illustrated in figure-1, here the advantage of using ‘C-LTM activation command’ (sent from source cell to UE) is that the serving cell can determine the potential candidate cell considering both the radio conditions at UE and the network conditions at each of the neighbouring cells, and then suitable candidate cells (with one or more cells among the configured C-LTM candidate cells) could be activated to start evaluation for C-LTM execution conditions and cell switch when event conditions are satisfied. Such solution provides more co-ordinated operation between the UE and the candidate cell to execute RACH-less UE entry in C-LTM.

Observation 4: C-LTM candidate activation (activation command sent from source cell to UE to start C-LTM event condition evaluation) can consider both radio conditions and the network conditions to select suitable candidate cells/beams for handover.

Observation 5: C-LTM candidate activation command provides more co-ordination between UE and the target cell to execute RACH-less C-LTM cell switch.

Proposal 2: When serving cell determines potential (one or more) candidate cells/beams based on the radio conditions at UE and the network conditions at each cell/beam, C-LTM activation command can activate C-LTM event evaluation at UE for one or more candidate cells and alongside indicate to those candidate cells to reserve CG-based resources for UE entry, for a co-ordinated RACH-less C-LTM cell switch.

Proposal 3: With C-LTM candidate activation mechanism, DG-based resource allocation for RACH-less C-LTM can be supported.

Based on the requirement for cell evaluation activation sent to UE using C-LTM candidate activation command, C-LTM procedure may have other open issues to consider at different execution phases during C-LTM evaluation phase or execution phase. Some of these are discussed below.

LTM configuration and preparation phase.

To support conditional LTM candidate activation command to start evaluation of event conditions, C-LTM configurations sent to UE during LTM preparation phase should not be used to start evaluation for event conditions. Here UE rather waits for the network decision indicated using candidate cell activation command either during initial C-LTM or subsequent C-LTM.

Proposal 4: UE need not start evaluation for C-LTM event conditions when candidate configuration for C-LTM is sent during C-LTM preparation phase, until C-LTM activation command is sent to start evaluation with indicated candidate cells during initial C-LTM or subsequent C-LTM procedure.

Proposal 4a: Network can configure C-LTM candidate configuration consisting of one or more candidate cells with CG-based (or DG-based) resource allocation, but the candidate cells need not reserve such resources (e.g. CG-based resources) until network sends C-LTM activation command to UE.

Early synchronization performed using C-LTM cell switch command.

Further when candidate cell activation command to start conditional LTM evaluation for one or more candidate cells are sent to UE, it can also be accommodated with early synchronization indication for each of the indicated candidate cells. For example, TCI state activation or PDCCH order to trigger TA acquisition can follow along the C-LTM activation command.

Proposal 5: Early synchronization using TCI state activation or PDCCH ordered TA acquisition can be performed along with C-LTM activation command, with the indicated one or more candidate cells.

# Conclusions

Generic issue in C-LTM:

Observation 1: CG-based RACH-less C-LTM results in inefficient resource utilization at the candidate cells, if the configured resources are reserved for UE to execute initial C-LTM cell switch and continued to be reserved for subsequent C-LTM cell switch.

Observation 2: To arrive conclusion about how network can interpret the potential candidate cells during C-LTM evaluation, it is important for RAN2 to progress on the aspect of who will provides the information about potential candidate cell.

Proposal 1: RAN2 to discuss how CG-based resource allocation can be utilized effectively when configured for C-LTM candidate cells, where resources are dedicated/ reserved for the UE to use during initial C-LTM execution and continued subsequent C-LTM executions. Two options can address this issue,

Option1: Serving cell determines the potential target cell and indicates potential UE-entry to C-LTM candidate cells, during initial C-LTM or subsequent C-LTM.

Option2: UE indicates the potential target cell to serving cell and serving cell interpret such information to indicate potential UE-entry to C-LTM candidate cell.

C-LTM candidate activation command:

Observation 3: Reusing Rel-18 LTM principle like cell switch command also for conditional LTM, to trigger the start of event evaluation with one or more indicated C-LTM candidate cells, can improve resource allocation efficiency either for CG-based or DG-based resources configured at UE.

Observation 4: C-LTM candidate activation (activation command sent from source cell to UE to start C-LTM event condition evaluation) can consider both radio conditions and the network conditions to select suitable candidate cells/beams for handover.

Observation 5: C-LTM candidate activation command provides more co-ordination between UE and the target cell to execute RACH-less C-LTM cell switch.

Proposal 2: When serving cell determines potential (one or more) candidate cells/beams based on the radio conditions at UE and the network conditions at each cell/beam, C-LTM activation command can activate C-LTM event evaluation at UE for one or more candidate cells and alongside indicate to those candidate cells to reserve CG-based resources for UE entry, for a co-ordinated RACH-less C-LTM cell switch.

Proposal 3: With C-LTM candidate activation mechanism, DG-based resource allocation for RACH-less C-LTM can be supported.

C-LTM candidate preparation with C-LTM candidate activation:

Proposal 4: UE need not start evaluation for C-LTM event conditions when candidate configuration for C-LTM is sent during C-LTM preparation phase, until C-LTM activation command is sent to start evaluation with indicated candidate cells during initial C-LTM or subsequent C-LTM procedure.

Proposal 4a: Network can configure C-LTM candidate configuration consisting of one or more candidate cells with CG-based (or DG-based) resource allocation, but the candidate cells need not reserve such resources (e.g. CG-based resources) until network sends C-LTM activation command to UE.

Early synchronization with C-LTM candidate activation:

Proposal 5: Early synchronization using TCI state activation or PDCCH ordered TA acquisition can be performed along with C-LTM activation command, with the indicated one or more candidate cells.