3GPP TSG-RAN WG2 Meeting #128	R2-2410468

Orlando, USA, 18th Nov – 22nd Nov 2024

Agenda item:	8.9.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Radio Interface Impacts of SF operation

WID/SID:	IoT_NTN_Ph3 - Release 19

Document for:	Discussion and Decision

# 1	Introduction

Following are the agreements in RAN2 on the SF operation for IoT-NTN until RAN2-127.

RAN2-126 Agreements:

1.	For the uplink/downlink messages transmission for MO, from RAN2 perspective the following steps are taken as baseline for S&F satellite operation (in case only eNB is on the satellite):

1) The UE sends uplink data signalling to eNB when service link is available and the eNB stores it.

2) When feeder link is available, the eNB sends the uplink data/NAS signalling to the CN.

3) The eNB (same or different) receives the downlink data/NAS signalling from the CN and stores it when feeder link is available (and service link is not available).

4) The eNB (same or different) sends the downlink data/signalling to the UE when service link is available again

2.	S&F indication can be provided by SIB (FFS on the details). RAN2 assumes that no NAS indication is needed

3.	RAN2 understands legacy UEs may be barred by legacy cellBarred and cellBarredNTN

RAN2-127 Agreements:

1.	RAN2 adopts the SA2 study conclusions on the possible S&F architectures as the baseline for further discussion (RAN2 will only consider the full CN and spit-MME payload options)

2.	RAN2 will consider both single satellite pass and multiple satellite pass scenarios

3.	RAN2 will consider both MO and MT data within scope

4.	UE is informed whether its serving satellite is currently operating in S&F via System Information broadcast (FFS if we also need a static indication that in general the NW supports the feature)

5.	RAN2 does not further discuss whether legacy UEs will always need to be barred in a S&F network, at least not before further progress in SA2 discussion. If there will be a need for this, mechanisms to bar legacy UEs are already in place and no further impact on RAN2 specs in expected.

7.	RAN2 agrees there will be a way to bar legacy UEs (using legacy cellBarred and/or cellBarred-NTN bit) and still allow R19 S&F capable UEs. FFS on the exact solution (e.g. new barring bits or whether this is linked to some other indication)

Agreements:

1.	The dynamic indication that “the cell is operating in S&F mode” is called “S&F operation” indication (we can come back on the exact name when putting this in the spec, if needed)

2.	RAN2 assumes that if an indication that “the cell is operating in S&F mode” is not provided in a NTN cell, the UE should assume that the NTN cell is operating in real-time mode (i.e. default/normal/”not-S&F” mode). RAN2 assumes there should be no distinction between the case that the UE is served by a NTN cell that do not support S&F capability and the case that the UE is served by a NTN cell that does support S&F capability but indication that “the cell is operating in S&F mode” is not provided.

3.	An S&F explicit capability indication by the serving cell, conceived as a static indication of whether the S&F capability is supported or not by a specific satellite/NTN-cell, in addition to the indication that “the cell is operating in S&F mode”, is not needed

4.	If the “S&F operation” indication is not broadcast, a Rel-19 UE (regardless whether supporting S&F or not) shall follow the legacy barring procedure

5.	When present, the “S&F operation” indication has two possible settings:

‘1’: the cell is operating in S&F mode for all UEs (Rel-19 UEs supporting S&F are allowed to access the cell)

‘0’: the cell is operating in S&F mode for UEs in Connected mode (which are not required to monitor the “S&F indication”), but idle Rel-19 UEs supporting S&F are barred (Rel-19 UEs not supporting S&F will follow legacy barring procedure).

# 2	Discussion

## 2.1 Remaining Issues related to SF Operation indication

In RAN2-127bis, introduction of new parameter ‘SF Operation’ to indicate the cell support for SF mode and also its operational status is agreed. The absence of this flag implicitly indicates the cell is operating in Real-Time (RT) /Normal mode.  Handling of mode transition and also the impacts to the normal traffic (non delay tolerant) and how the SF UE can handle the SF operation and the mode changes requires further analysis with this agreement.

With the above agreement the mode transition can be achieved using current system information modification procedure as below.

- The cell is operating in SF mode operation during most of the time and only switches to normal mode for short duration when feeder link is available.

- When feeder link is available, the System Information will be modified to exclude the SF-Operation parameter. If the Rel-19 Flag for barring normal devices was set to barred the NW may release the barring if it wants to allow the Rel-19 UE during this window.

- This change needs to be indicated via system information modification procedure that also involves paging to all the camped devices.

- For SF mode capable UE this absence is indication that the UE can also start the transmission of normal traffic now.

- When feeder link becomes unavailable, the mode change needs to be again triggered via system information modification procedure.

For short duration of feeder link connectivity there will be two system information modification procedures needed that triggers paging to all the UE.  These steps take additional signaling towards UE and also impacts the energy efficiency of the camped SF capable UE.

Observation 1: Use of system Information modification procedure for switching between SF operation and normal operation is not signaling efficient considering the short duration of normal operation window for SF mode cell.

The system information modification for SF operation change can be avoided with introduction of parameter similar to T-Service. However this parameter and related UE behaviour is different from T-Service. So new parameter will be required.  The modified system behaviour with the new parameter is illustrated below.

- When cell is operating in SF mode it is aware of the time for CN connectivity. This absolute time is included in the system information as “T-SF-Service”.

- The UE camped onto this cell knows the remaining time based on this parameter and also switches its mode on expiry of this timer value.

- NW also changes the mode in system information. However the system information modification procedure is not triggered. NW now also changes the time value based on the duration of expected feeder link connectivity.

- Here UE is also expected to read the new value of the T-SF-Service on expiry from the system information and restart the timer for the new operation mode.

Proposal 1: New parameter T-Service-SF is introduced to support the transition of SF operation from SF mode to Normal and vice-versa.

Proposal 2: On expiry of the T-Service-SF the UE reads the new value of T-Service-SF and also the new SF operation related parameters to start the validity timer for the new mode.

Whether system information value tag is still needed to support other scenarios of UE entering into this cell after out of coverage or power up scenario needs further discussion.

Proposal 3: FFS need for system information value tag in addition to T-Service-SF parameter.

Need for additional parameters in the system information to indicate the maximum expected delay for the services was proposed from companies during RAN2-127bis.  As the NAS level registration procedure for SF capable UE already includes information about wait timer for next cell access and also the list of satellite-ID for next access that can help the UE to estimate the time of storage this additional information is not essential. Moreover whether the response can be delivered for MO traffic in next access is not fully guaranteed and it depends on the storage and other UE traffic at the time of feeder link connectivity. So this information will not be accurate.

Proposal 4: Additional information related to maximum delay impact of SF operation is not broadcasted in system information. UE can deduce this information from other parameters related SF operation provided in NAS level procedures.

## 2.2 RAN signalling procedure changes for MME Deployment Options

RAN2 127 made following agreement related to the architecture options for further RAN2 impacts

RAN2 adopts the SA2 study conclusions on the possible S&F architectures as the baseline for further discussion (RAN2 will only consider the full CN and spit-MME payload options)

The signalling steps for SF operation for these architecture options are illustrated below

For Split-MME on board option

For control-plane CIoT solution where CIoT packets are delivered as NAS payload, followings are signaling steps for the store and forward operation.

For NAS signalling procedure

- The uplink NAS signalling messages are received at MME on-board and MME will send information towards the UE with wait timer and the list of satellite-ID on which the UE can complete the remaining procedure.

- The NAS component at UE need to start timer for resuming the procedure later at different satellite.

For CIOT packet transmission

- UE sends uplink data transmission as NAS payload to MME on-board and MME stores the data packets when it does not have feeder link connectivity

- When feeder link is available the MME function delivers the packets to MME at Ground station which further delivers the packets to SGW in Ground station.

For Full- CN on board option

For signalling procedure

- RRC and NAS signalling are not impacted as the complete MME functionality is available in the satellite.

- Each node on board operates with separate MME-ID or PLMN ID. NAS signalling further provides details on other satellites where the UE can continue its operation.

For CIoT data Transmission (CIoT CP)

  - The CIoT-Control Plane packets received via NAS signalling is stored at SGW on board when feeder link is not available.

  - The SGW on board transfers the packets to SGW on ground station on feeder-link connectivity.

Observation 2: For CIoT-CP operation the RAN2 signalling procedure for CIOT data transmission is not impacted for both MME-Split and Full-CN Architecture options.

The store and forward deployments of PLMN is expected to operate in same architecture option across its cells. Hence all the UE subscribed for this PLMN needs to support the modified NAS signalling procedure that includes delayed procedure and reception of information on selected satellite-ID for the NAS procedure.

Observation 3: RAN2 assumes Store-forward PLMN operates with same architecture option across all NTN cells.

Proposal 5: RAN2 signalling is not needed to indicate the architecture option (Split MME or Full CN).

For the Split MME architecture, the UE need to wait for one of the listed satellites to complete the pending NAS procedure. However it is possible that the UE may find different satellite on wait timer expiry. In such case whether the UE should terminate the current procedure or attempt for recovery procedure via the new satellite needs to be addressed. Hence, we propose that RAN2 to check with SA2 on how to address the scenario where UE does not find the listed satellites on wait timer expiry to complete the procedure.

Proposal 6: RAN2 checks with SA2 to address the unknown UE mobility scenario in which the UE does not find the target SAT for completing the remaining network procedure after the waiting time but another SAT instead.

## 2.3 CIoT-UP Support for SF Operation

The signalling procedure considered in SA2 study only focus on the procedures for CIoT-CP solution where the CIoT packets are delivered as NAS PDU using NAS security keys.  However, there will be additional impacts on RAN2 signalling procedure and security aspects for CIoT-UP solution mainly for the MME Split Option.  We propose that RAN2 discuss on whether the SF mode operation needs to be supported for CIoT-UP operation and if yes RAN2 to identify the signalling procedure changes and user-plane operation impacts for this procedure. In such case LS to SA2 can be considered that RAN2 intend to further investigate CIoT-UP solution that may need further changes in SA2 procedures.

Observation 4: CIoT-UP solution support will require additional RAN impacts for the user plane data forwarding for MME split option.

Observation 5: SA2 Technical report only focus on CIoT-CP solution as the basis for the store-forward study.

Proposal 7:  RAN2 to discuss and conclude on the need for support of CIOT-UP solution for SF operation.

## 2.4 Access Control and Paging Changes for SF operation

When NTN node is serving new area, it may need to handle the new data transmissions from the coverage area and also complete the data transmission in downlink that was received and stored from CN at the time of earlier connectivity. In such cases the access control and data transmission across these two variants of users needs to be differentiated. Mainly to avoid the delivering the stored data becomes invalid or time-out at UE.  In the same way for the uplink transmissions depending on the storage available at NTN node to receive and buffer the packets the access control and scheduling of transmissions needs to be handled. This can be achieved by explicit indication of support for MO or MT traffic along with SF operation parameter in system information.

Proposal 8:  RAN2 to consider inclusion of additional information for access control for MO or MT in SF operation

When the mobile originated IoT data transmission that requires application level ack are sent to eNB operating in store and forward mode the UE may expect the corresponding downlink packets after connecting to the next eNB. In this case UE monitors the next gNB for receiving ACK and new MT packets if any stored in the eNB. The data packet available at this eNB may contain ACK for earlier MO traffic and MT traffic toward the UE. In some other cases, the next eNB may not have any downlink ACK for earlier transmission depending on its CN connection time. In different scenarios based on the network awareness of CN connectivity time and the actual stored downlink data to be delivered towards UE, it is possible to improve the paging monitoring towards UE.

Proposal 9:  RAN2 to consider paging enhancements in SF mode for delivering ACK for MO Traffic and MT Traffic towards IoT-NTN UE.

In addition to paging enhancements, it is also beneficial for the UE battery life to consider that a satellite may not have more MT data for the UE. The typical case is that if the feeder link is not available, the satellite has no more MT data for the UE if the satellite buffer is drained since the satellite will not receive further MT data from CN. Until the feeder link recovery, the UE will not benefit from continuing paging monitoring on the current cell, nor will it benefit from reselecting to and monitoring paging from other cells of the same satellite. Thus, RAN2 can consider whether Idle mode procedures can be enhanced when there is no more MT data to be delivered in S&F operation.

Proposal 10: RAN2 to consider idle mode procedure enhancements, when there is no MT data to be delivered by a store & forward satellite.

# 3	Conclusion

In this paper we analyse the radio signalling procedure impacts for store and forward operation. Following proposals are

made based on the analysis.

Remaining issues related to SF Operation Indication and Transition

Observation 1: Use of system Information modification procedure for switching between SF operation and normal operation is not signaling efficient considering the short duration of normal operation window for SF mode cell.

Proposal 1: New parameter T-Service-SF is introduced to support the transition of SF operation from SF mode to Normal and vice-versa.

Proposal 2: On expiry of the T-Service-SF the UE reads the new value of T-Service-SF and also the new SF operation related parameters to start the validity timer for the new mode.

Proposal 3: FFS need for system information value tag in addition to T-Service-SF parameter.

Proposal 4: Additional information related to maximum delay impact of SF operation is not broadcasted in system information. UE can deduce this information from other parameters related SF operation provided in NAS level procedures.

RAN2 Impacts for CN Architecture Options

Observation 2: For CIoT-CP operation the RAN2 signalling procedure for CIOT data transmission is not impacted for both MME-Split and Full-CN Architecture options.

Observation 3: RAN2 assumes Store-forward PLMN operates with same architecture option across all NTN cells.

Proposal 5: RAN2 signalling is not needed to indicate the architecture option (Split MME or Full CN).

Proposal 6: RAN2 checks with SA2 to address the unknown UE mobility scenario in which the UE does not find the target SAT for completing the remaining network procedure after the waiting time but another SAT instead.

CIoT User Plane for SF Operation

Observation 4: CIoT-UP solution support will require additional RAN impacts for the user plane data forwarding for MME split option.

Observation 5: SA2 Technical report only focus on CIoT-CP solution as the basis for the store-forward study.

Proposal 7:  RAN2 to discuss and conclude on the need for support of CIOT-UP solution for SF operation.

Access control and Paging procedure for SF Operation

Proposal 8:  RAN2 to consider inclusion of additional information for access control for MO or MT in SF operation in

Proposal 9:  RAN2 to consider paging enhancements in SF mode for delivering ACK for MO Traffic and MT Traffic towards IoT-NTN UE.

Proposal 10: RAN2 to consider idle mode procedure enhancements, when there is no MT data to be delivered by a store & forward satellite.

# Reference

[1]	RP-240776 WID for IoT-NTN Enhancements