3GPP TSG-RAN WG2 Meeting #128s	R2-2410470
Orlando, US, November 18th – 22nd, 2024

Agenda Item:	8.5.3

Source:	InterDigital

Title:	On-demand SIB1 request and reception

Document for:	Decision

# Introduction

RAN approved a new work item on network energy saving (NES) [1], which was revised as [2]. The revised WID includes the following objective where study item objective was converted to the normative phase, as in the following:

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

RAN2 made the following agreements in RAN2 #127Bis meeting [3]

Agreements on OD-SIB1

- We will inherit all agreements made during SI phase to WI phase.

- A NES cell can include neighbouring NES cell’s WUS configuration.

- NES cell’s WUS configuration, it is included in a new SIB (including its own WUS configuration).

- In on-demand SIB1 procedure, the UE considers RACH failure when PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1.

- The MAC layer will indicate the RACH failure for SI request to upper layers. FFS: after that the UE upper layer will consider the cell as barred.

- The legacy UE behaviour can be reused upon on-demand SIB1 acquisition failure, i.e., the NES UE should follow the intraFreqReselection in MIB of NES cell.

- A cell for which SIB1 request configuration is available, can periodically broadcast SIB1.

- If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

- Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication in MIB e.g. via ssb-SubcarrierOffset. Detailed solution is up to RAN1. If this works, no separate barring bit for R19 NES UEs is introduced.

- NES UEs should be allowed to reselect to cells that are prevented from legacy UEs (e.g. by excluded cell list, reselection priorities).

- RAN2 will not start the discussion on the issue when the UL WUS configuration update in Cell A is not synchronised with the UL WUS configuration update in the NES cell, unless we’re asked to do that by other WG, e.g. RAN3.

This contribution discusses various aspects to support on-demand SIB1 for UEs in idle/inactive mode.

# UL-WUS configuration for On-demand SIB1

According to the WID for enhanced NES, the triggering for on-demand SIB1 will use UL WUS signal based upon existing signal/channel. One important step to enable on-demand SIB1 transmissions is the provisioning of UL WUS configuration for the UEs which are currently in idle/inactive mode.

When Cell A is providing the UL-WUS configuration for one or more NES cells, e.g., to support the scenario 1a, the Cell A may transmit UL-WUS configuration as part of the system information.

RAN2 #127Bis [3] agreed to provide UL-WUS configuration through a new SIB.

UL-WUS configuration will allow the UE to identify the NES cell, as agreed in RAN2#126 [5] through inclusion of PCI and frequency as part of the UL-WUS configuration. Based upon the received UL-WUS configuration, the UE can transmit the UL WUS to the NES cell. Thus, at minimum, the UL-WUS configuration provides the RACH configuration for the NES cell. The RACH configuration should provide the suitable RO and preamble determination for the UE based upon the SSBs being transmitted by the NES cell. This was agreed in RAN2#125Bis [5].

In order to transmit Msg1, the UE needs to know the mapping of SSB to RO. In legacy, SSB to RO mapping is part of the RACH configuration. Thus, following the legacy behavior, the RACH configuration of the NES cell, provided through UL-WUS configuration, should include the SSB to RO mapping to request on-demand SIB1.

The reception of RAR in response to the UL-WUS transmission will require that UE knows a CORESET, e.g., CORESET 0, where it can receive and decode the response. The CORESET configuration for RAR can be the CORESET 0 or a different configuration, which can be discussed in RAN2.

RAN1 #118bis made the following agreement: “Search space for RAR in response to UL WUS on a NES Cell can be provided by the UL WUS configuration. If not, follow the search space zero for the NES Cell.” If the network intends to transmit RAR through CORESET 0 in response to a UE’s UL-WUS transmission, the UE may acquire the CORESET 0 configuration from the SSB (e.g., same as legacy UEs get CORESET 0 configuration through SSB/MIB). Nevertheless, the network may have additional flexibility by providing a CORESET for RAR different from CORESET 0.

Proposal 1: 	RAN2 to discuss whether the UE acquires CORESET 0 configuration (i) through SSB of the NES cell, or (ii) as part of UL-WUS configuration for the NES cell.

One aspect related to the UL-WUS RACH configuration is whether the RACH configuration is as per legacy or if there is a separate RACH configuration to trigger SIB1. Although RACH configuration triggering SIB1 transmissions shared with legacy RACH functionality can work, a separate RACH configuration as trigger can be more advantageous, separate from legacy UEs, and can be used for all UEs in the cell. It is not likely to need too many preambles for requesting OD-SIB1, as in legacy request for other SI. Legacy UEs are also not likely to be camped on such SIB1-less cell.

Proposal 2:	Support a dedicated RACH configuration for requesting OD-SIB1.

# RA procedure for WUS transmission

RAN2 has agreed that Msg3 is not used for OD-SIB1 request. In a legacy RA procedure initiated for SI request, the procedure also does not require Msg3 transmission and is considered successful upon reception of a RAR with the selected RAPID. A similar condition for success can be used for determining the success of an RA initiated for OD-SIB1 request.

Proposal 3:	Similar to legacy RA procedure initiated for SI request, an RA initiated for OD-SIB1 request is initiated with 4 step RA and is considered successful upon reception of RAR.

In a legacy RA procedure initiated for SI request, the UE can be configured with ra-AssociationPeriodIndex and si-RequestPeriod. si-RequestPeriod specifies the periodicity of the SI-Request configuration in number of association periods, i.e. the periodicity at which a UE can request SI. RAN2 can thus discuss whether such parameters need to be provided in the UL WUS configuration for OD-SIB1 request, when multiple SSBs are present.

Proposal 4:	RAN2 to discuss whether to configure si-RequestPeriod in the UL WUS configuration for OD-SIB1.

# Acquisition for On-demand SIB1

RAN2 has agreed that a UE will receive an RAR in response to its UL-WUS transmission over an OD-SIB1 NES cell. In legacy RA procedure initiated for acquisition of SI request, the RAR format is simplified to contain the RAPID only. RAN2 also agreed that a combined RA procedure for OD-SIB1 request and initial access in NES cell is not supported. It is therefore assumed that there isn’t a need for msg3 transmission post successful reception of the RAR. Therefore, many of the RAR parameters, e.g., the UL grant (with relevant parameters) and timing advance related information may not be needed as here UEs may not need to perform a Msg-3 transmissions.

The network can provide useful information in RAR related to the scheduling of OD-SIB1 transmission. E.g., the network can provide a time, or a window when the UE may expect the subsequent OD-SIB1 transmission.

Proposal 5:	The scheduling of OD-SIB1 transmission is indicated part of the RAR received after WUS transmission.

# SIB1 Validity

One signaling aspect for UL WUS transmission is related to the target cell recipient of the UL WUS signal. If the UL WUS configuration is available, the NES cell should be the target receiver for the UL WUS signal. If the UL WUS is transmitted directly to the NES cell, the NES cell may transmit SIB1 after detecting the PRACH signal. For the situations when the anchor cell is not good enough for the UEs, and these UEs are located much closer to the NES cell than an anchor cell, having the possibility of transmitting the UL WUS to the NES cell can be more beneficial.

In RAN2 #117, it was discussed to have an OD-SIB1 configuration validity criterion, such that if a UE has a valid version of SIB1 stored, UE does not initiate the on-demand SIB1 request. When the UE camps on a NES cell, the network may not know when the UE acquired SIB1. For the cells not transmitting SIB1, it may be important to maintain a valid version of SIB1 at the UEs. Following the validity functionality for other SIBs, time-based validity for SIB1 can be introduced.

Proposal 6:	Time based validity can be introduced for OD-SIB1 maintenance.

In a scenario where the UE has a valid UL WUS configuration and is camped on the NES cell, if the UE needs to acquire SIB1 from the NES cell, e.g., after a certain validity period, the UE may transmit UL WUS to the NES cell.

Proposal 7:	Upon expiry of a validity time associated with OD-SIB1, if the UE is camped on the NES cell and a valid UL WUS configuration is available, the UE transmits UL WUS towards the NES cell once triggered.

# UL-WUS configuration validity

One design aspect with OD-SIB1 through the transmission of UL-WUS is to maintain the validity of the UL-WUS configurations. As UL-WUS configuration will be provided to the UE through SIB (e.g., new or existing SIBs), it would make sense to reuse the legacy validity behavior. Following the validity functionality for other SIBs, time-based validity for UL-WUS can be introduced.

Proposal 8:	Time based validity is introduced for UL WUS configuration maintenance.

When a UE performs cell (re-)selection to a NES cell, it may transmit UL-WUS based only upon a valid UL-WUS configuration.

In one scenario, UL WUS configuration may expire with time. In another scenario, the network may change/update the UL WUS configuration for the NES cell that the UE is camped on. To acquire the NES cell configuration from a different anchor cell may be challenging and even not feasible in some situations and deployments. RAN2 #127Bis [3] agreed that the new SIB includes the cell’s own WUS configuration. Thus, the UE should be able to acquire the UL-WUS configuration from the NES cell directly when it is already camped on the NES cell. This can be achieved by the legacy SI modification procedure. If the network modifies the UL-WUS configuration for the NES cell, the NES cell can indicate this modification through system information update message.

Proposal 9:	The network can indicate the modification of UL-WUS configuration for a NES cell through the NES cell itself by SI modification indication.

In a scenario where a UE does not have a valid UL-WUS configuration and is camped on the NES cell, the UE may attempt to re-acquire a valid UL-WUS configuration for the NES cell. If the UE has neither valid SIB1 nor valid UL-WUS configuration for the NES cell, the UE will perform cell reselection and considers the NES cell as barred.

Proposal 10:	If a UE cannot acquire a valid UL-WUS configuration for its camped NES cell and the UE doesn’t have/cannot acquire a valid SIB1, the UE considers the NES cell as barred and performs cell reselection.

# UE detecting an OD-SIB1 cell without UL-WUS configuration

RAN2 #127bis had some discussion on how to handle the case if a UE detects an OD-SIB1 cell but does not have a (valid) UL-WUS configuration to request OD-SIB1. This may for example be the case when the UE is performing cell re-selection from another RAT, e.g., 4G LTE.

Without going into the details of other RAT and the deployment scenario, in our understanding if a R-19 UE is able to determine that a detected cell is a NES cell, but the UE does not have any a valid WUS configuration for this cell, if the NEC cell is the best one and the UE detects an OD-SIB1, the UE should attempt to acquire the WUS configuration from another cell A. If the UE cannot acquire any WUS configuration for the NES cell, only then the UE should bar the NES cell as the UE cannot acquire the WUS config.

Proposal 11:	If a R19 UE detects a NES cell part of cell (re)-selection but does not have the corresponding UL-WUS configuration to request OD-SIB1, the UE attempts to acquire a WUS config from another cell after cell (re)-selection. The UE considers the cell as barred if it does not acquire a WUS config for the NES cell.

Another scenario discussed is when the R19 capable NES UE bars the NES cell post OD-SIB1 acquisition failure, but later acquires a valid WUS acquisition for it from another cell A. In such case, the UE can consider the cell as not barred if it acquires a valid WUS configuration for it.

Proposal 12:	If a R19 NES UE has barred the NES cell post OD-SIB1 acquisition failure, but later acquires a valid WUS acquisition for the NES cell, the UE can consider the cell as not barred.

# Conclusions

The following proposals are made in this document for on demand SIB1 acquisition in IDLE/Inactive states:

Proposal 1: 	RAN2 to discuss whether the UE acquires CORESET 0 configuration (i) through SSB of the NES cell, or (ii) as part of UL-WUS configuration for the NES cell.

Proposal 2:	Support a dedicated RACH configuration for requesting OD-SIB1.

Proposal 3:	Similar to legacy RA procedure initiated for SI request, an RA initiated for OD-SIB1 request is initiated with 4 step RA and is considered successful upon reception of RAR.

Proposal 4:	RAN2 to discuss whether to configure si-RequestPeriod in the UL WUS configuration for OD-SIB1.

Proposal 5:	The scheduling of OD-SIB1 transmission is indicated part of the RAR received after WUS transmission.

Proposal 6:	Time based validity can be introduced for OD-SIB1 maintenance.

Proposal 7:	Upon expiry of a validity time associated with OD-SIB1, if the UE is camped on the NES cell and a valid UL WUS configuration is available, the UE transmits UL WUS towards the NES cell once triggered.

Proposal 8:	Time based validity is introduced for UL WUS configuration maintenance.

Proposal 9:	The network can indicate the modification of UL-WUS configuration for a NES cell through the NES cell itself by SI modification indication.

Proposal 10:	If a UE cannot acquire a valid UL-WUS configuration for its camped NES cell and the UE doesn’t have/cannot acquire a valid SIB1, the UE considers the NES cell as barred and performs cell reselection.

Proposal 11:	If a R19 UE detects a NES cell part of cell (re)-selection but does not have the corresponding UL-WUS configuration to request OD-SIB1, the UE attempts to acquire a WUS config from another cell after cell (re)-selection. The UE considers the cell as barred if it does not acquire a WUS config for the NES cell.

Proposal 12:	If a R19 NES UE has barred the NES cell post OD-SIB1 acquisition failure, but later acquires a valid WUS acquisition for the NES cell, the UE can consider the cell as not barred.

# References

RP-240170, Revised WID: Enhancements of network energy savings for NR, Mar 2024

RP-242353, Revised WID: Enhancements of network energy savings for NR, September 2024

RAN2 Chairman’s note, RAN2 #127Bis, October 2024.

RAN2 Chairman’s note, RAN2 #127, August 2024.

RAN2 Chairman’s note, RAN2 #126, May 2024.

3GPP TR 38.864, Study on network energy savings for NR (Release 18), V18.1.0, Mar 2023