3GPP TSG-RAN WG2 Meeting #128	R2-2410475

Orlando, FL, USA, November 18 – 22, 2024

Agenda item:	8.1.2.3

Source: 	Qualcomm Incorporated

Title: 	LCM for positioning use case

Document for:		Discussion and Decision

# 1.	Introduction

At RAN2#126, the following agreements were made [3]:

The LPP Capability Transfer procedures (RequestCapabilities/ProvideCapabilities messages) are used to indicate supported AI/ML positioning capabilities. FFS how to handle dynamic capabilities, depending on further RAN1 progress and understanding of the functionality. Wait for RAN1 for associate ID discussion. At least for Case 1, existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ ProvideLocationInformation messages) are used for providing the results of the UE sided model inference operation. FFS further details on signaling enhancements

At RAN2#127, the following functionality definitions were agreed for the Beam Management use case [4]:

Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling) Applicable functionalities refer to functionalities that the UE is ready to apply for inference Activated functionalities refer to functionalities already enabled for performing inference

A general procedure for AI/ML positioning was agreed at RAN2#127bis [5]:

1: The following procedures for LCM for UE sided model for AI positioning case 1 is the baseline: Step 1: LMF may request the UE to report the supported functionalities at the UE side by LPP request capabilities message. Step 2: UE sends LPP provide capabilities message to LMF with the supported functionalities at the UE side. Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition). Step 4: UE reports the applicable functionality to the LMF by the LPP provide capabilities message. Step 5: The LMF requests the inferred location information using the LPP request location information message. Step 6: UE reports the inferred location using LPP provide location information message. 2: Whether the inference configuration is provided in step 3 or/and step 5 is FFS (to be revised based on RAN1 progress). 3: Whether network side additional condition is needed and what it contains is FFS (to be revised based on RAN1 progress). 4: FFS whether LMF controls the UE sending unsolicited LPP provide capabilities (i.e. whether step 4 is sent reactively or proactively). FFS the signalling details. 5: RAN2 will decide whether AI positioning will be a new method after further details from RAN1 are received.

In this contribution, we further discuss some details of the above baseline procedure and provide our view on the AI/ML functionality definition for the positioning use case.

# 2.	AI/ML functionality definition for the positioning use case

AI/ML positioning functionality generally refers to the use of artificial intelligence (AI) and machine learning (ML) techniques to enhance the accuracy and reliability of the positioning system [11].

For UE positioning, there are 4 positioning modes currently defined [6], [7]:

-	UE-assisted mode: the UE obtains location measurements and sends the measurements to another entity (e.g. an LMF) to compute a location;

-	UE-based mode: the UE obtains location measurements and computes a location estimate making use of assistance data provided by serving PLMN;

-	standalone mode: the UE obtains location measurements and computes a location estimate without making use of assistance data provided by serving PLMN;

-	network based mode: a serving PLMN obtains location measurements of signals transmitted by a target UE and computes a location estimate.

The AI/ML positioning cases considered in this Work Item are summarized in Figure 1 [1]:

-	Case 1: UE-based positioning with UE-side model, direct AI/ML positioning;

-	Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning;

-	Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning;

-	Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning;

-	Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning.

The AI/ML positioning cases above adopt the UE-based, UE-assisted, and network-based (NG-RAN node assisted) modes but with the extension of AI/ML assisted and/or direct AI/ML positioning as illustrated in Figure 2:

-	direct AI/ML positioning: The AI/ML model directly outputs the determined location of the UE;

- 	AI/ML assisted positioning: Rather than directly determining the UE's location, the AI/ML model aids conventional approaches by producing or enhancing measurements. For instance, the AI/ML model may identify LOS vs. NLOS conditions, provides timing and/or angle measurements, etc.

Figure 1: AI/ML positioning cases considered in the Work Item [1].

Figure 2: Direct and Assisted AI/ML positioning.

The supported positioning modes for each 3GPP positioning method are summarized in Table 4.3.1-1 of TS 38.305 [7], which is repeated in Table 1 below. The NR positioning methods are shown in grey highlight.

Table 1: Supported versions of UE positioning methods according to 38.305 [7].

| Method | UE-based | UE-assisted, LMF-based | NG-RAN node assisted | SUPL Note 8 |
| --- | --- | --- | --- | --- |
| A-GNSS | Yes | Yes | No | Yes |
| OTDOA Note1, Note 2 | No | Yes | No | Yes |
| E-CID Note 4, Note 7 | No | Yes | Yes | Yes for E-UTRA |
| Sensor | Yes | Yes | No | No |
| WLAN | Yes | Yes | No | Yes |
| BluetoothNote 9 | Yes | Yes | No | No |
| TBS Note 5 | Yes | Yes | No | Yes (MBS) |
| DL-TDOA | Yes | Yes | No | Yes |
| DL-AoD | Yes | Yes | No | Yes |
| Multi-RTT | No | Yes | Yes | Yes |
| NR E-CID | No | Yes | Yes | Yes (DL NR E-CID) |
| UL-TDOA | No | No | Yes | Yes |
| UL-AoA | No | No | Yes | Yes |
| NOTE 1: This includes TBS positioning based on PRS signals. NOTE 2: In this version of the specification only OTDOA based on LTE signals is supported. NOTE 3: Void NOTE 4: This includes Cell-ID for NR method when UE is served by gNB. NOTE 5: In this version of the specification only for TBS positioning based on MBS signals. NOTE 6: Void NOTE 7: Enhanced Cell ID based on LTE signals. NOTE 8: This shows whether the positioning method is supported by SUPL ULP. NOTE 9: UE-based positioning mode is supported only for Bluetooth AoD positioning method. |  |  |  |  |

Each of the defined NR positioning methods could be enhanced with AI/ML techniques to improve the accuracy and reliability of the positioning method. This "AI/ML positioning functionality" could be defined via the two additional modalities of "direct AI/ML" and "assisted AI/ML" positioning, as shown in Table 2 below.

Table 2: Supported AI/ML modality of NR UE positioning methods.

| Method | UE-based | UE-assisted, LMF-based |  | NG-RAN node assisted |  |
| --- | --- | --- | --- | --- | --- |
|  | Direct AI/ML Positioning | Direct AI/ML Positioning | AI/ML Assisted Positioning | Direct AI/ML Positioning | AI/ML Assisted Positioning |
| DL-TDOA | YesNOTE1 | YesNOTE2b | YesNOTE2a | No | No |
| DL-AoD | YesNOTE1 | YesNOTE2b | YesNOTE2a | No | No |
| Multi-RTT | No | YesNOTE3 | YesNOTE4 | YesNOTE3 | YesNOTE4 |
| NR E-CID | No | YesNOTE2b | No | YesNOTE5 | YesNOTE6 |
| UL-TDOA | No | No | No | YesNOTE5 | YesNOTE6 |
| UL-AoA | No | No | No | YesNOTE5 | YesNOTE6 |
| NOTE 1: Corresponds to 'Case 1' NOTE 2a: Corresponds to 'Case 2a' NOTE 2b: Corresponds to 'Case 2b' NOTE 3: Corresponds to 'Case 2b + Case 3b' NOTE 4: Corresponds to 'Case 2a + Case 3a' NOTE 5: Corresponds to 'Case 3b' NOTE 6: Corresponds to 'Case 3a' |  |  |  |  |  |

Observation 1:	(NR) positioning functionality is currently defined by a "positioning method" (e.g., DL-TDOA, DL-AoD, etc.) and a "positioning mode" (e.g., UE-based, UE-assisted, etc.).

Observation 2:	AI/ML positioning cases can be defined by a "positioning method" and a "positioning mode" (as in legacy), and an additional "AI/ML modality" defined as direct- and assisted-AI/ML positioning.

Proposal 1:	For the positioning use case, the "AI/ML functionality" is defined by a "positioning method" and a "positioning mode" (as in legacy), combined with a new "AI/ML modality" defined as 'direct AI/ML positioning' and 'AI/ML assisted positioning'.

Note, since for Case 2a (UE assisted) (and Case 3a (NW-based)) at least the "AI/ML enhanced" existing measurements can be reported, there seems no need (so far) to introduce a new positioning method dedicated to UE-based mode (Case 1):

Agreement [2] For AI/ML assisted positioning Case 2a, at least LOS/NLOS indicator and/or timing information are supported for reporting. - If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. - If timing information is reported, the timing information at least can be reported via DL RSTD or UE Rx-Tx time difference as defined in 38.215. - Note: details of the report are pending further discussion. Agreement [2] For AI/ML assisted positioning Case 3a, at least LOS/NLOS indicator and/or timing information are supported for reporting. - If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. - If timing information is reported, the timing information at least can be reported via UL RTOA or gNB Rx-Tx time difference as defined in 38.215. - Note: details of the report are pending further discussion.

# 3.	Functionality Identification

The "baseline procedure" (for UE-sided model) according to section 1 is illustrated in Figure 3 below.

Figure 3: Generic LPP positioning procedure.

## 3.1	Supported Functionality

At Step 1 in Figure 3, a "LMF may request the UE to report the supported functionalities at the UE side by LPP request capabilities message". The UE would respond with a LPP provide capabilities message "with the supported functionalities at the UE side".

For positioning, the UE "supported functionality" (capability) is defined within a LPP Provide Capabilities message. The LPP positioning capabilities contain the following main information [8]:

-	Positioning methods supported (e.g., DL-TDOA, DL-AoD, Multi-RTT, etc.).

-	Positioning modes per method supported (UE-based, UE-assisted, standalone).

-	Auxiliary positioning measurements and information per method supported (e.g., RSRP, multipath measurements, LOS/NLOS measurement, etc.).

-	Assistance data details per method supported (e.g., DL-PRS, position calculation assistance, etc.).

With Proposal 1, the "supported functionality" for the AI/ML positioning use case would be the "AI/ML modality" for each positioning method, which could be included in the LPP positioning capabilities for each applicable positioning method (e.g., per Table 2 above).

However, for Case 1 and Case 2a/2b (i.e., the LPP-based methods), there would be no need to explicitly distinguish between 'direct AI/ML positioning' and 'AI/ML assisted positioning', since this would be implicit in the UE-based and UE-assisted mode indication, respectively. Therefore, the LPP Provide Capability messages would only need to include an additional indication on whether the supported UE-based and UE-assisted positioning methods are also supported in "AI/ML Enhanced Mode".

Proposal 2:	The "supported AI/ML functionality" can be indicated in the LPP 'method'-ProvideCapabilities messages (with 'method' equal to 'DL-TDOA', 'DL-AoD', and 'Multi-RTT') with an "AI/ML Enhanced Mode" indication, which – together with already specified capabilities – would define the supported "AI/ML modality" per Proposal 1 for each supported positioning method.

RAN1 may define additional measurements or assistance data specifically for AI/ML enhanced positioning. These additional elements together with their corresponding capability can be included in the applicable LPP capability/assistance data/measurement transfer messages as usual.

## 3.2	Applicable Functionality

Whether a "supported functionality" is also "applicable" for an LPP session depends on whether the UE available/trained model is applicable under the current network conditions (see also section 3.4 below). As also discussed in [10], a change in UE positioning capabilities can happen at any time during an LPP session. A UE would normally provide its updated capabilities via unsolicited LPP Provide Capabilities message (e.g., Step 4 in Figure 3). It is currently FFS in the agreements from RAN2#127bis whether the LMF should control the UE sending updated capabilities (see section 1):

4: FFS whether LMF controls the UE sending unsolicited LPP provide capabilities (i.e. whether step 4 is sent reactively or proactively). FFS the signalling details.

Since a UE does generally not know the LMF capabilities, such a "control" is needed when a UE cannot infer the LMF capabilities from other messages (e.g., LPP Provide Assistance Data message). Currently, such an "LMF control" is specified for e.g., LPP segmentation capabilities and UE remote status indication (see e.g., LPP CommonIEsRequestCapabilities [8]). I.e., a UE should not send updated capabilities related to LPP segmentation or remote status if the LMF does not support the corresponding functionality. The same applies to AI/ML positioning functionality. It would be useless/overhead if a UE reports its "applicable AI/ML functionality" when a LMF is not AI/ML positioning capable.

However, for the actual "positioning capabilities" (in this case, the "supported/applicable AI/ML functionality"), the UE would always know the LMF support, either from Step 1 or Step 3 or Step 5 (at the latest) in Figure 3. I.e., the LPP Request Capabilities at Step 1 in Figure 3 indicates which positioning capabilities are requested (e.g., DL-TDOA, etc.) and therefore, the UE knows which positioning functionality is supported by an LMF and would provide corresponding updated capabilities (if needed) e.g., at Step 4. For example, a UE would not report its changed Bluetooth positioning capabilities (e.g., at Step 4 in Figure 3) if a UE has not received a request for Bluetooth Capabilities before (e.g., at Step 1 in Figure 3). If Step 1 did not happen (e.g., since the LMF received the positioning capabilities from the AMF [6]), a UE would know the LMF supported functionality from Step 3 or Step 5 in Figure 3. In that sense, the Step 4 in Figure 3 is always "reactive".

Proposal 3:	There is no need to explicitly control the UE sending unsolicited LPP provide capabilities (e.g., at Step 4 in Figure 3). This NW control is implicit in a LPP positioning session.

From the discussion above and current specifications, it seems also evident that there is no need to explicitly distinguish between "supported" and "applicable" functionality e.g., in a LPP Provide Capabilities message (Step 2 and Step 4 in Figure 3). The LMF would always interpret the LPP Provide Capabilities message as "supported" functionality. However, this support may change during an LPP session.

Proposal 4:	There is no need to explicitly distinguish between "supported functionality" and "applicable functionality" in a e.g., LPP Provide Capabilities message. The server always considers the UE reported capabilities as "applicable" (as currently defined in LPP). However, the "applicable functionality" (i.e., LPP capabilities) may change during an LPP session.

## 3.3	Activated Functionality

According to the definition in section 1 above, "activated functionality refers to functionalities already activated and performing inference". For positioning, there seems no use case/need for such a definition. According to Figure 3, there is no difference in UE/server behaviour when a LPP Provide Capabilities message provides "supported"/"applicable" or "activated" functionality. A UE may have multiple LPP location sessions ongoing (i.e., already performing inference for another location request) and a LMF may also be aware of multiple location requests for a UE. The handling of concurrent location requests is up to UE and LMF implementation [6].

For example, if a UE has functionalities already activated and performing inference when a LPP Request Capabilities message at Step 1 in Figure 3 arrives, the UE will provide its 'supported' = 'applicable' = 'activated' functionality at Step 2. The UE and LMF would not behave differently in this case. A UE may receive a location request from an LMF after the LMF receives a location request from an AMF [6], [7]. If there is already an ongoing location session with the same UE ("activated functionality"), and this location request fulfils the requirements for the new location request, the LMF normally does not send a second concurrent location request but uses the UE results of the first (ongoing = "activated") request to fulfil both location requests from the AMF. Therefore, there seems no need/use case for an "activated functionality" reporting. Functionality is/will be activated when a UE receives a LPP Request Location Information message. A LMF will be aware of "activated functionality" once it sent a LPP Request Location Information to a UE.

Proposal 5:	There is no need to report "activated functionality" to an LMF e.g., in a LPP Provide Capabilities message.

## 3.4	Network-side (additional) Conditions

Whether a "supported functionality" is also "applicable" for an LPP session depends on whether the UE available/trained model is applicable under the current network conditions.  Although, the details of these network conditions may depend on RAN1 progress, it appears obvious that these network conditions comprise (at least) all aspects affecting position measurements and position calculation. These aspects have already been standardized as assistance data which include (for the NR positioning methods) [7], [8]:

-	DL-PRS configuration information (IE NR-DL-PRS-AssistanceData)

-	Available on-demand DL-PRS configurations (IE NR-On-Demand-DL-PRS-Configurations)

-	Position calculation assistance comprising:

-	TRP/ARP location information (IE NR-TRP-LocationInfo)

-	Boresight direction of DL-PRS Resources (IE NR-DL-PRS-BeamInfo)

-	Synchronization information between TRPs (IE NR-RTD-Info)

-	DL-PRS beam-power information (IE NR-TRP-BeamAntennaInfo)

-	Expected LOS/NLOS at a-priori UE location (IE NR-DL-PRS-ExpectedLOS-NLOS-Assistance)

-	Timing Error Groups for the DL-PRS Resources (IE NR-DL-PRS-TRP-TEG-Info)

-	Integrity information

-	PRU measurement and location information (IE NR-PRU-DL-Info)

The above assistance data can also be available from System Information (posSIB).

Support of the above assistance data depends on the UE supported positioning method and mode and are part of the UE capabilities [8].

Observation 3:	The "Network Conditions" for positioning comprise (at least) the already standardized LPP Assistance Data required for positioning measurements and/or position calculation. UE supported Assistance Data are already part of the UE capabilities.

RAN1 may also define "network additional conditions" for the positioning use cases (if needed in addition to the already specified "network conditions" summarized above (Observation 3)). It has been proposed that any "Network-side Additional Conditions" are identified via an "associated ID". However, this appears unnecessary for the positioning use case and rather sub-optimal, because of the following reasons.

(1)	Assistance Data are always needed for (NR) positioning. At minimum, the UE needs to know the available DL-PRS configuration to perform DL-PRS measurements. For position calculation, additional assistance data like TRP/ARP locations, DL-PRS beam information, synchronization information, etc. are required, dependent on the positioning method. Therefore, the NW provides these assistance data to a target anyhow, either via dedicated LPP signalling or via System Information (posSIB). Thus, an "implicit" identification via an "associated ID" seems not needed. Additional assistance data ("network-side additional conditions") can be easily added to the existing framework, if needed (as it has been done in the past from Release to Release).

(2)	Any "associated ID" for the "Network-side Additional Conditions" would imply a "hard-decision" and thus, suboptimal. For example, if the synchronization information in the network changes, the "associated ID" would have to change to indicate a change in "Network-side Additional Conditions". Such a change may imply that there is a discrepancy between model training and inference. However, this may not necessarily be the case. E.g., some implementations/methods may be rather robust for changes in NW synchronization, or may tolerate a certain level of change, etc. A simple "associated ID" change indication does not provide any information to what degree the information has changed, and therefore, corresponds to a "hard decision" not to use a trained model for inference in the given scenario. Providing the (normal) assistance data as "Network-side Additional Conditions" leaves the decision on whether a trained model is still suitable for inference in a given situation up to implementation. E.g., a UE would not only recognize any condition change between training and inference, but also which parameter has changed and to what degree it has changed and could decide (based on model implementation) whether such a change has any impact on inference in the current scenario.

In addition, assuming the "network side conditions" are the already defined positioning assistance data (per Observation 3), any "network side additional conditions" can be easily added to the existing assistance data framework. Although, we note from the agreement from RAN2#126:

"Wait for RAN1 for associate ID discussion"

we propose based on the discussion above:

Proposal 6:	RAN2 assumes that any "Network-side Additional Conditions" for the positioning use case are the LPP Assistance Data elements, possibly with some enhancements, if needed. An "Associated ID" to identify "Network-side Additional Conditions" is not needed.

# 4.	Functionality Management/Control

For functionality/model selection, activation, deactivation, switching, and fallback operation, which may be generally referred to as "functionality management" or "functionality control", TR 38.843 [11] defines in section 7.2.4 the following:

- Management: o For UE-side model, the model/functionality control (e.g., selection, (de)activation, switching, fallback, etc.) may be performed by the UE when the monitoring resides within the UE. o For gNB-side model, the model/functionality control (e.g., selection, (de)activation, switching, fallback, etc.) is performed by the gNB. o The model/functionality control (e.g., selection, (de)activation, switching, fallback, etc.) may be performed by the LMF when the monitoring resides within the LMF or UE.

For UE-side models (Case 1, 2a), the functionality control can be inside the UE (when the monitoring resides within the UE – see also section 5 below). When the UE receives the assistance data (Step 3 in Figure 3) together with a Location Request (Step 5 in Figure 3), the UE selects an applicable model and performs the model inference based on the obtained input data (e.g., measurements).

Proposal 7:	AI/ML positioning functionality is activated in a UE when an LPP Request Location Information message has been received and may be deactivated in the UE once the LPP Provide Location Information message has been returned to the server.

With Proposal 1 and Proposal 2 (and corresponding Table 2), the UE would receive a location request for one of the currently defined positioning methods (LPP 'method'-RequestLocationInformation messages (with 'method' equal to 'DL-TDOA', 'DL-AoD', and 'Multi-RTT')) including the "AI/ML Enhanced Mode" indication requesting inferred location information (at Step 5 in Figure 3). The 'method'-RequestLocationInformation message could include an indication whether fallback to non-AI/ML based location information is allowed or not. After Step 5 in Figure 3 a UE would have all information available to perform the functionality management operation of activating/deactivating a particular model (applicable to the current conditions) or fallback to non-AI/ML based measurements.

Note, a fallback to non-AI/ML based measurements would then not mean a change of selected positioning method. E.g., if requested AI/ML enhanced measurements/location cannot be obtained, a UE would report the 'normal' (non AI/ML enhanced) measurements/location (instead of reporting an error cause).

Observation 4:	With Proposals 1 and 2 (Table 2), a fallback to non-AI/ML based measurements would not imply a change of selected positioning method. If AI/ML enhanced measurements/location estimate cannot be obtained in a given environment/situation, a UE could report the 'normal' measurements/location estimate instead and positioning with the LMF selected positioning method would still be possible.

Proposal 8:	The LPP 'method'-RequestLocationInformation messages (with 'method' equal to 'DL-TDOA', 'DL-AoD', and 'Multi-RTT') can include an indication whether fallback to non-AI/ML enhanced location information is allowed/requested or not.

Proposal 9:	For UE-sided models (Case 1/2a) the UE decides whether fallback to non-AI/ML positioning is required, e.g., based on model availability or model performance.

# 5.	Functionality/Model Monitoring

According to TR 38.843 [11], Model Monitoring is defined as follows:

Model monitoring: A procedure that monitors the inference performance of the AI/ML model.

Therefore, "model monitoring" is closely related to "model inference".

RAN1 is studying two options for model performance monitoring for UE-side models [4]:

Agreement For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded. Conclusion For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, - Option A-4 can be realized by implementation in a manner transparent to specification specification if the PRU sends information to the target UE side in a proprietary method. No further discussion on Option A-4.

The input data for monitoring are essentially the same as for model training and inference together with the "ground-truth labels" (which are also required for model training). Therefore, the entity which performs data collection and deploys the model/runs the inference seems in the best position to also monitor the performance of the trained models. Model monitoring feedback would be needed at the model training entity (e.g., to manage any model (re-)training) and at the inference entity (e.g., to decide on model applicability/fallback (see also section 4 above)). Therefore, for UE-sided models, this "monitoring entity" should be the UE and/or "server for data collection for UE-side model training".

Proposal 10:	For UE-sided models (Case 1/2a) the model performance monitoring (and associated model performance monitoring metric calculation in label-based model monitoring) is performed at the UE and/or "server for data collection for UE-side model training".

# 6.	Summary

In this contribution, we discussed some LCM aspects for AI/ML positioning. The following observations and proposals were made.

Functionality Definition

Observation 1:	(NR) positioning functionality is currently defined by a "positioning method" (e.g., DL-TDOA, DL-AoD, etc.) and a "positioning mode" (e.g., UE-based, UE-assisted, etc.).

Observation 2:	AI/ML positioning cases can be defined by a "positioning method" and a "positioning mode" (as in legacy), and an additional "AI/ML modality" defined as direct- and assisted-AI/ML positioning.

Proposal 1:	For the positioning use case, the "AI/ML functionality" is defined by a "positioning method" and a "positioning mode" (as in legacy), combined with a new "AI/ML modality" defined as 'direct AI/ML positioning' and 'AI/ML assisted positioning'.

Supported Functionality

Proposal 2:	The "supported AI/ML functionality" can be indicated in the LPP 'method'-ProvideCapabilities messages (with 'method' equal to 'DL-TDOA', 'DL-AoD', and 'Multi-RTT') with an "AI/ML Enhanced Mode" indication, which – together with already specified capabilities – would define the supported "AI/ML modality" per Proposal 1 for each supported positioning method.

Applicable Functionality

Proposal 3:	There is no need to explicitly control the UE sending unsolicited LPP provide capabilities (e.g., at Step 4 in Figure 3). This NW control is implicit in a LPP positioning session.

Proposal 4:	There is no need to explicitly distinguish between "supported functionality" and "applicable functionality" in a e.g., LPP Provide Capabilities message. The server always considers the UE reported capabilities as "applicable" (as currently defined in LPP). However, the "applicable functionality" (i.e., LPP capabilities) may change during an LPP session.

Activated Functionality

Proposal 5:	There is no need to report "activated functionality" to an LMF e.g., in a LPP Provide Capabilities message.

Network-side (additional) Conditions

Observation 3:	The "Network Conditions" for positioning comprise (at least) the already standardized LPP Assistance Data required for positioning measurements and/or position calculation. UE supported Assistance Data are already part of the UE capabilities.

Proposal 6:	RAN2 assumes that any "Network-side Additional Conditions" for the positioning use case are the LPP Assistance Data elements, possibly with some enhancements, if needed. An "Associated ID" to identify "Network-side Additional Conditions" is not needed.

Functionality Management/Control

Proposal 7:	AI/ML positioning functionality is activated in a UE when an LPP Request Location Information message has been received and may be deactivated in the UE once the LPP Provide Location Information message has been returned to the server.

Observation 4:	With Proposals 1 and 2 (Table 2), a fallback to non-AI/ML based measurements would not imply a change of selected positioning method. If AI/ML enhanced measurements/location estimate cannot be obtained in a given environment/situation, a UE could report the 'normal' measurements/location estimate instead and positioning with the LMF selected positioning method would still be possible.

Proposal 8:	The LPP 'method'-RequestLocationInformation messages (with 'method' equal to 'DL-TDOA', 'DL-AoD', and 'Multi-RTT') can include an indication whether fallback to non-AI/ML enhanced location information is allowed/requested or not.

Proposal 9:	For UE-sided models (Case 1/2a) the UE decides whether fallback to non-AI/ML positioning is required, e.g., based on model availability or model performance.

Functionality/Model Monitoring

Proposal 10:	For UE-sided models (Case 1/2a) the model performance monitoring (and associated model performance monitoring metric calculation in label-based model monitoring) is performed at the UE and/or "server for data collection for UE-side model training".

# References

[1]	RP-242399, "Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface".

[2]	RP-240499, "Status Report to TSG RAN#103".

[3]	RP-241153, "Status Report to TSG RAN#104".

[4]	RP-242398, "Status Report to TSG RAN#105".

[5]	RAN2#127bis Chair Notes.

[6]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[7]	3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[8]	3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

[9]	R2-2405070, "LCM for positioning use case", Qualcomm Incorporated.

[10]	R2-2408543, "LCM for positioning use case", Qualcomm Incorporated.

[11]	3GPP TR 38.843: "Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface".