3GPP TSG-RAN2 Meeting #128	R2-2410477

Orlando, USA, Nov 18th – 22nd, 2024

Agenda item:	8.2.2

Source:	Qualcomm Incorporated

Title:	Views on Functionality Aspects of Ambient IoT

WID/SID:	FS_Ambient_IoT_solutions – Release 19

Document for:	Discussion and Decision

# Introduction

Rel-19 Ambient IoT SID [1] describes the objectives including the RAN2 protocol and function studies.

In RAN2 #125bis meeting, the following agreements were made for CP and UP functions separately.

Agreements

- RRC connection management is not supported.  FFS how the resource configuration is provided to the device (if needed based on RAN1 progress)

- RRM L3 measurement reporting is not supported by Ambient IoT devices.

- RAN2 assumes, AIoT devices are not required to support ASN.1 encoding/decoding.

- Periodical System information and MIB are not supported by AIoT devices. This doesn’t preclude any RAN1 defined broadcast signals.

- RAN2 assumes that RRC layer is not necessary between the reader and the device.   RAN2 will continue to study the functionalities required and later discuss whether we will have: 1) a new AS protocol on top of A-IoT MAC layer; or 2) A-IoT MAC

Agreement

- SDAP is not supported for UP protocol stack.

- PDCP layer is not needed.  FFS how to handle AS security (if needed pending SA3 dicsussion) and any other really needed functionalities.

- RLC layer is not needed.   FFS how to handle segmentation (if needed and depending on RAN1 design and upper layer packet size).  RAN2 considers segmentation and reassembly would add complexity, however further discussions are needed.

- No HARQ and RLC AM

- FFS about the level of visibility required by the reader and what information is necessary for AS layer operations.

- RAN2 assumes that no per-packet QoS and no per-QoS flow is supported at AS level (for both UL/DL).  FFS how to handle the general QoS requirements from SA2

RAN2#126 agreed the following regarding the function aspects:

Agreements on functionality

1	Multiple “AIoT logical channels” for upper layer data are not supported.   FFS if AIoT logical channel concept is used depending on final modeling issue.

2	legacy NR BSR/SR is not needed for A-IoT communication.

3	FFS whether further indication of device message size/status is needed

4	AS-layer (above PHY layer) RLC-like sretransmission/repetition is not supported.  This doesn’t preclude the reader and device sending the payload again as new transmission from MAC perspective.   FFS how we handle segmentation case (if needed)

RAN2 #127 made following agreements related to function aspects.

Agreements

-	RAN2 will study the need and use of energy status report to be delivered from the device to the reader in case the device does not have energy for the follow up messages.   FFS details on signaling

-	RAN2 will study the need and use of a simple message “size” reporting to the reader

Agreements

- At least the following information are considered useful to be visible to the reader from CN

-	The service type of A-IoT (e.g. inventory, command) . FFS if more information on command type (e.g. read/write/disable) is useful

-	targeted for one or more than one devices;

-	approximate number of target devices (if available).

FFS on mandatory/optional

2	RAN2 assumes that commands (e.g., read/write/disable) and/or inventory are carried over the AIOT interface as upper layer data.

RAN2 #127bis made following agreements related to function aspects

Agreements on AS ID RAN2 assumes that if “AS ID” is defined it is used at least for purpose of D2R scheduling and R2D reception. Up to RAN1 to decide whether a “AS ID” is defined. RAN2 assumes this “AS ID” should be a short AS layer ID, rather than the full upper layer device ID. FFS on the length. FFS if AS ID can be based on partial upper layer device ID. From RAN2 perspective, following options are possible for “AS ID”: - Option 1: a random ID if used in Msg1 can be reused; - Option 2: reader assigns this “AS ID”. FFS via which R2D message. RAN2 will aim to define one common design for all RA procedures (if technically possible)

Agreements on energy bit indication

Capture in the TR the option: the device may include energy status indication in D2R messages (e.g. MSG1,MSG3 and Command Response message), if the device can.  1 bit indication can be captured in the TR.    We will capture the use case in the TR.   FFS whether it is reader controlled.

Agreements

From RAN2 perspective, it is beneficial for the reader to know an estimate of expected D2R message size.  Two options can be captured: 1) from the CN and 2) from the device (simple message size indication).    Capture the advantages/disadvantages.

The D2R message size would be beneficial but it is not essential.

Ask SA2 if it is possible to provide the expected (e.g. approximate, estimate, exact, max) future response D2R message size.  Is it always available, sometimes, or never.

In this paper, we discuss the functionality aspects of Ambient IoT (AIoT). We will discuss some remaining control plane functionality aspects, followed by user plane functionality aspects and then provide a summary.

# Discussion

The traffic types of DO-DTT and DT with the focus on indoor inventory and command should be considered. The service requirements on inventory procedure and command procedure should be supported from RAN perspective.

In inventory procedure, the AIoT application in core network would like to discover the AIoT devices who are nearby the Readers and sends the inventory request to Readers. The inventory request message may include filter criteria, and other upper layer information. The Reader that receives the inventory request performs the inventory procedure. The Reader forwards the inventory request to all AIoT devices. The inventory command from Reader could be the group signaling which are sent to all AIoT devices who have harvested enough energy and are able to receive.

In RAN2 #125bis meeting, it has been agreed that the AIoT access procedure is triggered by the Reader and Reader may provide the information that the AIoT devices need to respond for the AIoT access trigger. The AIoT devices who are triggered by the inventory request will perform the AIoT access towards the Reader in response to the received inventory request.

Figure 1 Ambient IoT inventory

In command procedure, the AIoT application issues a Command Request including command container, a list of AIoT device identifiers that the command is destined or filter criteria that identify the AIoT devices that are supposed to act upon the command. Therefore, the trigger message sent from Reader may include the device information or filter criteria for triggering AIoT devices to perform AIoT access. The trigger message may also include the command container if AIoT application provides commands to Reader in the command procedure.

## Control plane functionalities

RAN2#125bis discussed about the targeted lower complexity and cost of AIoT devices and agreed that traditional Control Plane functions like RRC connection management, RRM L3 measurement reporting, ASN.1 encoding/decoding, legacy periodical SIB/MIB, legacy paging message, legacy RRC layer, legacy DRX, tracking area update from the device etc. will not be supported by AIoT.

However, RAN2 still has to conclude on some of the remaining CP aspects which will be helpful to resolve the following aspect captured in RAN2#125bis: RAN2 will continue to study the functionalities required and later discuss whether we will have: 1) a new AS protocol on top of A-IoT MAC layer; or 2) A-IoT MAC.

### AIoT Device Registration, Subscription and CN Awareness

Due to the target on very low complexity, cost as well as power consumption, and given that potentially a very large number of AIoT devices may be present in a given vicinity but perform only bursty data transmissions always triggered by the Reader, one of the fundamental architectural differences between AIoT and existing 3GPP technologies may be how the devices are exposed to the Core Network functions.

In our understanding, SA2 would need to discuss and conclude on this. From RAN2 point of view, we think requiring such AIoT devices to support legacy UE-registration-like procedures would be very ambitious not only because of the complexity of the procedure and protocol, but also due to the number of communication steps required. The AIoT devices will not have enough energy source or energy storage to perform multiple steps just for the purpose of registration. Additionally, having a ‘subscription’ similar to legacy UE subscriptions per AIoT device may be a high burden for the core network to manage.

The AIoT device may harvest energy, accumulate it up to the required minimum amount of energy for communication, perform the communication steps and may fully deplete its energy. If the AIoT device runs out of energy after some communication steps, it may not be able to store any configurations, i.e. it will likely ‘forget’ everything. In other words, it is plausible that every communication session starts as if the device has just been powered up from the power off state. Then, it would not make sense to have the AIoT device perform the legacy registration procedures every time it needs to communicate.

It is plausible that for AIoT devices, each communication session may start as if the device has just been powered up from the power off state.

Legacy UE-registration-like procedure is not supported by AIoT.

### NAS functions

NAS control protocol in NR performs the functions listed in TS 23.501 such as registration, authentication, mobility management, security control etc. Supporting all these legacy NAS functionalities by the AIoT devices may be infeasible especially given their complexity, cost, processing power and energy/power constraints.

In our view, the functionalities like security and authentication for AIoT communication should be provided by AIoT function (AIoTF) using a separate protocol. We understand such details are being discussed in SA2.

Legacy NAS functions are not supported by AIoT devices. Consequently, NAS-based data transfer (CP-CIoT-like) is not supported.

RAN2 assumes new AIoT upper layer protocol (e.g. AIoT-AP) between AIoT device and AIoTF will be introduced by SA2.

## User plane functionalities

According to the AIoT UP agreements made in the previous RAN2 meetings, the legacy NR SDAP/PDCP/RLC protocol layers are not supported for AIoT UP protocol. Some functionalities in these protocols may need further discussion that may need other WG’s further inputs as well. The similar function could be implemented in other UP protocol layer or a new L2 layer.

For the AIoT MAC, not all legacy NR MAC functionalities are needed for AIoT.

### Logical and transport channels

RAN2#126 agreed the following.

Multiple “AIoT logical channels” for upper layer data are not supported.   FFS if AIoT logical channel concept is used depending on final modeling issue.

There is still FFS on the AIoT logical channel concept. The NR logical channels are used to carry the control signaling and UP data with different QoS requirements. Similar to NR UP data, the AIoT data can be carried in MAC PDU. In AIoT MAC, the upper layer control signaling and AIoT data can be mapped into one single logical channel and all MAC PDUs belonging to this one logical channel instruct the transport blocks to be delivered to the physical layer. Alternatively, the upper layer control signaling and AIoT data can be mapped to two separate logical channels. AIoT MAC can multiplex the MAC PDU from these two logical channels into one transport block.

The concept of logical channels as in NR MAC is supported for AIoT.

Regarding the transport channels, since RAN1 has already agreed to support PRDCH and PDRCH, from AIoT MAC perspective, at least two transport channels mapping to physical channels, i.e., PRDCH and PDRCH should be supported. Similar to NR DL-SCH and UL-SCH, AIoT should support transport channels RD-SCH and DR-SCH.

At least two transport channels, i.e., RD-SCH and DR-SCH are supported for AIoT.

### Segmentation and reassembly

In RAN2 #127, RAN2 agreed to study a simplified segmentation solution in the AIoT MAC layer considering the study in physical layer [2]. One candidate solution for segmentation was agreed to mainly focus on D2R and can be further considered on R2D. The segmentation number, segment number and the number of segments will not be supported for simplicity. The AIoT device can send an indication to inform Reader whether the data is segmented or whether the transmission data is the last segment. The details of the indication, e.g., the size can be further discussed.

When AIoT device segments the AIoT upper data, the size of segment can be determined by the granted TB size from Reader. One possible way is that the size of segment is always equal to the TB size. In this case, the padding bit may be needed if the rest of upper layer data in AIoT memory is smaller than the scheduled TB size. Alternatively, the segment size could be smaller than the TB size. Then, additional flag might be required to identify the end of segment data. The detailed solution needs more study. Essentially, the AIoT device segments the AIoT upper layer data based on the scheduled TB size from the Reader.

The AIoT device segments the AIoT upper layer data based on the scheduled TB size from the Reader.

RAN2 has agreed to use one indication to inform Reader on whether the data is segmented and whether the transmission AIoT data is the last segment. In addition, RAN2 has confirmed that it is beneficial for Reader to know an estimate of expected D2R message size. Such message size indication can be applied to the segmentation function. It is possible that AIoT device can indicate the simple message size of the remaining AIoT data when performing segmentation. It could be based on AIoT device to indicate the size of remaining data in memory so that Reader can be aware of how many data is left in AIoT memory.

The AIoT device can indicate the D2R ‘message size’ of remaining AIoT data to Reader when performing segmentation.

### AS ID

From upper layer perspective, RAN2 agreed to assume that ‘AS ID’ is used at least for the purpose of D2R scheduling and R2D reception, and such AS ID should be a short AS layer ID rather than the full upper layer device ID. There are two options for RAN2 further discussion. Option 1 is to reuse the random ID if the random ID is in the first D2R message, and while option 2 is to let Reader assign the AS ID to the device.

If random ID is carried in AIoT Msg1 (e.g., 3-step/2-step contention based AIoT access), when Reader echoes the random ID in Msg1 as contention resolution, Reader can already identify the AIoT device. The random ID could be as AS ID once UE receives the AIoT Msg2. Reader and AIoT device can use the random ID as AS ID in the following D2R scheduling and R2D reception.

If random ID is not carried in AIoT Msg1, somehow the AIoT device will send the product device ID (e.g., EPC ID) to Reader during the inventory procedure. Reader can identify the AIoT device according to the product device ID. One simple solution is that Reader and AIoT device can use the partial product device ID as the AS ID which can be carried in the R2D message. Using the partial product device ID is not require AIoT device to maintain any additional ID assigned from Reader and not require AIoT device to store any additional ID.

When random ID is not carried in AIoT Msg1, Reader assigns the AS ID (if needed) from the partial product device ID in a subsequent R2D message to the AIoT device.

When random ID is carried in AIoT Msg1, random ID can be re-used as the AS ID after contention resolution.

# Protocol stack

Based on the above discussion on the needed functionalities, in our view the overall AS protocol stack may look something like below.

Figure 3 AIoT protocol stack

As discussed above, the NR legacy UP protocols, i.e., SDAP/PDCP/RLC are not needed. NAS protocol would be replaced by simplified command protocol terminating at the AIoT Application Function. Note that the exact protocol layers in the Reader facing the AF side is not shown since that is unchanged and depends on whether Reader is a separate entity or a part of BS or UE based on topology which could be discussed in RAN3.

In our view, simplified AIoT protocol stack should be same for both CP and UP protocols, i.e., there is no need of CP/UP differentiation.

Simplified AIoT protocol stack is unified for AIoT CP and UP (i.e. no differentiation between CP and UP protocol stacks).

# Conclusion

We have the following observations, and we’d recommend RAN2 to discuss and adopt the following proposals:

Observation 1.	It is plausible that for AIoT devices, each communication session may start as if the device has just been powered up from the power off state.

Proposal 1:	Legacy UE-registration-like procedure is not supported by AIoT.

Proposal 2:	Legacy NAS functions are not supported by AIoT devices. Consequently, NAS-based data transfer (CP-CIoT-like) is not supported.

Proposal 3:	RAN2 assumes new AIoT upper layer protocol (e.g. AIoT-AP) between AIoT device and AIoTF will be introduced by SA2.

Proposal 4:	The concept of logical channels as in NR MAC is supported for AIoT.

Proposal 5:	At least two transport channels, i.e., RD-SCH and DR-SCH are supported for AIoT.

Proposal 6:	The AIoT device segments the AIoT upper layer data based on the scheduled TB size from the Reader.

Proposal 7:	The AIoT device can indicate the D2R ‘message size’ of remaining AIoT data to Reader when performing segmentation.

Proposal 8:	When random ID is not carried in AIoT Msg1, Reader assigns the AS ID (if needed) from the partial product device ID in a subsequent R2D message to the AIoT device.

Proposal 9:	When random ID is carried in AIoT Msg1, random ID can be re-used as the AS ID after contention resolution.

Proposal 10:	Simplified AIoT protocol stack is unified for AIoT CP and UP (i.e. no differentiation between CP and UP protocol stacks).

# References

- RP-240826, Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA

- R2-2409405, Reply LS to RAN2 on data block sizes for Ambient IoT, Mediatek.