3GPP TSG-RAN WG2 Meeting #128	R2-2410483

Orlando, Florida, USA 18th – 22nd November, 2024

Agenda item:	8.9.3

Source:	Samsung

Title:	Procedures for contention-based Msg3

WID/SID:	IoT_NTN_Ph3-Core

Document for:	Discussion and Decision

# Introduction

In RAN#105 the latest WID for IoT NTN Phase 3 is provided [1]:

The aim of this WI is enhancement of IoT-NTN with the following objectives: - Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required - Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete - Study and specify RRM requirement, if identified [RAN4] - Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2] Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI. Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

In this contribution, we provide some discussions on the objective of Support of Capacity enhancements for the uplink.

# Discussion

- Agreements from the last meeting:

Agreements:

1.	RAN2 will introduce support for DSA (i.e. the possibility to transmit more than one replica of CB-msg3, if configured by the NW). RAN2 does not intend to work on CRDSA in Rel-19.

2.	RAN2 will continue their work on CB-msg3 assuming that OCC2 might also apply to CB-msg3 transmission (“CB-NPUSCH”) (final decision whether this is feasible is up to RAN1). FFS whether an LS to indicate this to RAN1 is needed.

3.	At least system Information is used to provide CB-msg3 EDT cell-specific PUSCH resources for Msg3 transmission. FFS on signalling details.

4.	CB-msg3 EDT cell specific PUSCH resources for Msg3 transmission are provided per CE level (FFS whether we have a CE level specific configuration for DSA)

5.	RAN2 assumes that CB-msg3 EDT cell specific PUSCH resources are associated with number of repetitions, RSRP selection threshold to determine the CE level and largest TBS for Msg3 transmission, but this has to be confirmed by RAN1. FFS if there is an RSRP threshold that determines whether CB-msg3 EDT cannot be used (the UE will have to use 4-step RA)

6.	At least in the cases confirmed by RAN1/RAN4, a running TAT is not needed to initiate a CB-msg3 EDT transmission

7.	The RNTI used at least to schedule Msg4 transmission is derived based on the resource associated to the PUSCH occasion used for contention based Msg3 EDT transmission (FFS on the details. FFS how this is impacted by DSA)

8.	CB-msg3 EDT procedures and any Msg4 enhancement are only introduced for IoT NTN.

## Configuration of Contention-based Msg3 resources

For configuring the resources for contention-based Msg3 resources, it could be useful to agree and define some basic configurations for the purpose of future discussions. Some basic configurations can be seen in Figure 1.

Figure 1.

A number of PUSCH occasions for contention-based Msg3 is considered to be a Contention-based Msg3 frame. This will come in handy when attempting to define DSA. For this, there needs to be a periodicity and offset defined along with a number of Msg3 PUSCH occasions defined. Further down the line there will also need to be PRB allocation information which defines where in frequency and with how many PRBs that each Msg3 PUSCH occasion shall have. Therefore we propose:

Proposal 1: Network configures a set of Contention-based Msg3 PUSCH occasions in a Contention-based Msg3 frame.

Proposal 2: The Contention-based Msg3 frame consist of a periodicity and offset, and a number of Msg3 PUSCH occasions.

## DSA

In the last meeting the following was agreed:

1.	RAN2 will introduce support for DSA (i.e. the possibility to transmit more than one replica of CB-msg3, if configured by the NW). RAN2 does not intend to work on CRDSA in Rel-19.

In our understanding, support for DSA means that multiple PUSCH occasions will be selected in a contention-based Msg3 resource frame in a random fashion. The number of PUSCH occasions to select in a contention-based Msg3 resource frame can be made configurable.

Proposal 3: For DSA, the UE will be configured to randomly select and transmit in multiple contention-based Msg3 PUSCH occasions. The number of PUSCH occasions to select may be configurable.

## Response timer

In simplified terms, for eMTC and NB-IoT Msg4 the UE starts monitoring for Msg4 in the last subframe of the last repetitions.

Such behaviour could also potentially be used for contention-based msg3. In other words, the UE selects a Msg3 PUSCH occasion from the Contention-based Msg3 resource frame, and then starts to monitor based on the subframe of the PUSCH occasion plus the UE-eNB RTT. This is sufficient to ensure that both UE and the network is synchronized on when a response may be transmitted.

In DSA however, the UE would select a set of PUSCH occasions in a contention-based Msg3 resource frame in a random manner. And the reception of any of the transmissions will be subject to failure due to general decoding failure or due to collisions. This means that if the UE starts a response timer based on one of the transmissions, the network and the UE will not be aligned on when a response may arrive. This can be seen in Figure 2.

Observation 1: For DSA, the network and UE may not be aligned on the UEs response monitoring window if the response window is start based on the transmission in a PUSCH occasion.

Figure 2. Issue of response timer for DSA.

For this reason, RAN2 needs to discuss from exactly when the UE shall start any type of response timer.

Proposal 4: For DSA when UE selects multiple PUSCH occasions in a Contention-based Msg3 resource frame, RAN2 to discuss how a response timer shall be started to ensure UE and network is aligned on the monitoring window.

## RNTI, Msg4 and identifying a UE

In the last meeting the following was agreed:

7.	The RNTI used at least to schedule Msg4 transmission is derived based on the resource associated to the PUSCH occasion used for contention based Msg3 EDT transmission (FFS on the details. FFS how this is impacted by DSA)

In the case of selecting a single subframe to perform contention-based Msg3, deriving the RNTI based on the PUSCH occasion be done through a formula, similar to how the RA-RNTI for monitoring for RAR is calculated. Such formula may for instance include the subframe of the PUSCH occasion, frequency layer, if the frame can be TDMed etc.

Figure 3. A) selecting multiple PUSCH occasions, each with separate RNTI. B) selecting a single RNTI based on the first selected PUSCH occasion.

However, in DSA, the UE will randomly select a set of PUSCH occasions. In this case, it needs to be carefully studied how the UE selects the RNTI. For instance, if it is decided that the UE selects an RNTI for each selected PUSCH occasion, then the UE may need to monitor for multiple RNTIs, as seen in Figure 3A. If on the other hand the UE uses only a single RNTI, for instance based on the first selected PUSCH Occasion, or last selected PUSCH Occasion, the network needs to monitor for all possible RNTIs in every PUSCH Occasion. This is because the network cannot know which PUSCH Occasion that the UE has selected, as it is randomly selected. This can be seen in Figure 3B.

Observation 2: For DSA, if RNTI is based on each selected PUSCH occasion, the UE may have to monitor for multiple RNTIs.

Observation 3: For DSA, if RNTI is based on first selected PUSCH occasion, the network may have monitor for all combinations of RNTIs.

Proposal 5: For DSA when UE selects multiple PUSCH occasions in a Contention-based Msg3 resource frame, RAN2 to discuss how to determine the RNTI.

## Fallback procedures

In some circumstances, depending on the configuration, it may be expected that contention-based Msg3 transmission would be less reliable compared to legacy random access. Failure rate can be dependent on the synchronization of the network and the UE, the radio characteristics and the load on the contention-based Msg3 resources. Given that the legacy random access should be more reliable and able to take a higher load, a UE should be able to fall back towards legacy random access. There are two ways in which fallback to random access with RACH; 1) the UE detects the failure and initiates the fallback and 2) the network detects a failure and signals to a UE to fallback to random access with RACH. We think that it would make sense to have both options available.

Proposal 6: If contention-based Msg3 fails, it should be possible to fallback to random access with RACH.

Proposal 7: RAN2 introduces UE-initiated Msg3 fallback and network-signalled Msg3 fallback.

If the contention-based Msg3 attempts fails, it may also be argued that the UE may be scheduled directly, similar to the case of NR 2-step random access via the fallbackRAR. However, we believe that this complicates the scheme and unlike 2-step random access where there is both a preamble and a PUSCH which enables the network to distinguish a UE even if the MsgA PUSCH fails, contention-based Msg3 only has a PUSCH transmissions. Therefore, we do not think that fallback to scheduled Msg3 PUSCH is needed for contention-based Msg3.

Proposal 8: Fallback to scheduled Msg3 is not introduced for contention-based Msg3 procedures.

# Conclusion

In this contribution we discussion initial issues on uplink capacity enhancements:

Observation 1: For DSA, the network and UE may not be aligned on the UEs response monitoring window if the response window is start based on the transmission in a PUSCH occasion.

Observation 2: For DSA, if RNTI is based on each selected PUSCH occasion, the UE may have to monitor for multiple RNTIs.

Observation 3: For DSA, if RNTI is based on first selected PUSCH occasion, the network may have monitor for all combinations of RNTIs.

Proposal 1: Network configures a set of Contention-based Msg3 PUSCH occasions in a Contention-based Msg3 frame.

Proposal 2: The Contention-based Msg3 frame consist of a periodicity and offset, and a number of Msg3 PUSCH occasions.

Proposal 3: For DSA, the UE will be configured to randomly select and transmit in multiple contention-based Msg3 PUSCH occasions. The number of PUSCH occasions to select may be configurable.

Proposal 4: For DSA when UE selects multiple PUSCH occasions in a Contention-based Msg3 resource frame, RAN2 to discuss how a response timer shall be started to ensure UE and network is aligned on the monitoring window.

Proposal 5: For DSA when UE selects multiple PUSCH occasions in a Contention-based Msg3 resource frame, RAN2 to discuss how to determine the RNTI.

Proposal 6: If contention-based Msg3 fails, it should be possible to fallback to random access with RACH.

Proposal 7: RAN2 introduces UE-initiated Msg3 fallback and network-signalled Msg3 fallback.

Proposal 8: Fallback to scheduled Msg3 is not introduced for contention-based Msg3 procedures.

# Reference

RP-242397, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, Mediatek, RAN#105, Melbourne, Australia, September 2024.