3GPP TSG-RAN WG2 Meeting #128		R2-2410489

Orlando, USA, Nov 18th – 22nd, 2024

Agenda Item: 	8.1.3

Source:		Sony

Title:	Some aspects for NW side data collection

Document for:	Discussion & Decision

# Introduction

In this contribution we provide our views on NW side data collection for beam management. We also discuss the quality of input data for inference and the resource optimization for model monitoring.

# Discussions

# 2.1NW side data collection (BM)

RAN1 FL has provided the discussion points for the RAN1#119 meeting, as the following proposal:

##### (for 119) Proposal 5.1: For NW-sided model, for BM-Case 2, support to report the measurement results of multiple transmission occasions of resources for channel measurement at least for the RS types including P CSI-RS, SP CSI-RS and SSB in one L1 report. - Differential L1-RSRP reporting over multiple time instances is used - FFS on how to determine the multiple occasion(s) - FFS whether include the time stamp information explicitly or implicitly in the report - FFS on the max number of RS resources to be reported for one transmission occasion - FFS on the max number of the multiple transmission occasions. Note: Purpose, such as above “For NW-sided model, for BM-Case 2”, will not be specified in RAN 1 specifications

For BM-Case 2, if the measurement results from multiple transmission occasions of resources are included in one report, the time-related information corresponding to the measurement results should be explicitly or implicitly reported to distinguish these results from different transmission occasions in the time domain. And we think the max number of multiple transmission occasions may rely on the format of model’s input. Although for the NW-side model, the time window for input data collection and the maximum number of transmissions occasions within that window should be indicated to the UE, this will ensure that no additional unnecessary measurements are taken.

Observation 1: For BM-Case 2 and the NW-side model, the time-related information corresponding to the measurement results from different transmission occasions may be explicitly or implicitly reported.

This would imply that MDT framework should allow timestamp information for immediate MDT as well.

Proposal 1: For immediate MDT, UE report the time-related information corresponding to the measurement results from different transmission occasions.

Regarding time window, RAN1 should discuss the maximum number of transmission occasions within that window should be indicated to the UE, which should be based on the format of the model's input.

## 2.2 model monitoring resource optimization

For BM-Case 2, performing model monitoring needs to measure the full resource set configured by NW which results in significant measurement overhead and latency. Especially, in the case where the performance metric is calculated for multiple time instances to avoid the "ping-pong" effect. To reduce this overhead, we propose reusing part of the input data from the previous inference instance for each new inference instance. This approach can be illustrated as following picture: for each inference instance, the model's input is based on measurements from five consecutive slots and predict for future four consecutive slots. In the first inference instance, the input data is derived from the measurements of slots one through five. In the second inference instance, the input data is collected from two parts: the new measurement from slot six and the reused measurement data from slots two through five, which were measured in the first inference instance.

This approach doesn’t need NW to configure the full set of resources according to the model's input requirements. Instead, only a portion of the resources needs to be configured, while the other part of the input data is derived from the measurements obtained in the previous instance. This approach can significantly reduce the measurement overhead.

In this figure, the candidate beam in slot eight has been predicted in multiple inference instances. If the predicted results in each instance are the same or similar, this indicates that the model's performance is stable and suitable for the current scenario. Conversely, if the predicted results vary significantly, it may suggest that the model is failing to infer accurately.

Proposal 2: For BM-Case2, Further study is needed to reduce measurement overhead based on the RS resource set configured by NW in model monitoring.

## 2.3 Quality of collected data

gNB will configure a set of beams for UE to measure to collect input data. Hence, Set C [1] has been introduced. Set B as the model input can be chosen from Set C. We understand that model input data can be filtered based on specific rules, such as ensuring that the RSRP obtained through measurement is higher than a predefined threshold.

However, a challenge arises when there may not be enough high-quality input data available for the model. This can lead to two possible outcomes: firstly, inputting inadequate data into the model, resulting in a decline in predicted accuracy. If the predicted accuracy decreases significantly, the predicted beams are unsuitable for link communication. Alternatively, even if there is a decline in predicted accuracy, the results may still satisfy the link quality requirements. No matter whichever is the outcome, the gNB needs to understand the reason of the decline in model performance.

Hence, when the UE does not collect enough high-quality data for Set B from Set C, it should report to the gNB. Subsequently, the gNB can configure a new set for collecting input data. If the UE still cannot obtain high-quality data in the new set, it indicates that the scenario has changed and is no longer applicable for the current serving model. Model switching or fallback can be considered afterwards. We think this should in general apply to all use cases.

Proposal 3: At the inference stage, the UE report that the quality of input data has gone bad, to the gNB when the measurement results fail to meet the model's input requirements (applies to both beam management and positioning).

# Summary

In this contribution, we have the following proposal and observations for RAN2 to agree:

Observation 1: For BM-Case 2 and the NW-side model, the time-related information corresponding to the measurement results from different transmission occasions may be explicitly or implicitly reported.

Proposal 1: For immediate MDT, UE report the time-related information corresponding to the measurement results from different transmission occasions.

Proposal 2: For BM-Case2, Further study is needed to reduce measurement overhead based on the RS resource set configured by NW in model monitoring.

Proposal 3: At the inference stage, the UE report that the quality of input data has gone bad, to the gNB when the measurement results fail to meet the model's input requirements (applies to both beam management and positioning)

# References

- RP-240774: AI/ML for NR air interface