3GPP TSG RAN WG2 Meeting #128	R2-2410517

Orlando, USA, 18 - 22 November 2024

Agenda Item:	7.2.3

Source:	Huawei, HiSilicon

Title:	MAC CR rapporteur summary

Document for: Discussion and Decision

# 1	Introduction

This document is a summary of proposals on TS 38.321 for LTM.

# 2	Discussion

The following documents include proposals for TS 38.321:

| R2-2409777 | Coexistence of (e)RedCap UEs and LTM and other MAC corrections | vivo |
| --- | --- | --- |
| R2-2410461 | Remaining issues on LTM | Ericsson |
| R2-2410515 | Further MAC corrections | Huawei, HiSilicon |
| R2-2410628 | Clarification to 38.321 on RACH Initiation for RACH based LTM | ZTE Corporation |

## 2.1	Coexistence with RedCap (P1 in R2-2409777)

The proposal is to confirm that coexistence of RedCap and LTM is supported, and there is a TP adding two notes to the LTM cell switch MAC CE description about a restriction in the case msg1 repetition is also used, so this topic concerns a combination of WIs (NR_Mob_enh2-Core, NR_cov_enh2-Core, NR_redcap-Core, NR_redcap_enh-Core).

There is a CR in R2-2410516 with almost the same change, submitted to AI 7.0.2.17 that is expected to cover cross WI issues.

Proposal 1: Consider changes that concern several WIs (P1 in R2-2409777) in the discussions of AI 7.0.2.17.

## 2.2	Selection / indication of SSB to lower layers (P2 in R2-2409777 / P1 R2-2410515)

There is the following text in TS 38.321 clause 5.8.2:

For an uplink grant configured for configured grant Type 1 for RACH-less LTM cell switch, when there is an ongoing RACH-less LTM cell switch procedure, for each configured uplink grant valid according to TS 38.214 [7] for which the above formula is satisfied, the MAC entity shall:

1>	if an SSB corresponding to the configured UL grant has the same SSB index as the SSB associated with the TCI state indicated by the UL TCI state ID field, if present, or by the TCI state ID field otherwise, in the LTM Cell Switch Command MAC CE, as specified in clause 5.18.35:

2>	select the SSB associated with the TCI state indicated by LTM Cell Switch Command MAC CE.

2>	indicate the SSB index to the lower layer;

2>	consider this configured uplink grant as valid.

1>	else:

2>	consider this configured uplink grant as not valid.

NOTE 1a:	When there is an ongoing RACH-less LTM cell switch, the configured grant Type 1 which is not specifically configured for LTM (see cg-LTM-Configuration in TS 38.331 [5]) is not used.

NOTE 1b:	After completion of LTM cell switch, the UE stops using the grant configured for RACH-less LTM cell switch (see cg-LTM-Configuration in TS 38.331 [5]).

However, in the MAC CE procedure in 5.18.35, there is:

3>	indicate to lower layers the information regarding the TCI state information included in the LTM Cell Switch Command MAC CE.

Therefore, the above indication looks redundant.

Besides, the QCL RS in the indicated TCI state information may not be SSB, and there is no reason to ask the UE to "select an SSB" or indicate it to lower layers. Therefore, the above indication looks irrelevant.

Proposal 2: Remove "indicate the SSB index to the lower layer" from 5.8.2.

Proposal 3: Remove "select the SSB associated with the TCI state indicated by LTM Cell Switch Command MAC CE" from 5.8.2.

## 2.3	Text on pending SR (R2-2410461)

According to R2-2410461:

Let’s assume that the UE receives an LTM cell switch MAC CE within any TA value and the UE is not configured to calculated the TA on its own. In such a case, the UE will determine that the LTM cell switch procedure is with Random Access and the initiation of the Random Access procedure happens when the UE needs to send the RRCReconfigurationComplete message. According to this, the UE receives a UL grant in the RAR from the network and according to this the UE has two options:

- The UE sends the RRCReconfigurationComplete message according to the received UL grant within the RAR

- The UE uses the UL grant in the RAR to send the BSR (without any SR because the UE has already the grant) in case there is also data to be sent together with the RRCReconfigurationComplete message

In both cases, the SR is never pending and this mean that the UE may never enter in that part of the procedural text highlighted in green.

The text referred to is (from 5.4.4 Scheduling Request):

As long as at least one SR is pending, the MAC entity shall for each pending SR:

1>	if the MAC entity has no valid PUCCH resource configured for the pending SR; and

1>	if there is no ongoing RACH-less LTM cell switch; and

1>	if rach-LessHO is not configured:

2>	initiate a Random Access procedure (see clause 5.1) on the SpCell and cancel the pending SR.

<omitted text>

The rapporteur understands that the trigger to do RA in L3 handover is:

-	new data arrive in the buffer for SRB1, triggering BSR;

-	since there is no UL grant, BSR triggers SR;

-	since there is no valid PUCCH resource (no TA), the UE initiates RA.

If this is correct, SR is also triggered in the case of LTM cell switch. In the RACH-less LTM cell switch case, the UE has a TA so the condition "if the MAC entity has no valid PUCCH resource configured for the pending SR" is false and hte extra condition "if there is no ongoing RACH-less LTM cell switch" may not be useful.

However, the rapporteur would like companies to confirm the above understanding.

Proposal 4a: Discuss whether, in RACH-less LTM cell switch, SR is triggered but there is valid PUCCH resource configured for the pending SR so random access is not triggered by the text in 5.4.4, without the need for a special condition on "no ongoing RACH-less LTM cell switch".

Proposal 4b: If the above understanding is confirmed, remove the unnecessary bullet.

## 2.4	Configured uplink grant occasions (P2 in R2-2410515)

It is raised that RAN1 has adopted the following text in in TS 38.213 v18.4.0 clause 21.1:

# 21	L1/L2-triggered mobility procedures

[...]

## 21.1	Configured-grant PUSCH transmission in RACH-less LTM cell switch

[...]

If the UE is provided cg-LTM-Configuration, the UE performs configured grant Type 1 PUSCH transmission on the valid PUSCH occasions associated with the SS/PBCH block indexes same as the SS/PBCH block indexes provided by or associated with QCL RS of the CandidateTCI-State or CandidateTCI-UL-State indicated by the LTM Cell Switch Command MAC CE.

This seems to covers the same like clause 5.8.2 of TS 38.321:

For an uplink grant configured for configured grant Type 1 for RACH-less LTM cell switch, when there is an ongoing RACH-less LTM cell switch procedure, for each configured uplink grant valid according to TS 38.214 [7] for which the above formula is satisfied, the MAC entity shall:

1>	if an SSB corresponding to the configured UL grant has the same SSB index as the SSB associated with the TCI state indicated by the UL TCI state ID field, if present, or by the TCI state ID field otherwise, in the LTM Cell Switch Command MAC CE, as specified in clause 5.18.35:

[...]

2>	consider this configured uplink grant as valid.

However, the wording is "provided or associated with" in TS 38.213 but it is only "associated with" in TS 38.321.

As the details of the relation between SSB and TCI states is more in RAN1 scope, it is proposed to align the wording in TS 38.321 with TS 38.213 and refer to it.

For an uplink grant configured for configured grant Type 1 for RACH-less LTM cell switch, when there is an ongoing RACH-less LTM cell switch procedure, for each configured uplink grant valid according to TS 38.214 [7] for which the above formula is satisfied, the MAC entity shall:

1>	if an SSB corresponding to the configured UL grant has the same SSB index as the SSB provided by or associated with the TCI state indicated by the UL TCI state ID field, if present, or by the TCI state ID field otherwise, in the LTM Cell Switch Command MAC CE, as specified in clause 21.1 in TS 38.213 [6]:

Proposal 5: Discuss whether to add "provided by or" before "associated with" and refer to clause 21.1 in TS 38.213.

## 2.5	RACH Initiation for RACH based LTM (R2-2410628)

The document argues that if the UE follows the procedure, the UE will first create the RRCReconfigurationComplete message, then trigger BSR, then SR, then RACH, and the delay due to this could make the UE miss a RA occasion.

To save time, R2-2410628 proposes to specify an explicit trigger for RA in the reception of the LTM cell switch MAC CE, as an else branch.

This proposal was already made in RAN2#127bis and not agreed and the rapporteur does not notice any new argument.

The rapporteur's understanding is that:

-	a (clever ?) UE implementation can in parallel create the RRCReconfigurationComplete message and initiate RACH, without the need to change anything in TS 38.321;

-	for SCG LTM when SRB3 is not configured, RACH is triggered by TS 38.331 because there is no message, so adding a trigger means RACH is triggered twice in this case;

Proposal 6: Confirm that TS 38.321 already allows UE implementations to send a RA preamble and create the RRCReconfigurationComplete message in parallel (for LTM cell switch and for any L3 handover), so no change to TS 38.321 is needed.

# 3	Conclusion

This contribution makes the following proposals:

Proposal 1: Consider changes that concern several WIs (P1 in R2-2409777) in the discussions of AI 7.0.2.17.

Proposal 2: Remove "indicate the SSB index to the lower layer" from 5.8.2.

Proposal 3: Remove "select the SSB associated with the TCI state indicated by LTM Cell Switch Command MAC CE" from 5.8.2.

Proposal 4a: Discuss whether, in RACH-less LTM cell switch, SR is triggered but there is valid PUCCH resource configured for the pending SR so random access is not triggered by the text in 5.4.4, without the need for a special condition on "no ongoing RACH-less LTM cell switch".

Proposal 4b: If the above understanding is confirmed, remove the unnecessary bullet.

Proposal 5: Discuss whether to add "provided by or" before "associated with" and refer to clause 21.1 in TS 38.213.

Proposal 6: Confirm that TS 38.321 already allows UE implementations to send a RA preamble and create the RRCReconfigurationComplete message in parallel (for LTM cell switch and for any L3 handover), so no change to TS 38.321 is needed.

# 4	References

- R2-2409777	Coexistence of (e)RedCap UEs and LTM and other MAC corrections,	vivo

- R2-2410461	Remaining issues on LTM,	Ericsson

- R2-2410515 Further MAC corrections,	Huawei, HiSilicon

- R2-2410628	Clarification to 38.321 on RACH Initiation for RACH based LTM,	ZTE Corporation