3GPP TSG-RAN WG2 Meeting #128	R2-2410523

Orlando, USA, Nov 18th – 22nd, 2024

Agenda item:	8.12.2

Source:	Samsung

Title:	Discussion on Asymmetric DL sTRP/UL mTRP

WID/SID:	NR_MIMO_Ph5

Document for:	Discussion and Decision

# Introduction

In RAN2#127bis meeting, RAN2 had some initial discussion on asymmetric DL sTRP/UL mTRP.

Asymmetric DL sTRP/UL mTRP - Ericsson agree that we need a new MAC CE. Nokia, LG E agrees. - Vivo not sure whether a new MAC CE is needed. - Xiaomi prefer to have clean MAC CE design. - Sony wonders what is the # of UL TRP for this objective. Samsung think it is multiple. - HW think it is problematic if we configure a PL offset value in RRC and then modify via MAC CE, not sure if the proposals work. CMCC think there are existing examples, and it works. ZTE think for this potential issue we should first decide whether ‘delta’ offset is supported. Xiaomi think we need to use delta offset configuration if we use MAC CE to update the offset value. Ericsson think we should ask R1. - Samsung think we are not ready to already send questions to R1 in this meeting. Chair: companies are encouraged to check the listed questions in P6 in R2-2409023. We will discuss whether/what to ask to R1.

In this paper, we discuss more details on the PL offset MAC CE and other aspects of asymmetric DL sTRP/UL mTRP from RAN2 perspective.

# Discussion

In the so-called “asymmetric DL sTRP/UL mTRP” scenario, we consider dense UL-only TRPs, where UEs receives DL from a macro DL/UL TRP and can transmit UL to micro UL-only TRPs, as shown in the above Figure.

In this scenario, we consider a pathloss offset between the DL pathloss RS from the DL TRP and the actual pathloss for UL-only TRP.  Due to this pathloss offset, UL power control to the UL-only TRP are determined differently from the legacy power control. The PL offset is applied in PUCCH, PUSCH, SRS, PDCCH-order PRACH in FR1 as RAN1 agreed. The PL offset is used in UL power control in a way that UE reuses the legacy uplink power control formulation by replacing legacy PL with a PL which is derived from the DL PL RS and the PL offset.

RAN1#116 Agreement For the asymmetric DL sTRP/UL mTRP deployment scenarios, support to associate a UL TCI state with a PL offset: - When a UL TCI state associated with a PL offset is applied for the PUSCH/PUCCH/SRS transmission, the UE shall calculate the Tx power of the PUSCH/PUCCH/SRS based on the DL PL RS and PL offset associated with this UL TCI state. - Reuse the legacy uplink power control formulation by replacing legacy PL with UL PL which is derived from the DL PL RS and the PL offset. RAN1#116bis Agreement For FR1, a joint TCI state can be associated with a PL offset. - When a joint TCI state associated with a PL offset is applied for the PUSCH/PUCCH/SRS transmission, the UE shall calculate the Tx power of the PUSCH/PUCCH/SRS based on the DL PL RS and PL offset associated with this joint TCI state. - Reuse the legacy uplink power control formulation by replacing legacy PL with a PL which is derived from the DL PL RS and the PL offset. Agreement Support applying PL offset on PDCCH-order PRACH towards a UL TRP in FR1. - Note: The DL reference timing determination for PDCCH-order PRACH transmission to an UL TRP is still based on the DL RS defined in current RAN4 specification - Above is subject to a separate UE capability signaling

## PL offset MAC CE

Based on the LS R1-2407285 and the following RAN1 agreements, RAN1 has agreed to use MAC CE to update the PL offset values for joint/UL TCI states. Relevant RAN1 agreements are listed as follows.

RAN1#116bis Agreement For the association between PL offset and joint/UL TCI state, consider and down-select one from the following Alts: - Alt1a: One PL offset value is configured in a joint or UL TCI state by RRC only - Alt1b: One PL offset value is configured in a joint or UL TCI state by RRC. A MAC CE can update the PL offset value(s) for joint or UL TCI state(s). - Alt2a: A list of PL offset configurations is configured by RRC in BWP/CC and each PL offset configuration contains one PL offset value. One new RRC parameter is introduced in a joint or UL TCI state to indicate one of the configured PL offset configurations. - Alt2b: A list of PL offset configurations is configured by RRC in BWP/CC and each PL offset configuration contains one PL offset value. One new RRC parameter is introduced in a joint or UL TCI state to indicate one of the configured PL offset configurations. A MAC CE can update the association between a joint or UL TCI state and PL offset configuration - Alt3: A list of PL offset configurations is configured by RRC in BWP/CC and each PL offset configuration contains one PL offset value. A MAC CE can activate/indicate one PL offset configuration for each activated joint or UL TCI state. In each joint or UL TCI state, the initial PL offset value is 0dB. - Alt4: A list of PL offset values is provided in a joint or UL TCI state by RRC. Each PL offset value is applied to a corresponding measured PL range. Other alternatives are not precluded. RAN1#117 Agreement For the association between PL offset and joint/UL TCI state, support the following - Alt1b: One PL offset value is configured in a joint or UL TCI state by RRC, where different PL offset values can be configured to different joint or UL TCI states. A MAC CE can update the PL offset value(s) for joint or UL TCI state(s). RAN1#118 Agreement For the asymmetric DL sTRP/UL mTRP scenarios, the value range of PL offset includes at least [-10, 60] dB - FFS: Extending the range to lower than -10dB - Step size is 4dB RAN1#118bis Agreement The lower limit of the value range of PL offset is extended to -12 dB

As we can see, RAN1 has considered various options to configure/indicate PL offset for joint/UL TCI state(s). The following option is finally agreed

- A PL offset value is configured per joint/UL TCI state, MAC CE is used to update the PL offset value(s) for joint/UL TCI states.

- The value range of PL offset is [-12, 60] dB, with a step size of 4dB.

In the last meeting, we have initially discussed to which joint/UL TCI state the PL offset update by MAC CE is applied. Some think the PL offset update by MAC CE may only need to be applied to the activated joint/UL TCI states. In our understanding of the RAN1 agreements, the PL offset configured for any joint/UL TCI state can be updated by the MAC CE.

The reason is that the PL offset is applied to all UL transmissions on PUCCH, PUSCH, SRS, and FR1 PDCCH order PRACH in asymmetric DL sTRP/UL mTRP scenario. And Rel-17/18 unified TCI state framework is applied, according to the following RAN1 agreement.

RAN1#117 Conclusion For the asymmetric DL sTRP/UL mTRP deployment scenario, reuse the rel-17 unified TCI/ICBM and rel-18 unified TCI framework: - When rel-17 unified TCI/ICBM is configured: - For FR1: one joint TCI state or {one DL TCI state + one UL TCI state} can be applied. - For FR2: {one DL TCI state + one UL TCI state} can be applied. - When rel-18 unified TCI is configured: - For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied. - For FR2: {one DL TCI state + up to two UL TCI states} can be applied.

In Rel-17 unified TCI state framework for single-TRP and ICBM,

- PUCCH and PUSCH use DCI indicated joint/UL TCI state.

- Aperiodic (AP) SRS for beam management (BM) and AP/SP/Periodic SRS for codebook (CB), non-codebook (NCB), and antenna switching (AS) that is configured with followUnifiedTCIstateSRS uses DCI indicated joint/UL TCI state.

- AP SRS for BM that is not configured with followUnifiedTCIstateSRS, AP/SP (semi-persistent) SRS for CB, NCB, and AS that is not configured with followUnifiedTCIstateSRS, and SP SRS for BM use MAC CE signalled joint/UL TCI state (SP/AP SRS TCI State Indication MAC CE).

- Periodic SRS for CB, NCB, and AS that is not configured with followUnifiedTCIstateSRS, and periodic SRS for BM use the joint/UL TCI state configured in SRS-resource.

In Rel-18 unified TCI state framework for single-DCI multi-TRP,

- PUSCH with DG or type-2 CG use 1st or 2nd or both of the DCI indicated joint/UL TCI state(s) according to DCI field SRS resource set indicator.

- PUSCH with type-1 CG and PUCCH use 1st or 2nd or both of the DCI indicated joint/UL TCI state(s) according to RRC field applyIndicatedTCIState.

- SRS for CB/NCB/AS and AP SRS for BM that is configured with followUnifiedTCIstateSRS use 1st or 2nd or both of the DCI indicated joint/UL TCI state(s) according to RRC field applyIndicatedTCIState.

- AP SRS for BM that is not configured with followUnifiedTCIstateSRS, AP/SP SRS for CB, NCB, and AS that is not configured with followUnifiedTCIstateSRS, and SP SRS for BM use MAC CE signalled joint/UL TCI state (SP/AP SRS TCI State Indication MAC CE).

- Periodic SRS for CB, NCB, and AS that is not configured with followUnifiedTCIstateSRS, and periodic SRS for BM use the joint/UL TCI state configured in SRS-resource.

Based on the above, UE does not always use the DCI indicated joint/UL TCI state from the activated ones by MAC CE (i.e., Enhanced Unified TCI States Activation/Deactivation MAC CE). For some SRS, UE uses RRC configured or MAC CE indicated joint/UL TCI state which are not from the activated ones.

Observation 1: The PL offset is applied to all UL transmission on PUCCH, PUSCH, SRS, and FR1 PDCCH order PRACH, for which UE can use DCI-indicated or MAC CE indicated, or RRC configured joint/UL TCI state(s) in different cases.

Hence, we think PL offset should be configured per joint/UL TCI state by RRC, and PL offset update by MAC CE should be applicable to all joint/UL TCI state configured with a PL offset, not only limited to activated TCI states in Enhanced Unified TCI States Activation/Deactivation MAC CE. As RAN1 agreed, the value range of the PL offset is [-12, 60] dB with step size 4dB.

Proposal 1: PL offset is configured per joint/UL TCI state in RRC, with enumerated values from -12dB to 60dB with step size 4dB.

Proposal 2: PL offset update by MAC CE is applicable to all joint/UL TCI state configured with PL offset.

Another issue briefly mentioned in the last meeting is that whether the existing Enhanced Unified TCI States Activation/Deactivation MAC CE can be extended to indicate PL offset update. Based on the analysis above, we think a clean design is to introduce a new MAC CE to indicate PL offset update, instead of extending the existing MAC CE since the relevant joint/UL TCI states are not limited to the activated one. And the new MAC CE should be identified by an eLCID.

Proposal 3: Introduce a new MAC CE for PL offset update, namely, PL Offset Update MAC CE, which is identified by an eLCID.

On another issue raised in the last meeting, how to handle the RRC configured PL offset and the MAC CE updated PL offset for the same joint/UL TCI state should be clarified. In our view, for the same joint/UL TCI state, UE should apply the newest PL offset value no matter it is indicated by RRCreconfiguration or MAC CE update.

Proposal 4: For the same joint/UL TCI state, UE should apply the newest PL offset value no matter it is indicated by RRCreconfiguration or MAC CE update.

Regarding the design detail of the MAC CE, we consider several aspects as follows.

- How to indicate a PL offset value

  - Option 1: indicate an absolute PL offset, with value range [-12, 60] dB and a step size of 4dB.

  - Option 2: indicate a delta PL offset with respect to the RRC configured value, with value range [-12, 60] dB and a step size of 4dB.

For both options, 5 bits are needed to indicate a PL offset value. For simplicity, option 1 is preferred.

Proposal 5: The PL offset update MAC CE includes a PL offset field of length 5 bits, which indicates an absolute value from -12 to 60 with step size of 4.

- How to indicate the association between a PL offset and the joint/UL TCI state the PL offset applied to?

- Option 1: a single PL offset is indicated for each joint/UL TCI state ID to be updated.

- Option 2: a single PL offset is indicated for all joint/UL TCI states IDs to be updated with the same value.

Option 1 anyway needs to be supported and should be baseline. Although option 2 can save some bits, different joint/UL TCI state IDs may or may not have same PL offset, and supporting option 2 may complicate MAC CE design. Therefore, option 1 is preferred.

Proposal 6: For each joint/UL TCI state ID in the new MAC CE, a dedicated PL offset value is indicated.

- Whether the PL offset update MAC CE is applied to a single serving cell or multiple serving cells?

For simplicity, we propose the MAC CE is applied to a single serving cell with a Serving Cell ID field.

Proposal 7: The PL offset MAC CE includes a serving cell ID.

- How to indicate BWP ID for the indicated joint/UL TCI state(s)?

Since both joint TCI states configured in DL BWP and UL TCI states configured in UL BWP are applicable, DL BWP ID should be indicated for joint TCI states and UL BWP ID should be indicated for UL TCI states. Only joint TCI state(s) or UL TCI state(s) can be indicated in the MAC CE since this is according to the RRC configuration unifiedTCI-StateType: if unifiedTCI-StateType is configured with joint for the serving cell, DL BWP ID and joint TCI states are included in the MAC CE; if unifiedTCI-StateType is configured with separate for the serving cell, UL BWP ID and UL TCI states are included in the MAC CE.

Proposal 8: The PL offset MAC CE includes a BWP ID and TCI state ID(s). If unifiedTCI-StateType is configured with joint for the serving cell, the BWP ID refers to a DL BWP and TCI state ID(s) refer to joint TCI state(s) configured for the DL BWP; if unifiedTCI-StateType is configured with separate for the serving cell, the BWP ID refers to a UL BWP and TCI state ID(s) refer to UL TCI state(s) configured for the UL BWP.

## PHR

For PHR aspect, RAN1 has made the following agreements.

RAN1#118 Agreement For the asymmetric DL sTRP/UL mTRP scenarios, support to include PL offset in the calculation of Type 1 PHR based on actual PUSCH transmission and Type 1 PHR based on reference PUSCH Agreement Study whether to support Type 3 PHR reporting in a serving cell/BWP where the UE is configured with two separate SRS CLPC adjustment states. - Continue to study whether to support including PL offset in the calculation of Type 3 PHR.

That is the PL offset is applied to Type 1 PHR calculation for actual/reference PUSCH, FFS whether/how to consider PL offset for Type 3 PHR calculation. From RAN2 perspective, one issue to be discussed is whether PHR trigger is impacted. In TS 38.321 clause 5.4.6, the key trigger of PHR is pathloss of any PL RS changes more than a threshold.

- phr-ProhibitTimer expires or has expired and the path loss has changed more than phr-Tx-PowerFactorChange dB for at least one RS used as pathloss reference for one activated Serving Cell of any MAC entity of which the active DL BWP is not dormant BWP since the last transmission of a PHR in this MAC entity when the MAC entity has UL resources for new transmission; NOTE 1: The path loss variation for one cell assessed above is between the pathloss measured at present time on the current pathloss reference and the pathloss measured at the transmission time of the last transmission of PHR on the pathloss reference in use at that time, irrespective of whether the pathloss reference has changed in between. The current pathloss reference for this purpose does not include any pathloss reference configured using pathlossReferenceRS-Pos in TS 38.331 [5].

PHR is used to inform the UL transmission power headroom, i.e., how much more transmission power UE is able to add to the power being used by current transmission. PHR should be triggered whenever UL transmission power is changed more than a threshold. As specified in TS 38.213 clause 7, the PUSCH/PUCCH/SRS transmission power are formulated as follows.

PPUSCH,b,f,ci,j,qd,l=minPCMAX,f,ciPO_PUSCH,b,f,cj+10log102μ∙MRB,b,f,cPUSCHi+αb,f,cj∙PLb,f,cqd+∆TF,b,f,ci+fb,f,ci,l   [dBm]

PPUCCH,b,f,ci,qu,qd,l=minPCMAX,f,ciPO_PUCCH,b,f,cqu+10log102μ∙MRB,b,f,cPUCCHi+PLb,f,cqd+∆F_PUCCHF+∆TF,b,f,ci+gb,f,ci,l   [dBm]

[dBm]

As we can see, UE determine UL transmission power based on pathloss of PL RS and the change of pathloss implies the change of UL transmission power, thus, PHR is triggered based on the change of pathloss of PL RS in the legacy scenario.

Differently, in asymmetric DL sTRP/UL-only mTRP scenario, for a joint/UL TCI state configured with a PL offset, the UL transmission power is determined based on the PL and the PL offset of the PL RS. Thus, the PHR should be triggered when the change of PL of a PL RS is larger than a threshold, where the PL takes into account the PL offset, and the PL RS is associated to the joint/UL TCI that is configured with the PL offset. How to apply the PL offset to the PL of PL RS, e.g., (PL-PL_offset) or (PL+PL_offset), is up to RAN1.

Proposal 9: If a joint/UL TCI state is configured with a PL offset, PHR trigger is based on the change of PL of the PL RS, where the PL takes into account the PL offset and the PL RS is associated to the joint/UL TCI that is configured with the PL offset. How to apply the PL offset to the PL of PL RS, e.g., (PL-PL_offset) or (PL+PL_offset), is up to RAN1.

Since the PL change is calculated/determined in PHY layer and transparent to MAC, the PL offset can be applied when the PL change is calculated in PHY layer, which can be left to RAN1 how to specify. Alternatively, similar to the above NOTE 1 in the MAC specification, a note can be added to explain the PL change is determined based on PL offset. We propose to discuss which option to go.

Proposal 10: For PHR trigger, if PL offset is configured to joint/UL TCI state(s) associated to a PL RS, RAN2 decide one of the following options.

- Option 1: the PL offset of a joint/UL TCI state is applied in PHY layer to determine the PL change of the PL RS associated to the joint/UL TCI state. How to apply the PL offset to the PL of PL RS, e.g., (PL-PL_offset) or (PL+PL_offset), is up to RAN1. Send LS to RAN1.

- Option 2: Add a note in MAC specification to explain the PL change is determined by taking into account the PL offset as stated in Proposal 9.

## 2TA for single-DCI mTRP

In the latest WID, 2TA for sDCI mTRP operation for asymmetric DL sTRP/UL mTRP is added as a new objective. In Rel-18, the applicable scenario for 2TA operation is mDCI mTRP, with no consideration for sDCI mTRP. In the last meeting, RAN1 has made the following agreement regarding 2TA for sDCI mTRP.

RAN1#118bis Agreement Support 2TA for the asymmetric DL sTRP/UL mTRP deployment scenarios: - Remove the restriction that coresetPoolIndex needs to be configured for the 2TA feature. - One downlink reference timing is supported and applied to both TAGs. - (FFS) Note: UE autonomous TA adjustment is only applicable to the first TAG - One single n-TimingAdvanceoffset is configured and applied to both TAGs. - Any of the TCI states can be associated with any one of the two TAGs. - The RAR carrying TA adjustment for those 2 TAGs is reused for Rel-19 2TA - The MAC CE based TA adjustment for 2 TAGs is reused for Rel-19 2TA. - Introduce the optional UE capability of “Overlapping UL transmission reduction” for Rel-19 2TA - If UE does not report this UE capability, UE does not expect two UL transmissions associated with different TAGs are overlapped. - FFS: UE does not expect that in intra-slot TDM PUSCH type-B repetition transmission, two consecutive repetitions associated with different TAGs are overlapped.

From RAN2 perspective, in the current 2TA operation, each TCI state is associated to the TA of a TAG. To extend 2TA operation to asymmetric DL sTRP/UL mTRP, we think the current 2TA operation can be applied with minor changes on RRC configuration. Rel-17 RA procedure with 2TA and behaviour on TAT/TAG management can be directly applied from RAN2 perspective.

We can confirm the RRC impacts based on RAN1 agreements that

- the restriction that coresetPoolIndex needs to be configured for the 2TA feature can be removed in relevant RRC fields (e.g., tag2);

- a single n-TimingAdvanceoffset is configured.

Proposal 11: For 2TA in asymmetric DL sTRP/UL mTRP scenario, Rel-17 2TA operation is applied with the following RRC change:

- remove the restriction that coresetPoolIndex needs to be configured for the 2TA feature in relevant RRC fields (e.g., tag2);

- a single n-TimingAdvanceoffset is configured.

## PL offset in RACH

For PRACH transmission, RAN1 has agreed the following aspects.

- Support applying PL offset on PDCCH-order PRACH towards a UL-only TRP in FR1.

- The PL offset associated with one of the indicated joint/UL TCI state for UL-only TRP in unified TCI framework is applied on the PDCCH-order PRACH transmission.

- Regarding how to indicate the PL offset for PDCCH-order PRACH, agreements are made in RAN1#118bis meeting.

RAN1#116bis Agreement Support applying PL offset on PDCCH-order PRACH towards a UL TRP in FR1. - Note: The DL reference timing determination for PDCCH-order PRACH transmission to an UL TRP is still based on the DL RS defined in current RAN4 specification - Above is subject to a separate UE capability signaling RAN1#118 Agreement For indicating a PL offset for PDCCH-order PRACH transmission at least for FR1, support Alt3: - Alt3: The PL offset associated with one of the indicated joint/UL TCI state for UL TRP in unified TCI framework is applied on the PDCCH-order PRACH transmission RAN1#118bis Agreement For indicating PL offset for PDCCH-order PRACH, introduce a new 1-bit DCI field in DCI format 1_0: - This DCI field exists when the corresponding RRC parameter (which is a new RRC used to configure the presence of this 1-bit DCI field) is enabled and at least one TCI state is configured with PL offset. - When one joint/UL TCI state is indicated in Rel-17 unified TCI, - the bit field index 0 of this field indicates that PL offset is not included in the PRACH transmission power calculation - the bit field index 1 of this field indicates that the PL offset associated with the indicated TCI state is included in the PRACH transmission power. - FFS: Whether the bit field can be used to indicate other information - FFS: When two joint/UL TCI states are indicated in Rel-18 unified TCI Agreement For indicating PL offset for PDCCH-order PRACH, when two joint/UL TCI states are indicated in Rel-18 unified TCI, down-select one from the following Alts for the 1-bit DCI field in DCI 1_0 which indicates the application of PL offset on PRACH transmission: - Alt2: - the bit field index 0 of this field indicates that the PL offset associated in the first indicated joint/UL TCI state is included in the PRACH transmission power calculation - the bit field index 1 of this field indicates that the PL offset associated in the second indicated joint/UL TCI state is included in the PRACH transmission power calculation. - FFS: Whether a restriction that only one of the indicated joint/UL TCI states can be configured with PL offset is needed. FFS: If other information can be indicated by this same 1-bit DCI field for the PDCCH-order PRACH transmission

We notice that RAN1 has only considered PDCCH-order PRACH. It is not clear whether it refers to both PDCCH-order CBRA and PDCCH-order CFRA or not. In our understanding, to obtain the TA for UL-only TRP in the 2TA operation, UE has to apply the PL offset in PDCCH-order PRACH towards the UL-only TRP so that the UL-only TRP can receive the PRACH with a proper UL power control. Since both PDCCH-order CBRA and PDCCH-order CFRA can be used to obtain TA, we assume PL offset is applied to both CBRA and CFRA.

Proposal 12: RAN2 assumes PL offset is applied to both PDCCH-order CBRA and PDCCH-order CFRA towards a UL-only TRP in FR1.

On the other hand, it is also not clear whether PL offset is needed for PRACH in UE initiated RA (e.g., for BFR, SR, etc.) or not. In our view, PL offset is not necessary for UE initiated RACH for BFR, SR, etc. since even without applying the PL offset the macro DL/UL TRP should be able to receive the PRACH with legacy power control. This is different from PDCCH-order PRACH to the UL-only TRP, which is intended to obtain TA for the UL-only TRP. Thus, we propose not to consider PL offset in UE initiated RACH.

Proposal 13: RAN2 does not consider PL offset in UE initiated RACH.

Moreover, in the current Random Access procedure UE measures RSRP of DL pathloss RS and compares it with a RSRP threshold to determine RA resource selection (e.g., for NUL/SUL, 4-step/2-step, Msg1/3 repetition) for UL coverage consideration. Normally, UE measures RSRP of DL PL RS and transmits PRACH to the same TRP. However, in asymmetric DL/UL scenario, UE can measure RSRP from a DL TRP and transmits PRACH to a UL-only TRP. Since the measured RSRP of a DL TRP and the actual RSRP of UL-only TRP to be used for RSRP comparison can be different due to the PL offset, the pathloss offset between the two TRPs may have an impact on the RSRP comparison for RA resource selection. We propose to discuss whether the PL offset has an impact on the RSRP comparison for RA resource selection.

Proposal 14: Discuss whether the PL offset has an impact on the RSRP comparison for RA resource selection.

# Conclusion

Based on the discussion, we have the following proposals.

Observation 1: The PL offset is applied to all UL transmission on PUCCH, PUSCH, SRS, and FR1 PDCCH order PRACH, for which UE can use DCI-indicated or MAC CE indicated, or RRC configured joint/UL TCI state(s) in different cases.

Proposal 1: PL offset is configured per joint/UL TCI state in RRC, with enumerated values from -12dB to 60dB with step size 4dB.

Proposal 2: PL offset update by MAC CE is applicable to all joint/UL TCI state configured with PL offset.

Proposal 3: Introduce a new MAC CE for PL offset update, namely, PL Offset Update MAC CE, which is identified by an eLCID.

Proposal 4: For the same joint/UL TCI state, UE should apply the newest PL offset value no matter it is indicated by RRCreconfiguration or MAC CE update.

Proposal 5: The PL offset update MAC CE includes a PL offset field of length 5 bits, which indicates an absolute value from -12 to 60 with step size of 4.

Proposal 6: For each joint/UL TCI state ID in the new MAC CE, a dedicated PL offset value is indicated.

Proposal 7: The PL offset MAC CE includes a serving cell ID.

Proposal 8: The PL offset MAC CE includes a BWP ID and TCI state ID(s). If unifiedTCI-StateType is configured with joint for the serving cell, the BWP ID refers to a DL BWP and TCI state ID(s) refer to joint TCI state(s) configured for the DL BWP; if unifiedTCI-StateType is configured with separate for the serving cell, the BWP ID refers to a UL BWP and TCI state ID(s) refer to UL TCI state(s) configured for the UL BWP.

Proposal 9: If a joint/UL TCI state is configured with a PL offset, PHR trigger is based on the change of PL of the PL RS, where the PL takes into account the PL offset and the PL RS is associated to the joint/UL TCI that is configured with the PL offset. How to apply the PL offset to the PL of PL RS, e.g., (PL-PL_offset) or (PL+PL_offset), is up to RAN1.

Proposal 10: For PHR trigger, if PL offset is configured to joint/UL TCI state(s) associated to a PL RS, RAN2 decide one of the following options.

- Option 1: the PL offset of a joint/UL TCI state is applied in PHY layer to determine the PL change of the PL RS associated to the joint/UL TCI state. How to apply the PL offset to the PL of PL RS, e.g., (PL-PL_offset) or (PL+PL_offset), is up to RAN1. Send LS to RAN1.

- Option 2: Add a note in MAC specification to explain the PL change is determined by taking into account the PL offset as stated in Proposal 9.

Proposal 11: For 2TA in asymmetric DL sTRP/UL mTRP scenario, Rel-17 2TA operation is applied with the following RRC change:

- remove the restriction that coresetPoolIndex needs to be configured for the 2TA feature in relevant RRC fields (e.g., tag2);

- a single n-TimingAdvanceoffset is configured.

Proposal 12: RAN2 assumes PL offset is applied to both PDCCH-order CBRA and PDCCH-order CFRA towards a UL-only TRP in FR1.

Proposal 13: RAN2 does not consider PL offset in UE initiated RACH.

Proposal 14: Discuss whether the PL offset has an impact on the RSRP comparison for RA resource selection.

# References

- RP‑242356	Revised Work Item: NR mobility enhancements Phase 4, RAN#105.