3GPP TSG-RAN WG2 Meeting #128	R2-2410525

Orlando, USA, Nov 18th – 22nd, 2024

Agenda item:	8.8.2

Source:	Samsung

Title:	Discussion on Downlink Coverage Enhancement

WID/SID:	NR_NTN_Ph3-Core

Document for:	Discussion and Decision

# Introduction

For Rel-19 NR NTN, SSB periodicity extension has been included as the objective of DL coverage enhancement in the latest WID [1].

Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - RAN1 to report at the latest by RAN#106 with the list of targeted physical channels/signals for link level enhancements (if any), and with the targeted system-level enhancements (if any) - RAN1 should report on impact to backward compatibility, if any, for potential extension of the SSB periodicity at the latest by RAN#106, in conjunction with the targeted system-level enhancements. - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Rel-18 network energy saving techniques should be considered as baseline in the system level study

In the last meeting, RAN1 has agreed to support SSB periodicity 160ms in initial access.

Agreement For NR NTN, support extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is at least 160 ms. - FFS: whether 320ms can be supported as the maximum of the additional default value instead of or in addition to 160ms

In this contribution, we share our views on DL coverage based on the last meeting’s progress.

# Discussion

In the last meeting, we have discussed whether/how to prevent UEs not supporting DL CE from reselecting cells operating with DL CE.

According to the latest WID [1], RAN1 will report on impact to backward compatibility, if any, for potential extension of the SSB periodicity. Based on the analysis of the impact to legacy UE (including Rel-19 UE not supporting such a feature), we can further discuss whether there is a need to prevent legacy UE from reselecting cells operating with DL CE. In our view, strict prevention by NW configuration may not be necessary if the initial access in cell reselection is not impacted too much from NW perspective. NW can provide SSB periodicity information of NTN neighbor cells (e.g., by SMTC or in SIB), and UE can select cells properly up to implementation, e.g., with or without (de)prioritization of cells operating with DL CE.

Observation 1: NW can provide SSB periodicity information of NTN neighbor cells (e.g., by SMTC or in SIB), so that UE can select cells properly up to implementation, e.g., with or without (de)prioritization of cells operating with DL CE.

Proposal 1: Regarding whether/how to prevent UEs not supporting DL CE from reselecting cells operating with DL CE, wait for more RAN1 analysis on backward compatibility.

Based on the WID, Rel-19 DL coverage enhancement considers the scenario of beam hopping where only a limited number of satellite beams are illuminated at a certain time due to power sharing. RAN1 has replied in LS R2-2407919 that the beam footprint status in terms of "off", "common messages only" and "active traffic” in one of the RAN1 agreements is only defined for the sake of system-level evaluation methodology [2]. In our understanding, this implies that the beam footprint status in terms of "off", "common messages only" and "active traffic” are visible to UE.

By extending the SSB periodicity, SSB beams in status “off” can happen in the duration between two consecutive SSB bursts, i.e., the duration of 160ms when the SSB bursts (5ms) are not transmitted. This allows NW to transmit SSB bursts in turn for more cells so that a larger area can be illuminated by one satellite. Beam hopping with SSB periodicity of 160ms should not impact idle/inactive UE. As RAN1 indicated, they will have further reply on this aspect.

In terms of UE specific beam for “active traffic”, the current TCI state framework can allow different beams to be used for different UEs at different time. For a specific UE, the beam identified by a TCI state is used to transmit and received DL/UL channels and reference signals, which can be dynamically indicated by NW (e.g., by activation/deactivation of a list of TCI states in MAC CE and DCI dynamic indication of one TCI state from the activated ones).

Take PDCCH as an example. For PDCCH reception, a list of TCI states is configured in ControlResourceSet, and one TCI state from the list can be dynamically indicated to be used for PDCCH reception. By utilizing the existing DRX and beam management mechanism, NW can configure different TCI state lists for different UEs according to the beam hopping pattern and each UE monitors PDCCH in DRX active time using the configured TCI state list, so that the PDCCH monitoring pattern can be aligned with beam hopping pattern. An example is illustrated in the Figure below, where a satellite illuminates different areas in turn and for each illuminated area at a given time NW configures the corresponding DRX pattern and the corresponding TCI state list for UE to identify beams used for PDCCH reception.

Observation 2: By utilizing the existing DRX and beam management mechanism, PDCCH monitoring pattern by NW configuration can be aligned with beam hopping pattern.

Proposal 2: Wait for more RAN1 reply on cell/beam-level DTX/DTX for DL coverage enhancement.

# Conclusion

In this contribution we have the following proposals.

Observation 1: NW can provide SSB periodicity information of NTN neighbor cells (e.g., by SMTC or in SIB), so that UE can select cells properly up to implementation, e.g., with or without (de)prioritization of cells operating with DL CE.

Proposal 1: Regarding whether/how to prevent UEs not supporting DL CE from reselecting cells operating with DL CE, wait for more RAN1 analysis on backward compatibility.

Observation 2: By utilizing the existing DRX and beam management mechanism, PDCCH monitoring pattern by NW configuration can be aligned with beam hopping pattern.

Proposal 2: Wait for more RAN1 reply on cell/beam-level DTX/DTX for DL coverage enhancement.

# Reference

RP-241667 Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3

R2-2407919 Reply LS on DL coverage enhancements