3GPP TSG-RAN WG2 Meeting #128	R2-2410526

Orlando, USA, Nov 18th – 22nd, 2024

Agenda item:	8.8.4

Source:	Samsung

Title:	Discussion on MBS Broadcast Intended Area

WID/SID:	NR_NTN_Ph3-Core

Document for:	Discussion and Decision

# Introduction

For Rel-19 NTN, one objective is to support MBS broadcast service for smaller areas in an NTN cell. The detailed justification and objective in WID are provided as follows.

MBS feature provides an important add-value for NR NTN system, leveraging the large coverage of the NTN compared to TN. Terrestrial MBS features are equally available for NR NTN in the 5G specifications, but for some cases the intended service area is expected to be smaller than the coverage of a Uu cell, some enhancements need to be done to notify the service area of a Broadcast service. Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3] - Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary signaling between CN and NG-RAN. [RAN3]

In this contribution, we continue to share our views on the signaling of broadcast service area information and discuss service continuity with intended service area information.

# Discussion

## Signalling of intended service area

In the last meeting, we made some progress on intended service area signaling aspect.

For each MBS service we include one or more intended service area IDs into MCCH. FFS whether the list of the intended service areas (and related IDs) is also included in MCCH or if it is provided in a new or existing SIB. We will consider possible enhancements (including enhancements left up to UE implementation) to allow UE skipping MCCH re-acquisition when UE is not within intended service area of any interested broadcast service.

To signal the list of intended service areas, there are two options to be downselected: 1) provide in a new or existing SIB; 2) provide in MCCH. Our views on the pros and cons of the two options are as follows.

Option 1 is aligned with WID “specify SIB signaling”. It allows UE to skip acquiring MCCH if UE is not in any of the intended service areas. The existing SIB, e.g., SIB20, may have overhead restriction. Instead, we can define a new SIB to provide the geographic area information for the intended service areas and the related IDs. In this way, legacy UEs are not impacted. As the geographic area information is semi-static, frequent SIB update is not expected.

Observation 1: The geographic area information for the intended service areas is assumed to be semi-static which does not cause frequent SIB update.

Option 2 is cleaner in terms of providing all information in MBS broadcast configuration in MCCH. Overhead restriction is not an issue. However, UE may acquire MBS broadcast configuration in MCCH in vain if UE is not in any intended service areas of its interested services.

In our view, we would prefer to follow the WID to specify signaling in a new SIB as option 1. One aspect is that in the SIB every broadcast service session needs to be associated to a service area so that UE can be aware of the intended area of each service by acquiring SIB only without acquiring MCCH. We have agreed circles and polygons are used to describe the intended areas. This is needed only in the case that the intended service area of a service covers a part of the cell, not the whole cell.

Proposal 1: Introduce a new SIB to include a list of intended service areas and related IDs.

Proposal 2: The legacy SIB modification procedure is applied to update the intended service area information in the new SIB.

RAN3 has send a LS R2-2409519 regarding the intended service area information from CN to gNB.

RAN3 has discussed the NGAP impact to support NTN MBS in Rel-19. More specifically, how the AMF indicates to the gNB when the intended service area is smaller than the satellite footprint. RAN3 agreed that the intended service area provided to the gNB can be Mapped Cell ID(s), TAI(s), or geographical service area information. The name and encoding of the geographical service area information will align with RAN2 decision. RAN3 has started to work on the appropriate signalling solution (see attached agreed TPs – IE name and encoding are pending RAN2 discussion). RAN3 considers this may impact SA2 and CT4.

As RAN3 informed, the intended service area can be in a form of cell ID or TAI as legacy, or in a form of geographic area. If a service is provided for the whole cell, it can be simply indicated by a flag (e.g., cell ID) without geographic area information. UE can skip MCCH re-acquisition when UE is not within any intended service area indicated in the SIB.

Proposal 3: Circles and polygons are used to describe an intended area only in the case that the intended service area does not cover a whole cell. If any service is provided for the whole cell, the cell ID can be indicated and associated to the intended service area ID.

RAN2 has agreed that both (quasi)-earth-fixed and earth-moving cells are supported for MBS broadcast service. We understand the intended service area represents a fixed geographic area on the ground for both cases. However, due to the fast movement of an earth moving cell, the cell may provide an MBS broadcast service to an intended area for a quite short duration, which will lead to frequent updates of MBS broadcast service and the associate intended area and, thus, UE frequent acquisitions of SIB or MCCH. To avoid this issue, some additional information (e.g., duration, etc.) can be provided for an MBS broadcast service and its intended area for earth-moving cells. This is different from unicast service which is UE specific and does not need to provide intended service area.

Proposal 4: Discuss for earth moving cell how to avoid NW frequent update and UE frequent acquisition of information on MBS broadcast service and the intended area.

## Service continuity

For MBS broadcast service, SIB21 contains information related to service continuity for MBS broadcast reception. As specified in stage-2, to allow the UE to start or continue receiving MBS service(s) when changing cells, the gNB may indicate in the MCCH the list of neighbour cells providing the same MBS broadcast service(s) as provided in the serving cell. This allows the UE, e.g., to request unicast reception of the service before moving to a cell not providing the MBS broadcast service(s) using PTM transmission. To avoid the need to read MBS broadcast related system information and potentially MCCH on neighbour frequencies, the UE is made aware of which frequency is providing which MBS broadcast services via PTM, through User Service Description, as defined in TS 26.517, or the combination of User Service Description (USD) and Frequency Selection Area Identities (FSAI) in SIB21.

For an MBS broadcast service, the intended service area provided by a cell may cover a continuous area of the current serving cell and neighbor cell(s), e.g., cell 1 provides the whole green area shown in the Figure below. It is also possible that the intended service area provided by a cell covers only geographic area within the current serving cell, e.g., cell 1 provides only the part of the green area that falls into cell 1.

Cell 1

Proposal 5: Discuss how an intended service area is provided.

Option 1: An intended service area for a MBS BC service includes all geographic areas across the current serving cell and neighbor cell(s), and NW ensures the current serving and all neighbor cells that provide the same service signal the same intended service area information.

Option 2: For a MBS BC service, the current serving cell provides the intended service area only within the current cell.

Option 3: Up to NW implementation, i.e., both option 1 and 2 are possible.

In case of option 1, UE can know the intended service area of a MBS BC service from the current serving cell, UE can also know the list of neighbor cells that provides the same service based on the MBS-NeighbourCellList associated to this MBS BS service in MBSBroadcastConfiguration, and UE can know as well the frequency that provides the same service based on the frequency list in SIB21. When UE performs cell reselection, UE can prioritize the frequency that provides the same service and/or select a neighbor cell that provides the same service.

In case of option 2 or 3, to support service continuity across cells, the intended service area for neighbor cells can be provided by the current serving cell. One approach is to support the mapping between intended service areas and neighbor cells in the MBS-NeighbourCellList in MBSBroadcastConfiguration. Another approach is to support the mapping between intended service areas and the frequency list in SIB21. Based on the intended service area associated to a frequency or a neighbor cell, UE can prioritize or deprioritize a frequency in cell reselection.

Proposal 6: For option 2 and 3, consider to support service continuity by signaling the mapping between intended service area and frequency/neighbor cell.

# Conclusion

In this contribution we discussed the following proposals.

Observation 1: The geographic area information for the intended service areas is assumed to be semi-static which does not cause frequent SIB update.

Proposal 1: Introduce a new SIB to include a list of intended service areas and related IDs.

Proposal 2: The legacy SIB modification procedure is applied to update the intended service area information in the new SIB.

Proposal 3: Circles and polygons are used to describe an intended area only in the case that the intended service area does not cover a whole cell. If any service is provided for the whole cell, the cell ID can be indicated and associated to the intended service area ID.

Proposal 4: Discuss for earth moving cell how to avoid NW frequent update and UE frequent acquisition of information on MBS broadcast service and the intended area.

Proposal 5: Discuss how an intended service area is provided.

Option 1: An intended service area for a MBS BC service includes all geographic areas across the current serving cell and neighbor cell(s), and NW ensures the current serving and all neighbor cells that provide the same service signal the same intended service area information.

Option 2: For a MBS BC service, the current serving cell provides the intended service area only within the current cell.

Option 3: Up to NW implementation, i.e., both option 1 and 2 are possible.

Proposal 6: For option 2 and 3, consider to support service continuity by signaling the mapping between intended service area and frequency/neighbor cell.

# Reference

RP-240775, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3

3GPP, TS 38.331 v18.0.0; Radio Resource Control (RRC); Protocol specification.

R2-2409519, LS on Supporting MBS broadcast service for NR NTN