3GPP TSG-RAN WG2 #128                                             	    R2-2410530

Orlando, FL, USA, November 18 – 22 , 2024

Agenda item:	   8.6.2

Source: 		Qualcomm Incorporated

Title: 	Security handling for Inter-CU LTM

Document for:	Discussion

# 1	Introduction

RAN2 received a response LS (S3-244316) from SA3 on the questions posed by RAN2 on the security handling for inter-CU LTM.

SA WG3 has been considering the security mechanisms proposed for the inter-CU LTM feature. For this feature, SA WG3 want to preserve the principle that when there is a PDCP anchor change, then it is necessary to perform a key change (i.e. for solutions or handovers that do not require a PDCP anchor change, there is no need to provide rekeying).

The rekeying method preferred by most companies in SA WG3 is that after every inter-CU LTM cell switch execution that changes the PDCP anchor, the UE is provided with the NCC value via RRC signalling to be used by the UE for key derivation at the next inter-CU LTM cell switch. This solution was preferred as it provides the NCC value using the protected RRC signalling and hence is considered to be secure. SA WG3 understands that, RAN WG2 is aiming to avoid RRC configuration/signalling between cell switches for subsequent LTM procedures. If it is necessary for the RAN WGs to avoid RRC signalling between cell switches, then SA WG3 can analyse further methods like,

- Security threat of the unprotected MAC CE to deliver the NCC value.

- Sending NCC value per candidate cell via RRC message during LTM preparation phase.

SA WG3 will keep RAN WG2 and RAN WG3 informed as we make further progress on this topic.

In this contribution, we discuss the SA3 response and possible way forward in RAN2 on this topic.

# 2	Discussion

RAN2#123bis has sent an LS to SA3 asking for feedback on the proposed solutions in RAN2 for handling the security keys during inter-CU LTM. SA3 has agreed on a Work Item to investigate the general security aspects for LTM ([2]) and has sent an LS response in [3].

In the response LS, it is stated that the solution preferred by most companies in SA3 is to provide the NCC value via RRC signaling. One justification for this choice was the security protection provided by RRC signaling.

Observation 1: SA3 prefers that RRC signaling is used to provide the NCC value at or after after the MAC-CE based cell switch command for inter-CU LTM.

The LS mentions that SA3 can analyze other options (which were also mentioned in the RAN2 LS) such as using MAC CE or sending NCC value during LTM preparation phase.

Observation 2: Per LS response, delivery of NCC via MAC CE or in advance via RRC will require further analysis in SA3.

It is obvious that the transfer of an RRC message and RRC reconfiguration during every inter-CU LTM switch will incur significant overhead and delays to the subsequent inter-gNB LTM. This goes against the main premise of LTM which relies on L1/L2 signaling by avoiding L3 messages during LTM execution and during subsequent LTM procedures. This was also captured in the WID ([1]) objective as follows:

Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM

Observation 3: The Rel-19 Mobility WID states that the RRC configuration between cell switches should be avoided.

If RRC signaling is used for LTM during every inter-gNB cell switch, LTM will not provide any gains over L3 handover in terms of signaling efficiency and latency benefits.

Observation 4: Using RRC signaling and reconfiguration for every inter-CU LTM switch will incur the same latency and signalling overhead as L3 HO.

At this point, RAN2 can go in two directions:

Option 1: Use RRC signaling for delivery of NCC but work on options to eliminate such signaling at every subsequent inter-CU LTM HO

Option 2: Request SA3 to consider the other options, e.g. using MAC CE

Option 2 may not be feasible to complete in Rel-19 and MAC-CE is not secure to provide NCC(as commented by SA3 in the LS as well). If MAC-CE needs to be protected, additional work will be needed and this is not within the WID scopre. It took SA3 three meetings to provide the above LS response. A work in SA3 on these additional options will likely take even more time. Rel-19 should finish in RAN2 by Q3 2025 which means that RAN2 will have four meetings in 2025. SA3 will have three meetings in 2025 before the last meeting of RAN2 for Rel-19 in August 2025. Therefore, it is unlikely that RAN2 and SA3 will have sufficient time to agree on another option for security handling.

Observation 5: Rel-19 timeline and the pace of progress in SA3 so far makes it very difficult to work on another option other than RRC signaling.

Based on this, we propose to use the RRC signaling as a baseline and work on methods to eliminate delivery of NCC at every inter-CU LTM.

Proposal 1: RAN2 agree to use RRC signaling for the delivery of NCC for inter-CU LTM

Proposal 2: RAN2 agree to support methods to eliminate delivery of NCC for every inter-CU LTM (and without any AMF impact).

One option which can make Proposal 2 feasible is the solution discussed in R2-2408966 (4) which relies on not changing the CU anchor at every LTM switch. Even though the discussion on this option has been suspended due to the pending SA3 response, RAN3 has had several rounds of discussion without any progress. As discussed in the paper, the solution relies on mostly backhaul Xn signaling and can be done transparently over Uu.

Observation 6: RAN3 has discussed the solution in [4] of keeping the CU anchor unchanged during LTM for several meetings.

Observation 7: The solution in [4] is transparent to Uu signaling and relies on enhancements to backhaul signaling.

It is important to note that a NW implementation can send NCC via RRC whenever it chooses. However, as discussed above, this will not be desirable for a deployment to do this at every inter-CU LTM.

Observation 8: It will be up to NW implementation when to send NCC via RRC for LTM.

Given that the solution in [4] or variations of it can be done in RAN3 with minimal RAN2 involvement, RAN2 can agree to a high level solution and request RAN3 to investigate and specify the necessary signaling.

Proposal 3: RAN2 request RAN3 to specify solutions for inter-CU LTM which can keep the current CU for the UE unchanged during LTM to cells belonging to other CUs.

RAN2 can also inform SA3 on the above agreements. If this is used as the way-forward, no further work in SA3 is needed.

Proposal 4: RAN2 to respond to SA3 on using RRC for delivery of NCC (and thus no further SA3 work being necessary).

# 3	Conclusion

In this contribution, we discussed the SA3 response LS on the security handling for inter-CU LTM and propose the following:

Observation 1: SA3 prefers that RRC signaling is used to provide the NCC value at or after after the MAC-CE based cell switch command for inter-CU LTM.

Observation 2: Per SA3 LS response, delivery of NCC via MAC CE or in advance via RRC will require further analysis in SA3.

Observation 3: The Rel-19 Mobility WID states that the RRC configuration between cell switches should be avoided.

Observation 4: Using RRC signaling and reconfiguration for every inter-CU LTM switch will incur the same latency and signalling overhead as L3 HO.

Observation 5: Rel-19 timeline and the pace of progress in SA3 so far makes it very difficult to work on another option other than RRC signaling.

Proposal 1: RAN2 agree to use RRC signaling for the delivery of NCC for inter-CU LTM

Proposal 2: RAN2 agree to support methods to eliminate delivery of NCC for every inter-CU LTM (and without any AMF impact).

Observation 6: RAN3 has discussed the solution in [4] of keeping the CU anchor unchanged during LTM for several meetings.

Observation 7: The solution in [4] is transparent to Uu signaling and relies on enhancements to backhaul signaling.

Observation 8: It will be up to NW implementation when to send NCC via RRC for LTM.

Proposal 3: RAN2 request RAN3 to specify solutions for inter-CU LTM which can keep the current CU for the UE unchanged during LTM to cells belonging to other CUs.

Proposal 4: RAN2 to respond to SA3 on using RRC for delivery of NCC (and thus no further SA3 work being necessary).

# References

[1] RP-241515	Revised Work Item: NR mobility enhancements Phase 4, Apple, China Telecom

[2] S3-242401, New WID on security aspects of NR mobility enhancement, Samsung, Oppo

[3] S3-244316, Reply LS on security handling for inter-CU LTM in non-DC cases, Samsung

[4] R2-2408966, Inter-gNB LTM, Qualcomm Incorporated