3GPP TSG-RAN WG2 Meeting #128	R2-2410531

# Orlando, USA, 14-18 November 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 2007 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Cell DRX MPS exemption in RA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Peraton Labs, CISA ECD, AT&T, Verizon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR-Core |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 3GPP TS 38.300 provides exemptions for emergency calls and public safety related services like MPS to ensure that these services are not impacted by services like Cell DTX/DRX, as follows: “Once the gNB recognizes there is an emergency call or public safety related service, the network should ensure that there is no impact to that service (e.g. it may release or deactivate cell DTX/DRX configuration).” 3GPP TS 38.321 already supports exemptions from cell DRX for emergency service in clause 5.34.3, as follows: 1> if cell DRX is activated and the Serving Cell is not in the cell DRX Active Period: 2> if an emergency service is initiated by upper layers and this Serving Cell is the SpCell: 3> initiate a Random Access procedure (as specified in clause 5.1.1). No such exemptions from Cell DRX are supported for MPS in 3GPP 38.321. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In clause 5.34.3, add a new exemption for MPS when upper layers provide Access Identity 1 (MPS indication) to the MAC layer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Continued misalignment |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.34.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

### 5.34.3	Cell Discontinuous Reception

Cell DRX is configured if cellDTX-DRX-ConfigType is set to drx or dtxdrx. Cell DRX operation is activated and deactivated for each Serving Cell by:

-	receiving a cell DRX indication from lower layers indicating activation or deactivation of cell DRX operation, as specified in TS 38.213 [6];

-	configuring cellDTX-DRX-Config by upper layers: if cell DRX is configured and cellDTX-DRX-ActivationStatus is set to activated, cell DRX operation is activated upon cell DRX configuration; if cell DRX is configured and cellDTX-DRX-ActivationStatus is set to deactivated, cell DRX operation is deactivated upon cell DRX configuration; if cellDTX-DRX-Config is released, cell DRX operation is deactivated and all the corresponding configurations are released.

When cell DRX is configured and activated for a Serving Cell, the cell DRX Active Period includes the time while:

-	cellDTX-DRX-onDurationTimer is running for the associated Serving Cell.

For each Serving Cell configured with cell DRX, the MAC entity shall:

1>	if cell DRX is activated for this Serving Cell:

2>	if [(SFN × 10) + subframe number] modulo (cellDTX-DRX-Cycle) = (cellDTX-DRX-StartOffset):

3>	start cellDTX-DRX-onDurationTimer for this serving cell after cellDTX-DRX-SlotOffset from the beginning of the subframe.

NOTE 1:	In case of unaligned SFN across carriers in a cell group, the SFN of the SpCell is used to calculate the cell DRX duration.

1>	if cell DRX is activated and the Serving Cell is not in the cell DRX Active Period:

2>	not instruct the physical layer to signal a SR on a PUCCH resource for SR;

2>	not increment the SR_COUNTER for a SR;

2>	not start the sr-ProhibitTimer for a SR;

2>	not deliver any configured uplink grant and the associated HARQ information to the HARQ entity;

2>	not instruct a HARQ process associated with a configured uplink grant to trigger a new transmission or a retransmission;

2>	not report CSI on PUCCH and semi-persistent CSI configured on PUSCH;

2>	if an emergency service is initiated by upper layers and this Serving Cell is the SpCell:

3>	initiate a Random Access procedure (as specified in clause 5.1.1).

NOTE 2:	How the MAC layer in the UE is aware of an ongoing emergency service is up to UE implementation.

2>	if upper layers provide Access Identity 1 and this Serving Cell is the SpCell:

3>	initiate a Random Access procedure (as specified in clause 5.1.1).

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>