3GPP TSG-RAN WG2 #128	R2-2410543

# Orlando, US, 18th – 22nd November 2024

Agenda Item:	7.0.2.9

Source:	Ericsson

Title:	Discussion on RAN1 LS on time period

Document for:	Discussion

# 1	Introduction

An LS from RAN1 was received, [1] which discusses different interpretations of the “time period” as defined in RAN1 specifications 3GPP TS 38.213 and in relation to this base their discussion on two cases:

Case#1: Repetition number n2 and n4 are configured in one RACH-ConfigCommon, while n8 is configured in another RACH-ConfigCommon. All these repetition numbers are associated to one feature combination(i.e. msg1-repetition-r18). Figure 1 Case#2: Only single RACH-ConfigCommon is configured with multiple feature combinations. As shown in Figure 2, repetition number n2 and n4 are associated to one feature combination(i.e. msg1-repetition-r18), n8 is associated to another feature combination(i.e. msg1-repetition-r18+redCap-r17). Figure 2 In the two cases, RAN1 specification ambiguity is identified for the text “all configured number of preamble repetitions” of the concerned time period. For example, the text can be interpreted in the case#2 as Interpretation 1-2: the time period is determined per feature combination. That is, one time period for feature msg1-repetition-r18 is determined based on n2 and n4. Another time period for the feature combination msg1-repetition-r18+redCap-r17 is determined based on only n8. Interpretation 2-2: the time period is determined considering all feature combinations. That is, only one time period is determined based on n2, n4 and n8. Interpretation 3-2: the time period is determined per UE. That is, one time period for UEs capable of feature msg1-repetition-r18 but incapable of redCap-r17 is determined based on n2 and n4. The second time period for UEs capable of both the feature msg1-repetition-r18 and redCap-r17 is determined based on n2, n4 and n8.

This contribution proposes our view on the interpretation of these cases and the action in the LS.

# 2	Discussion

RAN1 states the following action to RAN2:

2. Actions: To RAN2 ACTION: RAN1 would like to respectfully ask RAN2 to provide answers to the above questions and consider potential clarification in RAN2 specification, if any.

To advance in the discussion, the question in the LS is discussed one by one.

Q0: Has the case#1 above been precluded by the existing RAN2 signaling and procedures? If yes, please share the relevant RAN2 specification.

No, case#1 has been discussed and agreed during RAN2#123bis, and later reflected in 3GPP TS 38.331 implicitly since IE featureCombinationPreamblesList-r17 is present in RACH-ConfigCommon, and RACH-ConfigCommon is present in AdditionalRACH-Config-r17.

Q1: Has the case#2 above been precluded by the existing RAN2 signaling and procedures? If yes, please share the relevant RAN2 specification.

No, case#2 is supported for the same reasons as the answer to Q0.

Q2: Has the following case#1-rev (a combination of the case#1 and case#2) been precluded by the existing RAN2 signaling and procedures? If yes, please share the relevant RAN2 specification. Case#1-rev: the same as the case#1 except that the feature combination msg1-repetition-r18 in the second RACH-ConfigCommon is replaced with msg1-repetition-r18+redCap-r17.

No, see answers to Q0 and Q2.

Q3: If the answer to Q1 is “not precluded”, according to the following specification excerpt of TS 38.331, a UE capable of msg1-repetition but incapable of RedCap ignores the RACH resource indicated by the third FeatureCombinationPreambles in the case#2. In addition to the ignored RACH resource, does it mean that the corresponding parameter repetition number n8 is also ignored, i.e. considered as “not configured” to the UE? featureCombination Indicates which combination of features that the preambles indicated by this IE are associated with. The UE ignores a RACH resource defined by this FeatureCombinationPreambles if any feature within the featureCombination is not supported by the UE or if any of the spare fields within the featureCombination is set to true..

The selection procedure for RACH resources in MAC (TS 38.321 5.1.1b) states the following:

Given this formulation above highlighted in orange, and since BWP-UplinkCommon contains both RACH-ConfigCommon and additionalRACH-ConfigList, if the current random access procedure allows for all repetitions, it could be argued that also the preamble partition containing msg1-RepetitionNum = n8 should be considered for initializing parameters. However, if the UE does not support RedCap, the complete set of random access resources as well as the parameter will be ignored, during the evaluation of the resources in 5.1.1b-c. The answer to Q3 is Yes.

Q4: If the answer to Q1 is “not precluded”, according to the following RAN2 agreement, a UE capable of RedCap must select the RACH resource indicated by the third FeatureCombinationPreambles in the case#2. For the RedCap UE, does it mean that the corresponding parameter repetition number n2 and n4 indicated by the first and second FeatureCombinationPreambles in the case#2 are considered as “not configured” to the UE? RAN2#116bis Proposal 1’: [Easy] In MAC perspective, a RedCap UE uses Msg1 early identification whenever transmitting preamble for CBRA, as long as the Msg1 early identification is configured for RedCap by NW. Agreed.

Yes, the UE will select only the preamble partition that indicates supported features according to the reasoning in answer to Q3.

Q5: If the answer to Q1 is “not precluded”, in a case where a UE selects a set of PRACH resources according to featurePriorities and S5.1.1d of TS 38.321, whether or not only parameter repetition numbers indicated for the sets of PRACH resources selected by the UE are considered as “configured” to the UE while all parameter repetition numbers indicated for all the other sets of PRACH resources as “not configured”?

Yes, in 5.1.1 (3GPP TS 38.321) parameters are initialized for a set of random access resources selected by 5.1.1b), they should hence be considered as “not configured” if they do not belong to the correct preamble partition/set of RACH resources.

RAN2 specifications allows for multiple preamble partitions on both shared and/or separate RO, i.e. on RACH-ConfigCommon and AdditionalRACH-Config.

There is nothing that needs to be clarified in RAN2 specifications based on the posted questions in the LS if this is intended behavior from a RAN1 perspective.

If the intended behavior is not according to RAN1 intention regarding aspects as “time period” etc. which is RAN1 specific, RAN2 updates might be needed.

However, in this case it should be highlighted that the specific question on which time period to apply is a RAN1 question, and the corresponding RAN2 impact is only related to signaling. Because of this it would be unwise if RAN2 would try to impact which interpretation to use, and the reply-LS should include answers to the questions, as well as emphasizing that RAN2 can steer the wanted behavior according to how RAN1 would like the behavior to be.

RAN2 replies to the LS with answers to the stated questions as well as a statement that no RAN2 specification clarifications seems necessary, unless RAN1 explicitly would like to change any of the described behavior.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	RAN2 specifications allows for multiple preamble partitions on both shared and/or separate RO, i.e. on RACH-ConfigCommon and AdditionalRACH-Config.

Observation 2	There is nothing that needs to be clarified in RAN2 specifications based on the posted questions in the LS if this is intended behavior from a RAN1 perspective.

Observation 3	If the intended behavior is not according to RAN1 intention regarding aspects as “time period” etc. which is RAN1 specific, RAN2 updates might be needed.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 replies to the LS with answers to the stated questions as well as a statement that no RAN2 specification clarifications seems necessary, unless RAN1 explicitly would like to change any of the described behavior.

# 4	References

- R1-2409262, LS on the time period for R18 preamble repetition, Hefei, China, October 14th – 18th, 2024.