# 3GPP TSG-RAN2 Meeting #128	R2-2410552

# Orlando, USA, Nov. 18-22, 2024

Agenda Item:	8.2.5

Study Item:	FS_Ambient_IoT_Solutions

Source: 	Qualcomm Incorporated

Title: 	Further aspects of Topology 2

Document for:	Discussion/Decision

# Introduction

RAN#103 approved SI for Ambient IoT in [1]. TR 38.848 [2] refers to the two topologies under consideration as shown below.

Figure 1: Topology 1					Figure 2: Topology 2

In Topology 1, the Ambient IoT device directly and bidirectionally communicates with a base station. In Topology 2, the Ambient IoT device communicates bidirectionally with an intermediate node between the device and base station. In this topology, typically the intermediate node would be a UE capable of functioning as Ambient IoT Reader. The Reader transfers Ambient IoT data and/or signalling between the base station and the Ambient IoT device.

RAN2#127 discussed the Topology 2 aspects and agreed the following:

Agreements

- RAN2 is responsible for the interface between intermediate node (i.e. Reader) and RAN for topology 2.

- A-IoT air interface  in topology 1 between A-IoT device and reader is fully reused in topology 2, i.e. topology is transparent to the A-IoT device and there is no impact on A-IoT device.

- RAN2 assumes that intermediate UE authorization is performed by upper layers.  RAN2 impact can be studied after further SA/RAN3 progress.

- RAN2 will study the impacts to RAN2 based on the SA2 identified architecture options (i.e. no new AS layer architecture options will be studied)

- NW is in control of resources to be used by AIoT air interface.

- Will support in-coverage and study cases for reader “temporarily” out of connection (e.g. RLF, HO).  May consider extendibility to future “temporarily” out of coverage case with full NW control of resources (if possible).

Further, in RAN2#127bis, following was agreed:

Agreements

- Dedicated resource configuration is only given to the UE reader via dedicated signalling.  Mechanisms for shared resource pool amongst readers are not considered in this release.

- The UE may perform the AIoT procedure on AIoT interface between the reader and the device only if the resource configuration is valid in the cell under network control.    FFS how valid resource is determined – we will consider temporary out of connection scenarios.    FFS on resource validity across multiple cells.

- The AIoT radio resource can be (re)configured during RRC reconfiguration (including during HO procedure, after re-establishment, etc).

This document discusses remaining aspects about topology 2 for Ambient IoT.

# Discussion

## Validity of resource configuration for the Reader

Note that, in the following discussion, resource allocation and resource configuration refers to the allocation/configuration of the radio resources for the UE/Reader that can be used in R2D/D2R link, but not the allocation of resources by the reader for the R2D/D2R link.

In our view, the resource validity should include potentially the time when UE reader is in IDLE/INACTIVE, i.e. moving to IDLE or INACTIVE should not be a sole reason to release AIoT resources and abandon AIoT operation within a reasonable timeframe.

UE reader being released to RRC_IDLE or RRC_INACTIVE shall not be the sole reason to require the Reader to abandon an ongoing AIoT operation and/or the AIoT resources allocated to it by the NW unless the resources have become invalid for other reason (e.g. based on timer or NW reconfigured or explicitly released the resource, etc.).

To keep the design simple, timer-based resource validity can be considered.

Timer based resource configuration validity is simple and fully under NW control.

Furthermore, it would be beneficial for the NW to be aware of the radio resources needed by a certain reader at a certain time for the more efficient operation. Based on the messages from the AIoT Function (AIoTF) to the UE reader, e.g. to perform an inventory or a command, the reader UE can request the needed radio resources from the gNB. This request can help the gNB scheduler to efficiently allocate the radio resources for the UE reader for the AIoT operation.

In addition, the AIoT resources that the UE reader is able to utilize may also depend on the deployment information where the reader may have better knowledge. For example, the tags deployed under a certain reader may be able to only work in a certain frequency band, and without such knowledge, gNB providing AIoT resources autonomously in another band will basically make the AIoT reader inoperable with the tags.

AIoT resource request from UE reader helps gNB for efficient resource allocation.

UE reader can request AIoT resources from the gNB. The actual resource allocation is decided by the gNB.

UE Reader in RRC_CONNECTED is configured with a resource validity timer. The resource configuration is valid until the validity timer expires.

UE reader being released to RRC_IDLE or RRC_INACTIVE shall not be a sole reason to require the Reader to abandon an ongoing AIoT Operation and the AIoT resources allocated to it by the NW unless the resources have become invalid for other reason (e.g. based on timer or NW reconfigured or explicitly released the resource, etc.).

## UE Reader temporarily out-of-connection

While one can argue that the UE reader should always be in coverage of the network e.g. to get the commands from AIoTF, as already agreed by RAN2, there can be valid scenarios where the UE Reader is temporarily out of connection e.g. due to HO or RLF. E.g., the commands from the AIoTF arrive to the UE Reader at time t1 while it is in coverage; the UE reader starts to perform the actual inventory or command with the AIoT devices (tags) at t2, but during this time before the AIoT operation is complete (recall that AIoT operations can take much longer than couple of milliseconds), the UE Reader suffers RLF.

In such case, the UE reader should not be required to prematurely stop all the ongoing AIoT operations as long as the previously provided resource allocation is still valid. In other words, merely going through temporary interruption in Uu should not require the Reader to abandon AIoT resources already provided to it by the NW unless the resources have become invalid for other reason (e.g. based on timer or NW reconfigured and released the resource, etc.)

Temporary interruption in the Uu shall not require the Reader to abandon AIoT resources already provided to it by the NW unless the resources have become invalid for other reason (e.g. based on timer or NW reconfigured and released the resource, etc.).

Then, after performing the inventory or command, the Reader may come back to the coverage at t3 and can successfully transmit the collected AIoT data to the AIoTF.

As explained above, UE Reader performing AIoT operations with the AIoT devices while suffering from temporary interruption in Uu is a valid scenario. Therefore, we propose:

Temporary interruption in the Uu shall not require the Reader to abandon AIoT operation and/or the resources already allocated to it by the NW unless the resources have become invalid for other reason (e.g. based on timer or NW reconfigured and released the resource, etc.).

## UE Reader’s RRC state

While one can argue that the UE reader should always be in RRC_CONNECTED state e.g. to get the commands from AIoTF, there can be valid scenarios where the UE Reader operates while being in other RRC states. E.g. the commands from the AIoTF arrive to the UE Reader at time t1 while it is in RRC_CONNECTED (or RRC_INACTIVE using MT SDT), and the UE Reader performs the actual inventory or command with the AIoT devices (tags) while being in RRC_IDLE or RRC_INACTIVE state, then UE Reader transitions to RRC_CONNECTED (or uses SDT from RRC_INACTIVE) after performing the inventory or command, to transmit the collected AIoT data to the AIoTF.

Note that the timescales for the AIoT operation and NR Uu operation are expected to be quite different. The NR Uu slots (and the potential resource allocation dynamicity) are in sub-milliseconds whereas AIoT operation can take multiple hundreds of milliseconds or multiple seconds. That means, while the UE reader is engaged in R2D/D2R, on the NR Uu side, it can reasonably go to RRC_IDLE or RRC_INACTIVE state saving on UE power.

Considering this, following was agreed, however explicit agreement on RRC_state of Reader was not reached in the last meeting:

2 The UE may perform the AIoT procedure on AIoT interface between the reader and the device only if the resource configuration is valid in the cell under network control.    FFS how valid resource is determined – we will consider temporary out of connection scenarios.    FFS on resource validity across multiple cells.

UE Reader performing AIoT operations with the AIoT devices is not limited to RRC_CONNECTED state, i.e. the UE Reader can be in RRC_IDLE or RRC_INACTIVE state while performing R2D/D2R activities, as long as the resource configuration is valid.

Therefore, we propose:

UE Reader can be in any RRC State while performing R2D/D2R activities. Consequently, the UE reader in any RRC state can collect AIoT data from the AIoT device(s).

## Handling of AIoT traffic by the UE reader

As explained above, the UE reader may collect AIoT data from the device(s) while the UE reader is in  RRC_IDLE/ RRC_INACTIVE (in addition to RRC_CONNECTED). However, it should be noted that the UE reader needs to be in RRC_CONNECTED mode to send the collected AIoT data to the AIoTF.

The UE reader may collect AIoT data from the device(s) while the UE reader is in  RRC_IDLE/ RRC_INACTIVE; however, it needs to be in RRC_CONNECTED mode to send the collected AIoT data to the AIoTF.

Therefore, if any AIoT data is collected while the UE reader is in RRC_IDLE/INACTIVE, it needs to establish RRC connection to send the AIoT data to the AIoTF. One question to be answered is whether the UE Reader must trigger RRC connection establishment procedure as soon as there is any AIoT data that is collected. In our understanding, AIoT data is delay tolerant traffic. Therefore, there is no need to immediately start RRC Connection establishment. The UE reader can accumulate/aggregate AIoT data from multiple AIoT devices over a NW-configured period of time and then move to CONNECTED mode to transmit the data to the NW.

The UE reader in RRC_IDLE/INACTIVE can collect/aggregate the AIoT data from one or more devices during a NW-configured time duration. After the expiry of such duration, if there is any AIoT data collected, the UE reader moves to RRC_CONNECTED to send the collected AIoT data to the AIoTF.

Another question is whether the UE reader needs to treat the collected AIoT data any differently compared to the UE’s normal traffic for other applications. Given the traffic characteristics may be different (e.g. AIoT data may be delay tolerant while other application data may be delay sensitive), we think the UE reader should be able to handle AIoT data vs other data as belonging to different priorities or traffic classes or with different QoS requirement, e.g. by using a separate DRB for the AIoT traffic.

The traffic characteristics and QoS requirements of AIoT data may be different from UE’s other data.

UE reader should be able to handle AIoT data differently from other data, e.g. using a separate DRB for the AIoT traffic.

## AIoT Data transport from Reader to the network

It is our understanding that companies have proposed various options for AIoT data communication between the UE Reader and the AIoT Function (AIoTF).

RAN2 previously agreed:

4 RAN2 will study the impacts to RAN2 based on the SA2 identified architecture options (i.e. no new AS layer architecture options will be studied)

That means the following 3 options are on the table.

Option 1 (RRC-based): In this option, UE Reader uses its RRC connection with the gNB to send/receive AIoT messages explicitly using RRC. gNB then forwards the AIoT messages to the AIoTF explicitly over N2.

Option 2 (NAS-based): In this option, UE Reader uses its NAS association with the AMF to send/receive AIoT messages as NAS PDUs. AMF can then forward the AIoT messages to the AIoTF. In this solution, the AIoT enabled gNB is unaware of the contents of the AIoT messages as it is received as a NAS container.

Option 3 (PDU session-based): In this option, UE Reader uses user plane signalling (establishes a PDU session till UPF) to communicate with AIoTF. UPF can then forward the AIoT messages to the AIoTF. In this solution, the AIoT enabled gNB is unaware of the contents of the AIoT messages as it is sent over user plane.

In the table below, we compare the three options by discussing the protocol stack, interface impacts and their pros vs. cons.

| RRC-based (option 1) | NAS-based (option 2) | PDU session-based (option 3) |
| --- | --- | --- |
| UE Reader uses its RRC connection with the gNB to send/receive AIoT messages explicitly in RRC. gNB then decodes the AIoT RRC messages and forwards the AIoT messages to the AIoTF explicitly over N2. | UE Reader uses its NAS association with the AMF to send/receive AIoT messages as NAS PDUs. AMF can then forward the AIoT messages to the AIoTF via 5GC interfaces. | UE Reader uses user plane signaling (establishes a PDU session till UPF) to communicate with AIoTF. UPF can then forward the AIoT messages to the AIoTF via 5GC interfaces. |
| AIoT enabled gNB is aware of the contents of the AIoT messages as it is received explicitly over RRC. However, it adds unnecessary RRC ASN.1 complexity. | AIoT enabled gNB is unaware of the contents of the AIoT messages as it is received as a NAS container AIoT enabled gNB can be made aware explicitly by the AIoTF providing assistance information. | AIoT enabled gNB is unaware of the contents of the AIoT messages as it is sent over user plane AIoT enabled gNB can be made aware explicitly by the AIoTF providing assistance information. |
| No end-to-end protocol between the UE reader and the AIoTF | There is an end-to-end protocol between the UE reader and the AIoTF. | There is an end-to-end protocol between the UE reader and the AIoTF |
| RRM like solution This is like how gNB can explicitly request RRM measurements to UE explicitly via RRC | LPP-like solution This is like how LPP is defined between UE and LMF and carried over NAS/NGAP/RRC NAS-SM like solution This is like how NAS-SM is defined between UE and SMF and carried over NAS-MM/NGAP/RRC | SUPL like solution This is like how SUPL (Secure User Plane Location Protocol) is used to carry location between a UE and location server PC3a like solution This is like how PC3a is used to transfer ProSe messages between UE and DDNMF |
| UE Reader selection can be performed by A-IoT enabled gNB or AIoTF | UE Reader selection performed by AIoTF | UE Reader selection performed by AIoTF |
| Overload on AMF if there are lot of AIoT devices | Overload on AMF if there are lot of AIoT devices | UPF can handle IP traffic better. |
| A-IoT enabled gNB might need to perform aggregation of the Inventory/Command Reports from UE Readers (and BS Readers) before sending it to AIoTF | A-IoT enabled gNB can forward the Inventory/Command Reports to AIoTF without performing aggregation and AIoTF performs aggregation | A-IoT enabled gNB can send Inventory/Command Reports to AIoTF without performing aggregation and AIoTF performs aggregation |
| RAN2 impact: High RAN2 would need to define new RRC signaling to support the reception/transmission of AIoT related messages from gNB to UE Reader RAN2 would need to define procedures when the UE Reader’s serving gNB changes (e.g., due to mobility) or RRC state changes | RAN2 impact: No/minimal | RAN2 impacts: No/minimal |
| RAN3 impact: High RAN3 would need to define explicit signaling over N2 for the AIoT messages In case the serving gNB of the UE Reader changes (e.g., due to mobility) or the RRC state of the UE reader changes, Xn coordination is needed to provide UE Reader’s context from source gNB to target gNB | RAN3 impact: No/minimal No new procedures needed when the UE Reader’s serving gNB changes or the UE Reader changes its RRC state | RAN3 impact: No/minimal No new procedures needed when the UE Reader’s serving gNB changes or the UE Reader changes its RRC state |
| SA2 impact: Medium SA2 needs to define the interface between AMF and AIoTF (Nac interface) | SA2 impact: Medium SA2 needs to define the interface between AMF and AIoTF (Nac interface) and support the end-to-end protocol | SA2 impact: Medium SA2 needs to define the interface between UPF and AIoTF (Nuc interface) and support the end-to-end protocol |

Considering there are high impacts to the UE reader, the AIoT enabled gNB, as well as the specification efforts for RAN2/RAN3 with Option 1 (RRC-based), it is our view that RAN2 should not prefer this option.

Comparing the pros and cons between the Option 2 (NAS-based) and Option 3 (PDU-based), it is clear that option 3 has less impacts to RAN2/3 work, can provide homogenous transport between different nodes (from Reader to the AIoTF), and can also support common protocol stack for the Reader in both topologies. So, while our preference is clear that option 3 should be the way to go, we invite companies to study further the options 2 and 3.

Given the high impacts to RAN2/RAN3, RRC-based AIoT data transport between the Reader and the gNB is not considered.

For AIoT data transfer between the reader and the AIoTF, PDU-session based AIoT data transfer has clear advantages over NAS-based AIoT data transfer.

Therefore, we propose:

PDU-session based AIoT data transfer between the reader and AIoTF is studied. FFS NAS-based data transfer between Reader and AIoTF.

# Summary

Based on the discussion above, we have following observations:

Observation 1.	UE reader being released to RRC_IDLE or RRC_INACTIVE shall not be the sole reason to require the Reader to abandon an ongoing AIoT operation and/or the AIoT resources allocated to it by the NW unless the resources have become invalid for other reason (e.g. based on timer or NW reconfigured or explicitly released the resource, etc.).

Observation 2.	Timer based resource configuration validity is simple and fully under NW control.

Observation 3.	AIoT resource request from UE reader helps gNB for efficient resource allocation.

Observation 4.	Temporary interruption in the Uu shall not require the Reader to abandon AIoT resources already provided to it by the NW unless the resources have become invalid for other reason (e.g. based on timer or NW reconfigured and released the resource, etc.).

Observation 5.	UE Reader performing AIoT operations with the AIoT devices is not limited to RRC_CONNECTED state, i.e. the UE Reader can be in RRC_IDLE or RRC_INACTIVE state while performing R2D/D2R activities, as long as the resource configuration is valid.

Observation 6.	The UE reader may collect AIoT data from the device(s) while the UE reader is in  RRC_IDLE/ RRC_INACTIVE; however, it needs to be in RRC_CONNECTED mode to send the collected AIoT data to the AIoTF.

Observation 7.	The traffic characteristics and QoS requirements of AIoT data may be different from UE’s other data.

Observation 8.	Given the high impacts to RAN2/RAN3, RRC-based AIoT data transport between the Reader and the gNB is not considered.

Observation 9.	For AIoT data transfer between the reader and the AIoTF, PDU-session based AIoT data transfer has clear advantages over NAS-based AIoT data transfer.

Based on these observations and discussion above, we have the following proposals:

Proposal 1:	UE reader can request AIoT resources from the gNB. The actual resource allocation is decided by the gNB.

Proposal 2:	UE Reader in RRC_CONNECTED is configured with a resource validity timer. The resource configuration is valid until the validity timer expires.

Proposal 3:	UE reader being released to RRC_IDLE or RRC_INACTIVE shall not be a sole reason to require the Reader to abandon an ongoing AIoT Operation and the AIoT resources allocated to it by the NW unless the resources have become invalid for other reason (e.g. based on timer or NW reconfigured or explicitly released the resource, etc.).

Proposal 4:	Temporary interruption in the Uu shall not require the Reader to abandon AIoT operation and/or the resources already allocated to it by the NW unless the resources have become invalid for other reason (e.g. based on timer or NW reconfigured and released the resource, etc.).

Proposal 5:	UE Reader can be in any RRC State while performing R2D/D2R activities. Consequently, the UE reader in any RRC state can collect AIoT data from the AIoT device(s).

Proposal 6:	The UE reader in RRC_IDLE/INACTIVE can collect/aggregate the AIoT data from one or more devices during a NW-configured time duration. After the expiry of such duration, if there is any AIoT data collected, the UE reader moves to RRC_CONNECTED to send the collected AIoT data to the AIoTF.

Proposal 7:	UE reader should be able to handle AIoT data differently from other data, e.g. using a separate DRB for the AIoT traffic.

Proposal 8:	PDU-session based AIoT data transfer between the reader and AIoTF is studied. FFS NAS-based data transfer between Reader and AIoTF.

# References

[1] RP-240826, Study on solutions for Ambient IoT (Internet of Things) in NR, RAN#103, March 2024

[2] TR 38.848, Study on Ambient IoT (Internet of Things) in RAN, Release 18

[3] TS 38.300, NR and NG-RAN Overall Description; Stage 2