3GPP TSG-RAN WG2 Meeting #128	R2-2410576

Orlando, USA, Nov 18th – 22nd, 2024

Agenda item:		8.2.4

Source:		Futurewei

Title:		Issues with previous agreement on Msg2 in 2-step CBRA

Document for:		Discussion and Decision

# Introduction

In meeting #127-bis, RAN2 reached the following agreements regarding A-IoT random access [1]:

Agreements

- In case of D2R data transmission failure, device follows the reader instruction:

     it is supported to re-access in another opportunity controlled/provided by the reader (i.e. retry the random access); and

     reader can repeat the R2D “upper layer command” to trigger the device to re-send the same D2R “response” (i.e., device just follows the received R2D to transmit D2R).

2	Support optional explicit R2D failure/success feedback indication for at least MSG3 for re-access purpose.  FFS for following D2R data.

3	For 2step CBRA, if mgs2 is not received by the device, the device is not expected to autonomously re-access.  The re-access is always controlled by reader.

4	For 2step and 3step CBRA, The A-IoT device performs re-access in another opportunity provided by the reader (i.e. retry the random access) at least in case of contention resolution failure.

5	R2D message is used to provide access occasion(s) which can be used for re-access purpose. One option to be captured in the TR is that this R2D message is AIoT paging message.   FFS additional indications to differentiate if needed in paging message.    FFS if other message can be used.

6	From RAN2 perspective, at least for TDMA the device can randomly select one access occasion for A-IoT Msg1 within the access occasions provided/assigned by the reader, as the baseline for CBRA.  FFS if this is applicable to FDMA.  Further enhancement option(s) can be considered after more RAN1 progress on TDMA/FDMA.

In this contribution, we discuss some issues with Agreement #3 above (on Msg2 in 2-step CBRA) and propose a revision to this agreement.

# Discussions

One of the reasons that Agreement #3 above was reached in the previous meeting, as a comprise, is that a use case was raised where the device may not require a positive acknowledgement from the reader for the Msg1 that the device has sent in a 2-step CBRA procedure. For example, the device may be semi-periodically triggered for sending a temperature reading using a 2-step CBRA procedure, where occassional D2R transmission errors in the Msg1s sent would not be critical to the service as long as certain percentage of the Msg1s can be received correctly during a specific period. Therefore, the device prioritizes its energy conservation and harvesting over the reliability of its Msg1 by becoming unavailable for communications as soon as sending its Msg1 in the 2-step CBRA procedures. It is up to the reader to use proper settings (e.g., the periodicity of R2D triggers, CW transmission power, radio resources and data rate scheduled for the Msg1, etc.) to ensure that the number of Msg1s received during a specific period exceeds a threshold defined by the service outage criteria. Therefore, the conclusion made in the previous meeting was that (for this type of devices) the absence of Msg2 does not cause the device to autonomously re-access. This agreement also implies that the device does not need to listen for Msg2, and therefore, the reader is not required to send Msg2 to acknowledge the Msg1 received in 2-step CBRA procedures.

However, there are other use cases where the devices do care about the result of the Msg1 they have sent in a 2-step CBRA procedure, whether the Msg1 is sent as a response to inventory request or to a read/write command. For these devices, the Msg2 in the 2-step CBRA procedure serves as an acknowledgement of the Msg1 sent, based on which the device may become unavailable for communication with the reader after the Msg2 until a next availability time for the device, while an absence of the Msg2 can cause the device to autonomously attempt re-access as soon as possible, without waiting for the reader to repeat the same A-IoT paging message or for the A-IoT CN to repeat the same A-IoT Service Request. On the other band, if the reader is not required to send Msg2 in response to Msg1 in 2-step CBRA procedures (as it is the case currently), without knowing whether the reader is expected to send out the Msg2 or not, in the absence of Msg2, these devices may waste a lot of energy waiting for the reader to repeat the same paging message or for the A-IoT CN to repeat the same A-IoT Service Request because there is great uncertainty in time as to when the reader or the A-IoT CN may discover the failure in the Msg1 transmission and arrange the repetition.

In our view, the correct design should be that the reader is required (or at least expected) to send Msg2 in response to Msg1 received in 2-step CBRA procedure. For a device, of which the service does not require the use of Msg2 as an acknowledgement to the Msg1 sent in 2-step CBRA procedures, the device can choose not to listen to Msg2 or to ignore it, if received, so that the device can conserve its energy and spend more time on energy harvesting, and if Msg2 is not received by the device, the device is not expected to autonomously re-access. Meanwhile, for a device expecting to receive Msg2 as an acknowledgement to its Msg1 sent in the 2-step CBRA procedure, the device can autonomously attempt re-access if Msg2 is not received by the device. In this way, a faster re-access by the device is enabled while the re-access is still under the reader’s control, i.e., with the absence of the Msg2 expected.

Proposal: Revise the previous Agreement #3 as shown at the beginning as follows:

- The reader is required (or at least expected) to send Msg2 in response to Msg1 received in 2-step CBRA procedure.

- For a device, of which the service does not require the use of Msg2 as an acknowledgement to the Msg1 sent in 2-step CBRA procedures, the device can choose not to listen to Msg2 or to ignore it, if received, so that the device can conserve its energy and spend more time on energy harvesting, and if Msg2 is not received by the device, the device is not expected to autonomously re-access.

- For a device expecting to receive Msg2 as an acknowledgement to its Msg1 sent in the 2-step CBRA procedure, the device can autonomously attempt re-access if Msg2 is not received by the device.

# Conclusions

We propose the following:

Proposal: Revise the previous Agreement #3 as shown at the beginning as follows:

- The reader is required (or at least expected) to send Msg2 in response to Msg1 received in 2-step CBRA procedure.

- For a device, of which the service does not require the use of Msg2 as an acknowledgement to the Msg1 sent in 2-step CBRA procedures, the device can choose not to listen to Msg2 or to ignore it, if received, so that the device can conserve its energy and spend more time on energy harvesting, and if Msg2 is not received by the device, the device is not expected to autonomously re-access.

- For a device expecting to receive Msg2 as an acknowledgement to its Msg1 sent in the 2-step CBRA procedure, the device can autonomously attempt re-access if Msg2 is not received by the device.

# References

R2_127bis_ChairNotes_24-10-18_13-00.