3GPP TSG RAN WG2 Meeting #128	 R2-2410581

Orlando, FL, USA, November 18-22, 2024

Agenda Item:	8.1.2.2

Source:	Futurewei

Title:	Discussion on LCM for UE-Side Model for Beam Management use case

Document for:	Discussion

# Introduction

In RAN2 meeting #127bis [6], the discussions were focused on the following topics.

- Signalling of functionality applicability report

- Reporting of non-applicable functionalities

- Activation/Deactivation/Fallback

- Data Collection and model training

On the signaling discussion, companies shared their views on what existing mechanisms can be used to report applicable functionalities and converged to use UAI and RRCReconfigurationComplete message for reporting applicable functionality.

Companies also discussed whether to report non-applicable functionalities. While some companies didn’t see the reasons for doing it, others think it necessary because the main motivation is to report when a functionality is no longer applicable; in this case, there should be a way to report a non-applicable functionality and the reason, i.e. no model for the functionality. The need of the mechanism was confirmed by the group and the corresponding agreement was reached.

The topics/concepts of Activation/Deactivation/Fallback were discussed again in this meeting. There were two camps of companies, one camp thinks we should define what Fallback is while the other camp does not think so. Companies also had different views on how fallback works; some think fallback should be configured or controlled by the network, while others think how it works is not clear. After an in-meeting ad-hoc discussion, companies agree that the existing procedures and terminologies from the CSI Framework should be used. For now, RAN2 will not define terminology specific to the activation or deactivation for AI/ML models; we can come back to this discussion later if necessary.

For data collection and model training, the discussion was focused on the mechanism of initiating the data collection and whether the network should have the control. The options include UE-request, NW-demand, or pre-configured. Although some companies think this can be UE-implementation issue, the group agreed that both data collection initiation and configuration for data collection are under network control.

The agreements reached are listed below.

Agreements for BM

- UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality.   We should aim to align the design on how the applicable functionality are signaled.   FFS on the applicability reporting content.

- FFS if inference configuration can be signalled in step3.

- UE can report to the network when an applicable AI functionality becomes non-applicable.  FFS how this is signaled (e.g. explicitly/implicitly).  Consider different scenarios, whether it is regarding an active functionality)

- Data collection initiation and configuration for data collection is under network control.  FFS how the NW determines whether data collection should be initiated (e.g. via UE requests (UE directly or UE server)

- For the purpose of discussion of AI/ML BM LCM operations, existing procedures and terminologies from the CSI Framework should be used, including those defined for aperiodic, semipersistent on PUCCH, semipersistent on PUSCH, and periodic reporting configurations (as/if defined in RAN1 pending response LS from RAN1).

- For now, RAN2 will not define terminology specific to the activation or deactivation for AI/ML models.  Can come back to this discussion later.

In this contribution, we will further discuss LCM for UE-side models focusing on the details, clarifications and revisions of the LCM procedures.

# Procedures for UE-side LCM

## Background Information

In RAN2 meeting #127 [5], the procedures for UE-side LCM were discussed, based on the result of post-meeting 126 email discussion. The following agreements were achieved [5].

- Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities.

- Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side.

- “Step 3”: Following configurations are provided from NW to UE:

1) UE is allowed to do UAI reporting via OtherConfig.

2) Network may provide NW-side additional condition.

- FFS on the RRC signalling and whether it is mandatory or optional.

3) FFS on configuration (e.g. inference configuration) of supported functionalities.

- FFS on the content of configuration.

- UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device.

  - FFS whether other configuration can be considered by UE (e.g. inference configuration).

  - FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

- “Step 4”: UE reports applicable functionality in the following scenarios:

- Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI

- As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration).

  - FFS via UAI or RRCReconfigurationComplete, etc

- Step 5:

1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5).

2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not.

- The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5.

  - FFS the initial activation state.

  - FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3.

  - FFS on additional L1/L2 signalling for activation/deactivation.

  - FFS if multiple applicable functionalities can be activated at the same time.

  - FFS what is the granularity of functionality

- We will write an LS to RAN1 to provide our agreements and ask specific questions that RAN2 needs to enable progress.

In the following sections, the discussions will be based on the above agreements.

## NW-side additional conditions

In Step 1 and Step 2 of the agreed LCM procedure, NW configures the UE to report supported functionalities and UE reports them. It is our understanding that the main purpose of Step 3 is for the NW and the UE to exchange necessary information so that the UE can decide on the applicable functionalities. For example, the NW provides NW-side additional conditions so that the UE can choose the right applicable functionality. Therefore, we think this step (i.e., the NW providing additional conditions to the UE) should be mandatory, unless the model was trained specifically for this NW.

Proposal 1: Revised “Step 3” as below.

- “Step 3”: Following configurations are provided from NW to UE:

1) UE is allowed to do UAI reporting via OtherConfig.

2) Network shall may provide NW-side additional condition if applicable.  FFS on the RRC signalling. and whether it is mandatory or optional.

3) FFS on The configuration (e.g. inference configuration) of supported functionalities is up to individual use case.

## Determining applicable functionalities

Based on current agreement, UE decides the applicable functionalities after NW’s configuration in Step 3. The RAN2 agreement on this step says that

- UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device.

  - FFS whether other configuration can be considered by UE (e.g. inference configuration).

  - FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

This agreement lists two topics for further study. The first one being whether other configuration can be considered by UE (e.g. inference configuration). This topic has been discussed in RAN1 with results, as a reply to RAN2’s LS. The agreements reached in RAN1 meeting #118bis are listed below.

Agreement

RAN 1 further study the following options for applicability for inference for UE-side model:

Option 1:

- In Step 3, following configurations are provided from NW to UE:

  - 1) UE is allowed to do UAI reporting via OtherConfig,

  - 2)+3) NW configures one or more CSI-ReportConfig for inference configuration, where the associated ID may be configured in CSI framework as working assumption applied.

  - FFS on whether some IEs in the CSI report configuration can be removed or modified

  - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3

- In Step 4, UE reports applicability(ies) of the above CSI-ReportConfig

  - FFS on one or more of the above CSI-ReportConfig to be reported

- FFS on activation (including when/how) of inference report after obtaining the applicability from UE Step 4.

- FFS: whether Step 5 is needed,

Option 2:

- In Step 3, following configurations are provided from NW to UE:

  - UE is allowed to do UAI reporting via OtherConfig,

  - NW configures one set or multiple sets of inference related parameters

    - Note: the set of inference related parameters is not configured by CSI-ReportConfig

    - FFS on the set of inference related parameters, at least including:

      - Set A related information

      - Set B related information

      - Report content related information

      - For BM-Case 2,

        - Time instances related information for measurements

        - Time instances related information for prediction

  - The associated ID(s) may be configured

    - wherein the associated ID(s) may be

      - FFS: a) part of one set of the inference related parameters, or

      - FFS: b) independently from the one set of the inference related parameters.

- In Step 4, UE reports applicability of the above one or multiple sets of inference related parameters, where the associated ID information may be associated.

- In Step 5, NW configures configuration(s) for CSI report for inference

Option 3:

- In Step 3, following configurations are provided from NW to UE:

  - 1) UE is allowed to do UAI reporting via OtherConfig,

  - 2) The associated ID(s) may be provided to UE, e.g., a new RRC parameter.

- In Step 4, UE reports by UAI

  - the applicable one or multiple sets of inference related parameters may be included.

    - FFS on the set of inference related parameters, at least including:

      - Set A related information

      - Set B related information

      - Report content related information

      - For BM-Case 2,

        - Time instances related information for measurements

        - Time instances related information for prediction

    - Note: not applicable may also be replied by UE

    - Note: if the inference related parameters are not supported for reporting, only the applicability(ies) or not is reported in Step 4.

  - the associated ID(s) may be included

    - FFS: a) as part of the inference related parameters, or

    - FFS: b) independently from the set of the inference related parameters.

- In Step 5, NW configures configuration(s) for CSI report for inference.

Note: There is no impact of configuring CSI report configuration for non-AI beam management in RRCReconfiguration.

We can see that in both Option 1 and Option 2, inference configuration can be done in Step 3. In Option 3, it is done in Step 5. Because all three options have been determined for further study in RAN1, we think RAN2 can just wait for RAN1’s final decision on the option they will choose from the three.

Observation 1: Given that other configurations (e.g. inference configuration) by the UE to determine applicable functionalities after Step 3 is currently being studied in RAN1. RAN2 can make a decision after RAN1 gets back to us.

The second topic for further study is how the applicable functionality is decided if NW-side additional conditions are not provided in step 3. It is common understanding that the additional conditions are used to align NW-side and UE-side conditions/configurations between training and inference stages to ensure expected model performance. The NW should provide additional conditions to the UE if it thinks these conditions are important for the performance. Therefore, when the additional conditions are not provided from the NW, it is reasonable to assume either they have already been considered by the supported functionality reported to the NW in Step 2 (e.g., in the case of NW-specific model) or they are not important for the performance. In this case, applicable functionality can be determined based on the supported functionality without additional conditions. In fact, for each use case or sub use case, there should be only one functionality so the determination should be straightforward (but there could be multiple models).

Observation 2: Applicable functionalities can be determined by selecting from supported functionalities reported to the NW in Step 2, without considering additional conditions after Step 3, if they are not provided from the NW. How this is done is a UE implementation issue.

## The granularity of functionality

After Step 5, the applicable functionality may be activated by receiving its inference configuration. One of the points for further discussion is the granularity of functionality. Although there are no detailed statements in the TR or agreements in R19 discussions on the granularity of a functionality, from the following quotes from the TR, we can see that a functionality is roughly at the level of or lower than (i.e., smaller than) an AI/ML-enabled Feature or FG, which is at the use case level (or sub use case level if the use case has sub use cases).

- Where AI/ML functionality resides depends on the specific use cases and sub use cases.

- For functionality identification, there may be either one or more than one Functionalities defined within an AI/ML-enabled feature, whereby AI/ML-enabled Feature refers to a Feature where AI/ML may be used. Note: UE may have one AI/ML model for the functionality, or UE may have multiple AI/ML models for the functionality.

- For AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models, functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG.

Observation 3: The granularity of functionality is at the use case or sub use case level, if the use case has sub use cases.

## Activating multiple applicable functionalities at the same time

After Step 5, the applicable functionality may be activated by the received inference configuration in Step 5. In this stage, one of the topics listed for further discussion is whether multiple applicable functionalities can be activated at the same time.

  - FFS if multiple applicable functionalities can be activated at the same time.

However, we think the question itself needs to be clarified first.

When we talk about multiple functionalities, there could be two different things.

- In the first case, when these multiple applicable functionalities are for the same use case/sub-use case, they are likely developed for different applicable situations. In this case we need some mechanisms to differentiate different applicable functionalities (e.g., functionality ID).

- In the second case, when these multiple applicable functionalities are not for the same use case/sub-use case, for example, one for beam management and the other for CSI prediction, then there is no reason to activate them at the same time, for the same use case/sub use case. Note that it should be always allowed to activate different functionalities for different use cases or sub use cases.

Based on the above discussion, we think the FFS belongs to the first case. We hence propose the following revision to the agreement. After this clarification, whether they can be activated at the same time can be further studied.

Proposal 2: Revise the procedures for UE-side LCM as below.

- The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5.

  - FFS the initial activation state.

  - FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3.

  - FFS on additional L1/L2 signalling for activation/deactivation.

  - FFS if multiple applicable functionalities for the same use case/sub use case can be activated at the same time.

  - FFS what is the granularity of functionality

## Reporting of UE internal capability/capacity

Besides AI/ML-based UE capability reporting (which associates with AI/ML functionality), another important aspect the UE needs to report to the NW is the UE’s internal capability or capacity for model inference, for example, its computational power and memory. Note this is different from AI/ML-based UE capability reporting; it is used for the NW to decide whether the UE will be able to handle the task of certain model inference.

This is also touched in the agreements, between Step 3 and Step 4, as listed below.

  - UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device.

    - FFS whether other configuration can be considered by UE (e.g. inference configuration).

    - FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

The “UE-side additional conditions (internally known by UE)” in this proposal can include the UE’s capability or capacity for model inference. we believe this information should also be made known to the NW (not only to the UE itself) for various reasons. This information may include UE’s computational power, memory, storage, model inference time, battery level etc. that are applicable for AI/ML functionality operations.

With this information, the NW can have a better understanding of the resources available at the UE side and activate/deactivate AI/ML-based functionality at the right time/situation. For example, if the UE’s current computational power or memory is not able to support the inference of a certain functionality, the NW will not instruct the UE to activate the functionality.

To capture this aspect, we proposed a revised agreement for the above text and Step 4.

Proposal 3: Revise the procedures for UE-side LCM as below.

- UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE, including UE’s capabilities and capacities that are related to AI/ML functionality operations) and model availability in device.

  - FFS whether other configuration can be considered by UE (e.g. inference configuration).

  - FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

- “Step 4”: UE reports applicable functionality and its internal capabilities/capacities in the following scenarios:

  - Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI

  - As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration).

    - FFS via UAI or RRCReconfigurationComplete, etc

# Conclusions

In this contribution, we discussed our views on LCM for UE-side models for the beam management use cases. Based on the discussions in the previous sections, our proposals are as follows.

Proposal 1: Revised “Step 3” as below.

- “Step 3”: Following configurations are provided from NW to UE:

1) UE is allowed to do UAI reporting via OtherConfig.

2) Network shall may provide NW-side additional condition if applicable.  FFS on the RRC signalling. and whether it is mandatory or optional.

3) FFS on The configuration (e.g. inference configuration) of supported functionalities is up to individual use case.

Observation 1: Given that other configurations (e.g. inference configuration) by the UE to determine applicable functionalities after Step 3 is currently being studied in RAN1. RAN2 can make a decision after RAN1 gets back to us.

Observation 2: Applicable functionalities can be determined by selecting from supported functionalities reported to the NW in Step 2, without considering additional conditions after Step 3, if they are not provided from the NW. How this is done is a UE implementation issue.

Observation 3: The granularity of functionality is at the use case or sub use case level, if the use case has sub use cases.

Proposal 2: Revise the procedures for UE-side LCM as below.

- The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5.

  - FFS the initial activation state.

  - FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3.

  - FFS on additional L1/L2 signalling for activation/deactivation.

  - FFS if multiple applicable functionalities for the same use case/sub use case can be activated at the same time.

  - FFS what is the granularity of functionality

Proposal 3: Revise the procedures for UE-side LCM as below.

- UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE, including UE’s capabilities and capacities that are related to AI/ML functionality operations) and model availability in device.

  - FFS whether other configuration can be considered by UE (e.g. inference configuration).

  - FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

- “Step 4”: UE reports applicable functionality and its internal capabilities/capacities in the following scenarios:

  - Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI

  - As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration).

    - FFS via UAI or RRCReconfigurationComplete, etc

# References

Chair’s notes, R2_126_ChairNotes_24-05-24_final, RAN2 meeting #126, Fukuoka, May 2024

RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN meeting #102, Qualcomm (moderator)

RP-233946, 3GPP TR 38.843, Technical Specification Group Radio Access Network; Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18) v2.0.1 (2023-12)

Chair’s notes, R2_125bis_ChairNotes_24-04-19_final, RAN2 meeting #125bis, Changsha, April 2004

Chair’s notes, R2_127_ChairNotes_ 24-08-23_17-00_eom, RAN2 meeting #127, Maastricht, Aug 2024

Chair’s notes, R2_127bis_ChairNotes_ 24-10-18_15-45-_eom_clean, RAN2 meeting #127bis, Hefei, Oct 2024