3GPP TSG RAN WG2 Meeting #128	 R2-2410582

Orlando, FL, USA, November 18-22, 2024

Agenda Item:	8.1.3

Source:	Futurewei

Title:	Discussion on Data Collection for NW-side Model Training

Document for:	Discussion

# Introduction

In RAN2 meeting #127bis [6], the group discussed and tried to conclude on the following aspects.

- Logging aspects

- Reporting aspects (on-demand/periodic/event based)

- SRB for reporting, and

- Content of the data

For data logging, the focus of the discussion was on whether event-triggered reporting should be supported. The supporters believe that to facilitate data filtering and reduce data logging, event-triggered data logging should be supported. As another reason, RAN2 has agreed to stop data collection depending on power state but there is no understanding when the UE can restart data collection. The opposers think we need to collect data for both good and bad conditions so that we can have a full view, and the model needs to work in both conditions. Therefore, we don’t need the event-triggered data collection. Among the supporters of event-triggered data logging, there are also different views on the triggering events (most believe it should be radio channel conditions). The group eventually decided to support both periodic and event-triggered logging for data collection.

On the reporting aspects, three topics were discussed.

- on periodic reporting

- on event-based reporting

- on whether the UE can initiate logged data transmission or wait for network to send the data

For periodic reporting, the pros may include

- simple mechanism,

- good for large amount of data, etc.

The cons may include

- may not be able to handle huge amount of data (must wait for the next report period),

- could be very inefficient (UE may not have data at reporting time),

- works only if logging is supported.

For event-based/on-demand reporting, some companies think it may be the solution when the UE stops logging due to its memory/battery status. It was supported after discussion.

Regarding whether the UE can initiate logged data transmission or wait for network to send the data, the group decided that the UE can indicates the availability of logged data to the network to assist network to trigger UEInformationRequest. Then UEInformationResponse is used for on-demand reporting of AI/ML training data collection.

On selecting the right SRB for reporting, most companies agreed that low priority SRB will be used (either new SRB or SRB4). Some companies pointed to SRB2 but other disagreed as it may impact NAS procedures that use SRB2. Some companies also proposed to use new SRB, but concern is there are only 32 SRBs and 5 of them have been used (27 left).

For the content of data, for both NW-sided/UE sided BM model training, RAN2 agreed that at least L1-RSRPs and/or beam-IDs needs to be collected by UE.

As a summary of meeting #127bis [6], RAN2 agreed with the following on NW-side data collection.

Agreements

- Periodic logging is supported for training data collection procedure in R19

- Event-triggered data logging will be supported.  At least radio condition-based event triggered logging will be supported.  FFS the details of radio condition-based event.  FFS if other events are supported.

- Periodic reporting of logged data is not supported.

- On-demand reporting of the logged measurements will be specified

- UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection.   FFS of details of the message

- The UE can indicate the availability of logged data to the network to assist network to trigger UEInformationRequest.  FFS trigger/definition of availability indication.  FFS how data availability indication is sent to the network.

- Low priority SRB will be used.    FFS new SRB or use of SRB4

- For data collection for both NW-sided/UE sided BM model training, at least L1-RSRPs and/or beam-IDs needs to be collected by UE.  FFS if other data needs to be collected based on RAN1 progress.

In this contribution, our discussion will focus on the mechanisms and principles identified for data collection for network side model training during R18, as indicated by the Chair.

# Background

In the TR [2], Table 7.3.1.2-1 lists existing data collection mechanisms available in current RAN specifications for the UE to report measurements to another entity acting as termination point for this data. As indicated by the TR, the analysis/selection of the data collection frameworks should focus on the RRC CONNECTED state for both data generation and reporting.

Table 7.3.1.2-1. Existing data collection methods identified.

| Involved network entity (termination point) | RRC state to generate data | Max payload size per reporting* | Contents to be collected | - End-to-End report latency** | Report type | Security and Privacy |
| --- | --- | --- | --- | --- | --- | --- |
| Method: Logged MDT |  |  |  |  |  |  |
| TCE/OAM (Data can be utilized by gNB) | IDLE / INACTIVE | <9kbyte | - L3 cell/beam measurements - location information - sensor information - timing information | - Procedure latency***: - Latency to enter CONNECTED state - Latency to receive gNB request signalling (~20ms) - Air interface signalling latency****: - ~20ms (RRC) - Other latency: - Forwarding latency between gNB and TCE | Upon gNB request after entering RRC_CONNECTED | AS security via RRC message Privacy via user consent |
| Method: Immediate MDT |  |  |  |  |  |  |
| TCE/OAM (Data can be utilized by gNB) | CONNECTED | <9kbyte | - L3 cell/beam measurements - location information - sensor information | - Procedure latency: - Report interval: - 120ms~30min for periodic report - TTT for event triggered report - Air interface signalling latency: - ~20ms (RRC) - Other latency: - Forwarding latency between gNB and TCE | - Event triggered - Periodic reporting | AS security via RRC message Privacy via user consent |
| Method: L3 measurements |  |  |  |  |  |  |
| gNB | CONNECTED | <9kbyte | L3 cell/beam measurements | - Procedure latency: - Report interval: - l20ms~30min for periodic report - TTT for event triggered report - Air interface signalling latency: - 20ms (RRC) | - Event triggered report - Periodic reporting | AS security via RRC message |
| Method: L1 measurement (CSI reporting) |  |  |  |  |  |  |
| gNB | CONNECTED | <1706bit in PUCCH <3840bit in PUSCH | L1 CSI measurement | - Procedure latency: - Report interval: - 4-320 slot for periodic and semi-persistent report - 0-32 slot after reception of DCI for aperiodic report - Air interface signalling latency: - 1 TTI (PUCCH) | - Aperiodic report - Semi-persistent report - Periodic report | No AS security |
| Method: UE Assistance Information (UAI) |  |  |  |  |  |  |
| gNB | CONNECTED | <9kbyte | Assistance information to show UE preference | - Procedure latency: - Upon generation of UE's preference - Air interface signalling latency: - ~20ms (RRC) | Up to UE implementation when to report | AS security via RRC message |
| Method: Early measurements |  |  |  |  |  |  |
| gNB | IDLE / INACTIVE | <9kbyte | L3 cell/beam measurements | - Procedure latency: - Latency to enter CONNECTED state - Latency to receive gNB request signalling (~20ms) - Air interface signalling latency: - ~20ms (RRC) | Upon gNB request after entering RRC_CONNECTED | AS security via RRC message |
| Method: LPP |  |  |  |  |  |  |
| LMF | CONNECTED | <9kbyte | Location information | - Procedure latency: - Latency to get upper layer trigger (for UE triggered) - Or latency to receive network request message (~20ms) - Air interface signalling latency: - ~20ms (RRC) - Other latency: - Forwarding latency between gNB and LMF | - UE-triggered - Network-triggered | AS security via RRC message |

* The payload size doesn't consider signalling overhead.
** The End-to-End report latency is the latency from availability of the measurement report at the UE side to the availability of the measurement report at the terminated network entity. The time to generate data or perform measurements depends on RAN1/RAN4 specification.
*** Procedure latency is the latency caused by procedures, including procedure to ready for reporting (e.g., entering CONNECTED state, report interval).
**** Air interface signalling latency is the latency to transmit one report, e.g., RRC signalling latency or PUCCH signalling latency.

The TR [2] also captured the following considerations for network-side data collection (Section 7.2.1.3.1).

In addition, A set of general data collection principles is expected to be considered for network-side model training. These include:

- UE to support data logging,

- UE to report the collected data periodically, event-based, and on-demand,

- The UE memory, processing power, energy consumption, signalling overhead should be considered.

Furthermore, and regarding the use cases in this study, the following is considered.

For CSI and beam management use cases, the training of network-side models can consider both gNB and OAM-centric data collection mechanisms.

- The gNB-centric data collection implies that the gNB can configure the UE to initiate/terminate the data collection procedure. The potential impact of L3 signalling for the reporting of collected data should be assessed.

- On the other hand, OAM-centric data collection implies that the OAM provides the configuration (via the gNB) needed for the UE to initiate/terminate the data collection procedure. MDT framework can be considered to achieve this. The potential impact on MDT for RRC_CONNECTED state should be assessed.

For positioning use cases, when considering LMF-side inference, it is assumed that the LPP protocol should be applied to the data collected by UE and terminated at LMF, while the NRPPa protocol should be applied to the data collected by gNB and terminated at LMF. While for LMF-side performance monitoring, it is assumed that the LPP protocol should be applied to the data collected by UE and terminated at LMF, while the NRPPa protocol should be applied to the data collected by gNB and terminated at LMF.

# Discussion

## Data logging

In RAN2 meeting #125bis [3] (Changsha), we reached the following agreements for Data Collection for NW-side Model Training.

- RAN2 supports enhancements to MDT for data collection framework for training.

  - FSS Whether to enhance logged or immediate MDT.

In RAN2 meeting #126 [4] (Fukuoka), the group has agreed to take immediate MDT as the baseline for OAM-centric data collection (see the agreement below).

- Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model

In RAN2 meeting #127 [5] (Maastricht), the group has further agreed the following.

- UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE.

- FFS on event-based data collection/logging.

In RAN2 meeting #127bis [6] (Hefei), the group has agreed the following on data logging.

- Periodic logging is supported for training data collection procedure in R19.

- Event-triggered data logging will be supported.

  - At least radio condition based event triggered logging will be supported.

  - FFS the details of radio condition based event.

  - FFS if other events are supported.

Agreement achieved so far can be summarized in the following table.

|  | Periodic | On-Demand | Event-Triggered |
| --- | --- | --- | --- |
| Data Logging | Yes |  | Yes |

As we can see from the table, the support of on-demand logging has not been determined. It is our understanding that on-demand data logging can be useful in some situations. For example, during the training, we may find it necessary to collect data for a specific configuration or a specific time frame during which that has not been collected before. In this example, the task won’t be event-based, and it may not need to be collected periodically. In this case the on-demand approach may be the most suitable one. We therefore think on-demand data logging should be supported.

Proposal 1: Support on-demand data logging and enhancement to logged MDT for data collection for NW-side models.

## Data reporting

### Event/trigger-based reporting

In RAN2 meeting #126 [4], one of the agreements we achieved was the following, on reporting mechanisms.

Enhance the immediate MDT framework to support periodical reporting.

- FFS whether and what event-based reporting is supported, and

- FFS on network request reporting.

In RAN2 meeting #127[5], we further achieved the following agreement, on the support of on-demand reporting.

- On-demand request from the network is supported.   FFS details on signaling.

Therefore, so far, we have agreed to support both enhanced periodical and on-demand (network request) reporting mechanisms. Meanwhile, the support of event-based reporting is still FFS, as agreed below.

- FFS whether AS buffer event-based reporting is supported.

In RAN2 meeting #127bis [6], the group has agreed the following on data logging.

- Periodic reporting of logged data is not supported.

- On-demand reporting of the logged measurements will be specified

As a summary, after Meeting #127bis [6], the supported methods for logged data reporting are shown in the following table.

|  | Periodic | On-Demand | Event/Trigger-based |
| --- | --- | --- | --- |
| Logged Data Reporting | No | Yes |  |

From the table, we can see that whether event-based/triggered reporting is supported has not been decided.

However, the TR [2]  clearly says both event-based and on-demand based reporting approaches are part of the “general data collections principles”, as shown below.

A set of general data collection principles is expected to be considered for network-side model training. These include:

- UE to support data logging,

- UE to report the collected data periodically, event-based, and on-demand,

- The UE memory, processing power, energy consumption, signalling overhead should be considered.

In addition, if we looked at the data collection methods identified in R18 (Table 7.3.1.2-1 in Section 2 of this TDoc), we can see event-based reporting mechanisms is identified as one of the reporting types. For example, event-triggered reporting is identified for

- Immediate MDT: collected by TEC/OAM

- L3 measurement: collected by gNB

- LPP: collected by LMF

In fact, for some use cases, such as positioning, event-triggered reporting can be more efficient and up-to-date, because measurements only needs to be done when the UE starts to move, which saves the traffic generated by periodical reporting. In addition, allowing event-based reporting provides flexibility of control and reporting.

Based on this consideration, we propose to support event-based reporting mechanisms.

Proposal 2: Support event/trigger-based reporting mechanisms in addition to periodical and on-demand request reporting for NW-side data collection.

### SRB Selection

In RAN2 meeting #127bis [6], it was agreed that low-priority SRB will be used for NW-side data collection.  However, it was not decided whether a new SRB or SRB4 should be used.

According to the standard (TS 38.331), SRB4 is used for transmitting RRC messages containing application layer measurement reporting information on the DCCH. From the usage it was designed for, it matches the data collection purpose very well. To determine whether SRB4 can be used for data collection for NW-side model training, we should look at its limitations and, if there are any, see if these limitations will impact the performance of data collection for AI/ML functionalities/models.

The first thing to consider would be the SRB priority. Currently, while there isn't a strict, fixed priority order for SRBs in 5G NR, the general priority can be inferred from their roles and the criticality of the messages they carry, from SRB0 to SRB4. SRB4 carries RRC messages containing application layer measurement reports. While important for network optimization, it has a lower priority compared to the other SRBs, as its traffic is not as time-critical as control signaling or user data. We think the data collection for AI/ML applications should be at the same level as what SRB4 was designed for (or even higher if later on other types of AI/ML approaches are introduced, e.g., reinforcement learning). Therefore, from the priority aspect, data collection should at least use SRB4; an SRB lower than SRB4 (e.g., if we choose a new SRB5) may impact the performance of data collection.

We then look at the limitations of SRB4 and see if it is suitable for data collection for model training. SRB4 has the following limitations.

- Security Dependency: SRB4 can only be configured after security activation. This means it cannot be used for initial setup or in scenarios where security is not yet established.

- Not Applicable to NB-IoT: SRB4 is not applicable for Narrowband IoT (NB-IoT) devices, which have specific requirements and limitations.

- Resource Consumption: Using SRB4 for measurement reporting can consume additional network resources, potentially impacting overall system performance.

- Delay Sensitivity: Timely delivery of measurement reports is crucial for network optimization. Due to its lower priority, SRB4 may not be suitable for some delay-sensitive measurements.

Based on our understanding, none of these limitations will impact the performance of SRB4 for the purpose of data collection. We therefore propose to use SRB4 for data collection for NW-side model training.

Proposal 3: SRB4 can be used for data collection for NW-side model training.

## UE capability/capacity for data processing and storage

In the TR [2], the “general data collections principles” include principles regarding UE’s capability and capacity for data collection for NW-side models.

A set of general data collection principles is expected to be considered for network-side model training. These include:

- UE to support data logging,

- UE to report the collected data periodically, event-based, and on-demand,

- The UE memory, processing power, energy consumption, signalling overhead should be considered.

We consider this a practical concern because the resources at the UE side are limited, for example, the computational power and memory (the collected data will be in the memory for the BM and Positioning use cases, unlike some of the application-level use cases). We believe it is necessary to provide a mechanism to report a UE’s capability/capacity for storing measured data temporarily for NW-side model training. Logged MDT may only be used when the UE has enough available resources to support it. Otherwise, immediate MDT should be used for data collection.

Proposal 4: Design a mechanism for a UE to report its capability and/or capacity to the NW, such as computational power, available memory and storage space. NW decides on whether immediate MDT or logged MDT to be used based on the UE’s capability and capacity.

# Conclusions

In this contribution, we continued to present our observations and views on topics related to NW-side data collection. Based on the discussions in the previous sections, our proposals are as follows.

Proposal 1: Support on-demand data logging and enhancement to logged MDT for data collection for NW-side models.

Proposal 2: Support event/trigger-based reporting mechanisms in addition to periodical and on-demand request reporting for NW-side data collection.

Proposal 3: SRB4 can be used for data collection for NW-side model training.

Proposal 4: Design a mechanism for a UE to report its capability and/or capacity to the NW, such as computational power, available memory and storage space. NW decides on whether immediate MDT or logged MDT to be used based on the UE’s capability and capacity.

# References

RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN meeting #102, Qualcomm (moderator)

RP-233946, 3GPP TR 38.843, Technical Specification Group Radio Access Network; Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18) v2.0.1 (2023-12)

Chair’s notes, R2_125bis_ChairNotes_24-04-19_final, RAN2 meeting #125bis, Changsha, April 2004

Chair’s notes, R2_126_ChairNotes_24-05-24_final, RAN2 meeting #126, Fukuoka, May 2024

Chair’s notes, R2_127_ChairNotes_ 24-08-23_17-00_eom, RAN2 meeting #127, Maastricht, Aug 2024

Chair’s notes, R2_127bis_ChairNotes_ 24-10-18_15-45-_eom_clean, RAN2 meeting #127bis, Hefei, Oct 2024