3GPP TSG-RAN WG2#128	     					      R2-2410604

Orlando, USA, November 18 – 22, 2024

Agenda item:	8.1.2.2

Source: 	Kyocera

Title: 	Considerations on LCM for UE-sided model in Beam Management use case

Document for:	Discussion

Introduction

In this contribution, the remaining issues regarding LCM of the UE-sided model for the Beam Management use case, functionality applicability reporting, and non-applicable functionality reporting are discussed.

Discussion

The Remaining Issues

Signaling of Functionality Applicability Report

In RAN2#127bis, the consensus was reached on the following points [1]; however, some issues regarding the applicability reporting content and the inference configuration in step 3 remain.

Agreements (RAN2#127bis) UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality. We should aim to align the design on how the applicable functionality are signaled. FFS on the applicability reporting content. FFS if inference configuration can be signalled in step3.

The Applicability Reporting Content

The applicability reporting are assumed to contain the functionality ID. However, its granularity is not clear as of now. RAN1 agreed as follows in RAN1#118bis [2].

Agreements (RAN1#118bis) Agreement Incorporating below text into the general part of the LS In RAN1’s discussion of RAN 2 terminologies on beam management, - The concept/terminology “functionality“ of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-specific FGs - The concept/terminology “ functionality“ of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters or information/parameters indicated by UE - The Activated functionalities may be enabled based on CSI framework.

According to the RAN1 agreement, Applicable functionality does not simply refer to the ‘functions’, but it’s rather a concept associated with multiple pieces of information or parameters, such as CSI-ReportConfig, the inference-related parameters or the information/parameters indicated by UE. Therefore, in cases where a specific functionality corresponds one-to-one with a specific CSI-ReportConfig, it’s conceivable that the functionality might be treated similarly to a CSI-ReportConfigId.

Alternatively, it would be considered that a functionality may be defined more broadly as a "set of functions or components," rather than just a single ID. In this case, the functionality may not need to correspond exactly with a specific ID, such as a CSI-ReportConfigId, but instead it may refer to the entire set of configurations and associated information encompassed by that ID.

Since the other aspects, i.e., ‘a set of inference related parameters’ and ‘information/parameters indicated by UE’, are unclear, by focusing on the CSI-ReportConfig for inference configuration, it’s assumed that it may have up to 48 functionality IDs, as same as CSI-ReportConfigId, i.e., maxNrofCSI-ReportConfigurations = 48. Therefore, the applicability reporting may contain at least the functionality IDs, while the details are left FFS and pending until the RAN1 progress.

The CSI-ReportConfigId IE and the maxNrofCSI-ReportConfigurations value in TS 38.331 are shown below.

CSI-ReportConfigId information element -- ASN1START -- TAG-CSI-REPORTCONFIGID-START CSI-ReportConfigId ::= INTEGER (0..maxNrofCSI-ReportConfigurations-1) -- TAG-CSI-REPORTCONFIGID-STOP -- ASN1STOP

6.4 RRC multiplicity and type constraint values – Multiplicity and type constraint definitions […] maxNrofCSI-ReportConfigurations INTEGER ::= 48 -- Maximum number of report configurations […]

Based on the above considering, it’s understood that Applicable functionalities are associated with corresponding configurations or certain parameters. By introducing IDs for each functionality, it’s possible to associate the ID with the specific configurations or parameters, enabling more efficient management of Applicable functionalities. Further details regarding this association are expected to be discussed in RAN1. So, from RAN2’s perspective, the following is proposed:

RAN2 should agree that the applicability reporting content must include at least functionality IDs. The exact details of this inclusion remain for further study in RAN1.

Inference Configuration in Step 3

For the FFS related to step-3, the following three options in the RAN1 agreement at RAN1#118bis [2] should be reviewed.

Agreement RAN 1 further study the following options for applicability for inference for UE-side model: Step; Option 1; Option 2; Option 3. Step 3; - In Step 3, following configurations are provided from NW to UE - UE is allowed to do UAI reporting via OtherConfig, - 2)+3) NW configures one or more CSI-ReportConfig for inference configuration, where the associated ID may be configured in CSI framework as working assumption applied. - FFS on whether some IEs in the CSI report configuration can be removed or modified - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3; - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - NW configures one set or multiple sets of inference related parameters - Note: the set of inference related parameters is not configured by CSI-ReportConfig - FFS on the set of inference related parameters, at least including: - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - The associated ID(s) may be configured - wherein the associated ID(s) may be - FFS: a) part of one set of the inference related parameters, or - FFS: b) independently from the one set of the inference related parameters.; - In Step 3, following configurations are provided from NW to UE: - 1) UE is allowed to do UAI reporting via OtherConfig, - 2) The associated ID(s) may be provided to UE, e.g., a new RRC parameter.. Step 4; - In Step 4, UE reports applicability(ies) of the above CSI-ReportConfig - FFS on one or more of the above CSI-ReportConfig to be reported; - In Step 4, UE reports applicability of the above one or multiple sets of inference related parameters, where the associated ID information may be associated.; - In Step 4, UE reports by UAI - the applicable one or multiple sets of inference related parameters may be included. - FFS on the set of inference related parameters, at least including: - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - Note: not applicable may also be replied by UE - Note: if the inference related parameters are not supported for reporting, only the applicability(ies) or not is reported in Step 4. - the associated ID(s) may be included - FFS: a) as part of the inference related parameters, or - FFS: b) independently from the set of the inference related parameters.. Step 5; -; - In Step 5, NW configures configuration(s) for CSI report for inference; In Step 5, NW configures configuration(s) for CSI report for inference.. FFS; - FFS on activation (including when/how) of inference report after obtaining the applicability from UE Step 4 - FFS: whether Step 5 is needed,.

According to the above agreement, it’s assumed that Option 3 might not provide any inference configuration in step 3. Therefore, it depends on the options whether the inference configuration is signaled in step 3. So, RAN2 should wait for RAN1’s progress.

RAN2 should wait for RAN1’s results to determine whether an inference configuration will be signaled in step 3.

Regarding Non-applicable Functionalities

Signaling of Non-Applicable Reporting (e.g., explicitly/implicitly)

In RAN2#127bis, RAN2 reached the following agreement on how the non-applicable functionality is reported [1].

Agreements (RAN2#127bis) UE can report to the network when an applicable AI functionality becomes non-applicable. FFS how this is signaled (e.g. explicitly/implicitly). Consider different scenarios, whether it is regarding an active functionality)

It was discussed how to report functionality that becomes non-applicable, either explicitly or implicitly. The explicit reporting contains each functionality that becomes non-applicable, whereas the implicit reporting only contains the functionalities that are currently applicable, i.e., the non-applicable functionalities are not reported explicitly.

Generally, the implicit reporting does not preclude the ability to convey additional information, such as reasons for functionalities becoming non-applicable. However, it can increase complexity. In contrast, the explicit reporting makes it easier to convey such supplementary information. Therefore, the critical factor may not be whether to use implicit or explicit reporting, but rather whether there is a need to communicate supplementary information.

The implicit reporting can convey reasons for non-applicability but adds complexity, while the explicit reporting simplifies this process.

RAN2 should first discuss whether supplementary information can be added to the report before considering additional reporting options for specific, detailed information, such as reasons.

Information on functionalities transitioning from applicable to non-applicable status may include reasons such as overheating, battery constraints, CPU resource limitations, or specific conditions affecting both the UE and NW sides. Additionally, information on how long the functionality is expected to remain non-applicable may also be relevant. These details can help the network side understand both the reasons and duration of the non-applicable status, making this supplementary information potentially valuable. However, the value of this information depends on specific operational needs and objectives, so it should be considered an optional component based on situational requirements.

Supplementary information, such as the reason for reporting non-applicable functionality, is useful for LCM.

RAN2 should agree that supplementary information, such as the reason for reporting non-applicable functionality, can be optional.

Based on Observation 1, if non-applicable reporting contains supplementary information, explicit reporting can be used; otherwise, implicit reporting can be used.

RAN2 should agree that if non-applicable reporting includes supplementary information, explicit reporting can be used; otherwise, implicit reporting can be used.

Conclusion

In this contribution, we discuss the issues in terms of LCM of AI/ML for NR air Interface. RAN2 is kindly asked to take into account the proposals below:

Proposal 1	RAN2 should agree that the applicability reporting content must include at least functionality IDs. The exact details of this inclusion remain for further study in RAN1.

Proposal 2	RAN2 should wait for RAN1’s results to determine whether an inference configuration will be signaled in step 3.

Observation 1	The implicit reporting can convey reasons for non-applicability but adds complexity, while the explicit reporting simplifies this process.

Proposal 3	RAN2 should first discuss whether supplementary information can be added to the report before considering additional reporting options for specific, detailed information, such as reasons.

Observation 2	Supplementary information, such as the reason for reporting non-applicable functionality, is useful for LCM.

Proposal 4	RAN2 should agree that supplementary information, such as the reason for reporting non-applicable functionality, can be optional.

Proposal 5	RAN2 should agree that if non-applicable reporting includes supplementary information, explicit reporting can be used; otherwise, implicit reporting can be used.

References

- “RAN2 Chair’s Notes”, (InterDigital), RAN2#127bis

- “RAN1 Chair’s Notes”, (CMCC), RAN1#118bis