3GPP TSG RAN2 Meeting #128                                                                R2-2410606

18th Nov. – 22nd Nov. 2024, Orlando, USA

Agenda item:	8.4.2

Source:	Samsung

Title:	Procedure and Configuration of LP-WUS in RRC Idle /
Inactive Mode

Document for:	Discussion & Decision

Introduction

Energy conservation has emerged as an important issue in recent mobile communication network environments. Accordingly, RAN2 starts with the new Work Item (WI), Low-power wake-up signal and receiver for NR (LP-WUS/WUR) [1]. And during the previous meeting #127, with respect to procedure and configuration of LP-WUS in RRC idle/inactive mode agenda, RAN2 agreed as follows:

LP-WUS entry/exit condition If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s). The LPWUS monitoring exit condition does not include MR measurements. Subgrouping For CN assigned LP-WUS subgrouping, RAN2 assumes similar procedure for PEI will be used for LP-WUS subgrouping. Final design is up to SA2/CT1/RAN3 discussion. For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping. RAN2 inform this conclusion to SA2/CT1/RAN3. This LS is approved in R2-2409226

And RAN1 reached the following agreements regarding procedures and configurations for idle/inactive modes during meeting #118bis[2] :

Working Assumption From RAN1 perspective, for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode, - The UE may start LP-WUS monitoring if - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - If UE starts LP-WUS monitoring, it may stop the legacy PO monitoring before UE receives LP-WUS indicating wake-up - The UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - FFS the serving cell measurement metrics - The entry/exit thresholds can be configured separately for different types of LR - It is left to RAN2 discussion whether the threshold(s) are always configured by the gNB. - Note: This may be revisited based on the RAN2/RAN4 discussion. Agreement For the offset value(s) between an LO and a reference PO/PF, consider the following options: - Definition: The gap between an LO and a PO is considered to be no less than the wake-up delay a UE supports if the gap between the end of the last LP-WUS MO the UE monitors in the LO and the start of the PO is no less than the wake-up delay. - Option 1: gNB configures a single offset value. - If the gap between an LO and the PO associated with the offset is no less than the wake-up delay a UE supports, the UE monitors the PO associated with the offset after receiving a wake-up indication in a LP-WUS. - Otherwise, - Option 1-1: the UE follows the legacy paging monitoring procedure. - Option 1-2: the UE monitors LP-WUS. If it receives a wake-up indication in a LP-WUS, it monitors the first PO after its reported wake-up delay. - Option 2: gNB configures one or multiple offset values. - For the same PO, each offset corresponds to a LO. - This does not preclude the possibility that the same LO may correspond to different POs with different offset values. - Option 2A: A UE does not expect that the gap between the LO associated with the largest offset and the corresponding PO is less than the wake-up delay the UE supports. The UE monitors the LO associated with one offset that has a gap between the LO and the corresponding PO no less than the wake-up delay. - This implies that the gNB needs to configure at least one offset value that is no less than the largest wake-up delay supported by the UEs. - FFS exactly how to choose the offset - Option 2B: - If the gap between the LO associated with the largest offset and the corresponding PO is no less than the wake-up delay a UE supports, the UE monitors the LO associated with one offset that has a gap between the LO and the PO associated with the offset no less than the wake-up delay. - FFS exactly how to choose the offset - Otherwise, - Option 2B-1: the UE follows the legacy paging monitoring procedure. - Option 2B-2: the UE monitors LP-WUS. If it receives a wake-up indication in a LP-WUS, it monitors the first PO after its reported wake-up delay. - FFS exactly how to choose the offset - FFS the UE shall monitor the LO associated with additional offset(s) Note: The PO mentioned above refers to legacy PO configured for the UE. Agreement Each LP-WUS is QCLed with one SSB. Each LP-SS is QCLed with one SSB. - FFS QCL Type A or Type C and/or Type D - FFS implicit QCL determination or some signaling is required Agreement The number of beams for LP-SS is the same as the number of beams for the LP-WUS MOs in an LO. Working Assumption The maximum number of subgroups per PO supported in Rel-19 is 32.

In this contribution, we would like to discuss the detailed procedures and configurations to support LP-WUS in RRC IDLE/INACTIVE mode.

Discussion

Based on the findings from the LP-WUS/WUR study conducted during the Study Item (SI) phase in Release 18, it has been proposed that LP-WUS/WUR should be implemented as a power saving mechanism specifically designed for RRC_IDLE and RRC_INACTIVE mode devices. Furthermore, the concept of an "ultra-deep sleep state" has emerged as a potential power saving strategy for both RRC_IDLE and RRC_INACTIVE modes. By leveraging LP-WUS/WUR, a user equipment's (UE) Main Receiver (MR) can effectively transit into an ultra-deep sleep state, thereby achieving significant power savings. While in this ultra-deep sleep state, the UE can use its LP-WUS/WUR functionality to continuously monitor network signaling activities, waking up the MR when necessary.

- Entry/Exit condition

According to the previous RAN2 discussion, now we have the entry/exit condition(s) of LP-WUS monitoring configured in SIB (e.g., SIB1).

Since with the agreed entry condition and with the NR measurement already ruled out from the exit condition, now the remained discussion is with respect to exit condition of LP-WUS monitoring, which is the serving cell measurement result based on LR below a configured threshold.

Proposal 1. The exit condition of LP-WUS monitoring is met when the LR serving cell measurement result is below the configured threshold.

- Subgrouping

To further enhance the energy saving gain, RAN2 agreed to take PEI subgrouping method as baseline for LP-WUS subgrouping, i.e. CN assigned and UE_ID based subgrouping.

For UEs in RRC idle/inactive state, LP-WUS can either replace PEI (LP-WUS → PO) or coexist with PEI (LP-WUS → PEI → PO). Thus, in the above two different cases, subgrouping of LP-WUS can limit the monitoring of PF/PO, or PEI occasions, respectively.

- Scenario 1) When LP-WUS replaces PEI and subgroups are applied on LP-WUS, it reduces the number of PF/PO that UEs need to monitor.

- Scenario 2) When both LP-WUS and PEI exist and subgroups are applied on LP-WUS and PEI, it reduces the number of PEI occasions/ PF/PO that UEs need to monitor.

For Scenario 1, where UE monitors PO upon receiving LPWUS signal, UEs monitoring a PO can be divided into several LPWUS subgroups based on UE_ID.  LP WUS subgroup ID based on UE_ID is determined as follows:

LP WUS subgroup ID = (floor (UE_ID/(N*Ns)) mod LPWUSsubgroupsNumForUEID) +   
                  (LPWUSsubgroupsNumPerPO - LPWUSsubgroupsNumForUEID)

- LPWUSsubgroupsNumForUEID: number of LPWUS subgroups for UE_ID based LPWUS subgrouping in a PO, which is broadcasted in system information

- LPWUSsubgroupsNumPerPO: number of LPWUS subgroups per PO, which is broadcasted in system information

For Scenario 2, where UE first monitors PEI upon receiving LPWUS signal and then monitors PO (if indicated by PEI).  LP WUS subgroup ID based on UE_ID can be determined independently from paging subgroup ID used for PEI as follows:

LP WUS subgroup ID = (floor (UE_ID/(N*Ns)) mod LPWUSsubgroupsNumForUEID) +  
(LPWUSsubgroupsNumPerPO - LPWUSsubgroupsNumForUEID)

- LPWUSsubgroupsNumForUEID: number of LPWUS subgroups for UE_ID based LPWUS subgrouping in a PO, which is broadcasted in system information

- LPWUSsubgroupsNumPerPO: number of LPWUS subgroups per PO, which is broadcasted in system information

Some companies have proposed to consider PEI subgrouping while determining the LPWUS subgroup ID. In our view this cannot reduce false alarms or triggers to wakeup MR. It may reduce PDCCH monitoring in PF/PO after receiving PEI. However, when LP WUS is configured, power savings from reducing PDCCH monitoring in PF/PO after receiving PEI is not significant compared to power saving from MR deep sleep/LP WUS monitoring.  ’.

Proposal 2. LP WUS subgroup ID based on UE_ID is determined as follows:

LP WUS subgroup ID = (floor (UE_ID/(N*Ns)) mod LPWUSsubgroupsNumForUEID) +               (LPWUSsubgroupsNumPerPO - LPWUSsubgroupsNumForUEID)

- LPWUSsubgroupsNumForUEID: number of LPWUS subgroups for UE_ID based LPWUS subgrouping in a PO, which is broadcasted in system information

- LPWUSsubgroupsNumPerPO: number of LPWUS subgroups per PO, which is broadcasted in system information

- LS from RAN1: UE not supporting LP-WUS on all the bands

RAN2 received the LS from RAN1[3] as follows:

In RAN1, the common understanding is that UE may not support LP-WUS reception on all the bands supported by the UE. Request RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs.

So far it has been assumed that UE supports LP-WUS reception on all the bands supported by the MR i.e. no difference of each supported band between MR and LR from UE perspectives. Hence, before delving into the details on any issue and specification support, we should be very cautious what needs to be addressed in this release according to the RAN1 LS.

During RAN1 discussion, some companies mentioned that LR and MR (serving cell) can be operated under different bands i.e. operating LP-WUS with lower frequency band (e.g. 700MHz) and MR operating frequency band (e.g. 2.6GHz) for better coverage of LP-WUS/LP-SS. We see non-trivial issues to support it i.e. the feasibility to support the RRM relaxation and serving cell measurement offloading feature, how to define entry/exit conditions due to mismatch between LR and MR measurement etc. We think that supporting LR and MR on different carriers should be out of scope.

Proposal 3. From RAN2 perspective, supporting LR and MR on different carriers is not essential, i.e. LR shall be operated on the same carrier/band as serving cell.

Network may not support LP-WUS/LP-SS on all the bands supported by the network. Similarly, it is not desirable to mandate UE to support LP-WUS reception on all the bands supported by the UE. We do not see any critical issue for this.

Proposal 4. The supported bands on LR can be a subset of UE supported bands on MR.

Some specification efforts are required to support LP-WUR feature as subset of UE supported bands on MR. For example, separate UE capability per band granularity may be required to indicate LP-WUS feature. Without separate UE capability, network has a limited control to load balance IDLE/INACTIVE UEs via dedicated signalling i.e.

- For re-direction case, network has to blindly redirect LP-WUS reception capable UEs to perform cell selection on the particular band with no idea whether LP-WUS reception is supported by the UE there.

- For cell-reselection case, dedicated frequency priorities are to be provided such that LP-WUS reception capable UEs may support LP-WUS reception on (part of) frequencies when network supports LP-WUS there.

Observation 1. Per band UE capabilities may be required for UEs with limited LP-WUS supported bands.

Also, some enhancement on cell barring mechanism and cell reselection priorities in system information could be considered in order to load balance LP-WUS reception capable UEs and LP-WUS reception non-capable UEs properly.

Observation 2. RAN2 could enhance the cell barring mechanism and reselection priorities handling for UEs with limited LP-WUS supported bands.

.

- 3. Conclusion

RAN2 is requested to discuss and agree to the following proposals:

Proposal 1. The exit condition of LP-WUS monitoring is met when the LR measurement result is below the configured threshold.

Proposal 2. LP WUS subgroup ID based on UE_ID is determined as follows:

LP WUS subgroup ID = (floor (UE_ID/(N*Ns)) mod LPWUSsubgroupsNumForUEID) +               (LPWUSsubgroupsNumPerPO - LPWUSsubgroupsNumForUEID)

- LPWUSsubgroupsNumForUEID: number of LPWUS subgroups for UE_ID based LPWUS subgrouping in a PO, which is broadcasted in system information

- LPWUSsubgroupsNumPerPO: number of LPWUS subgroups per PO, which is broadcasted in system information

Proposal 3. From RAN2 perspective, supporting LR and MR on different carriers is not essential, i.e. LR shall be operated on the same carrier/band as serving cell.

Proposal 4. The supported bands on LR can be a subset of UE supported bands on MR.

Observation 1. Per band UE capabilities may be required for UEs with limited LP-WUS supported bands.

Observation 2. RAN2 could enhance the cell barring mechanism and reselection priorities handling for UEs with limited LP-WUS supported bands.

- References

[1] RP-234056 Low-power wake-up signal and receiver for NR (LP-WUS/WUR), CMCC

[2] Draft_Minutes_report_RAN1#118bis_v020, Draft Report of 3GPP TSG RAN WG1 #118bis v0.2.0, RAN1

[3] R2-2407921, LS on LP-WUS operation in IDLE/INACTIVE mode (R1-2407559; contact: Apple), Apple, RAN1