3GPP TSG RAN2 Meeting #128                                                                R2-2410607

18th Nov. – 22nd Nov. 2024, Orlando, USA

Agenda item:	8.4.3

Source:	Samsung

Title:	RRM measurement relaxation and offloading in RRC Idle /
Inactive Mode

Document for:	Discussion & Decision

Introduction

Energy conservation has emerged as an important issue in recent mobile communication network environments. Accordingly, RAN2 starts with the new Work Item (WI), Low-power wake-up signal and receiver for NR (LP-WUS/WUR) [1].

Among the WID[1], objectives related with RAN2 RRC IDLE/INACTIVE mode are as follows:

For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

And during the previous meeting #127, with respect to RRM measurement relaxation and offloading agenda, RAN2 agreed as follows:

General aspects RAN2 only discuss RRM measurement offloading/relaxation for LP-WUS UEs. Serving cell measurment offloading (i.e., there is no serving cell measurement by MR) For serving cell measurement offloading (i.e., there is no serving cell measurement by MR): - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered. - The exit condition is based on the LR measurement results.

In this contribution, we would like to discuss the detailed RRM measurement relaxation and offloading methods to support LP-WUS in RRC IDLE/INACTIVE mode.

Discussion

A UE in RRC_IDLE or RRC_INACTIVE state needs to perform mobility measurements as defined in TS 38.304 to camp on the optimal cell. Consequently, for the majority of UEs, the serving cell measurements require the most frequent execution, thus having the greatest influence on UE energy consumption.

Using the MR for serving cell measurements has a significant impact on UE energy consumption and reduces the LP-WUS/WUR gain according to the observations from the study evaluation results. In order to achieve the UE power saving gain by LP-WUS/WUR, the RRM measurement on serving cell and neighboring cell via MR can be relaxed or may be stopped when UE is using LP-WUS and MR is in ultra-deep sleep. However, it should be noted that these RRM relaxations may affect the mobility performance in RRC_IDLE or RRC_INACTIVE states.

RRM relaxation is a key feature to reduce NR UE power consumption, implemented through less frequent measurement of serving and neighbor cell qualities, such as Reference Signal Received Power (RSRP), Reference Signal Received Quality (RSRQ), and Signal-to-Interference-and-Noise Rate (SINR). RRM relaxation can be initiated through RRM relaxation triggers, and in Release 16, two types of triggers were specified: low mobility criteria and not at cell edge criteria. The subjective term 'low mobility' is utilized to define the mobility level of a UE with the preconfigured specific values like RSRPSearchDeltaP and TSearchDeltaP.  The subjective term 'not at cell edge' is utilized to define the channel quality of a UE with the preconfigured threshold, RSRPSearchThresholdP. When a condition (or both conditions) are satisfied, the terminal may alleviate RRM measurement by performing RRM once every multiple of a pre-configured DRX cycle, or even stop RRM measurement.

Serving cell measurement offloading

As in previous RAN2 agreements and TR [2], NR may support MR RRM measurement relaxation on the serving cell and/or neighbor cell in case when the UE is in ultra-deep sleep, provided that RRM measurement on LR is feasible/supported. And now RAN2 defined the ‘Serving cell measurement offloading’ as follows:

Serving cell measurement offloading

: serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required

If a UE supports the serving cell measurement offloading, the serving cell quality could be continuously measured using LP-WUR via LR-SS.

Considering the use cases of serving cell measurement offloading, the triggering criteria does not need to differ from the current criteria of NR RRM measurement relaxation. Therefore, we may either reuse the MR measurement-based criteria or even introduce LR measurement-based new criteria for serving cell measurement offloading triggering conditions such as:

- MR low mobility and/or not at cell edge criteria

: Based on the MR measurement results, the UE may perform MR serving cell measurement offloading.

- LR low mobility and/or not at cell edge criteria

: Based on the LR measurement results, the UE may perform MR serving cell measurement offloading.

Proposal 1. For the serving cell measurement full offloading for UEs capable of LP-WUS, in addition to ‘MR greater than a certain RSRP threshold’,

    - The MR-based ‘low mobility’ criteria is supported.

Neighbor cell RRM measurement relaxation

For neighboring cell MR RRM measurement relaxation, if the UE supports the LR RRM measurement and the serving cell measurement is offloaded, it would be reasonable to also relax the neighboring cell MR RRM measurement. Otherwise, the UE may not benefit from energy savings. Therefore, RAN2 has agreed to support NR serving cell MR RRM measurement relaxation if the UE supports LP-WUR RRM measurement offloading.

With the previous RAN2 agreement, we now have the remained criteria to be discussed in RAN2, as follows:

Proposal 2. For neighbor cell MR measurement relaxation for UEs capable of LP-WUS, in addition to MR ‘UE not at cell edge’ criterion,

    - The MR-based ‘low mobility’ criterion is also supported which is the same as Rel16.

    - The LR-based ‘enhanced’ criteria are supported, e.g., ‘UE not at cell edge’ and ‘low mobility’.

In addition, if any of the exit conditions of LP-WUS monitoring (discussed in Agenda item 8.4.2) are met, UE would activate MR and perform MR transmission/reception. Hence, the UE should deactivate the ongoing RRM measurement offlading/ relaxation if any of the exit measurements occur.

- MR RRM measurement offloading/ relaxation should be halted if an exit condition of LP-WUS monitoring is met.

Unlike the exit condition, the entry condition is somewhat challenging since the entry condition itself would not ensure the 'low mobility' or 'not at cell edge' of the UE that can relax MR RRM measurements.

Proposal 3. The MR neighbor cell RRM measurement relaxation should be stopped if any of the exit conditions of LP-WUS monitoring are met.

If the entry condition is met, UE monitors LP-WUS using LR. Even though UE switches to LR and monitors LPWUS, UE have to regularly wakeup the MR for inter frequency measurements wherein the UE measures the SSBs transmitted on neighboring frequency for cell (re-)selection.  Note that LR cannot be used neighbor cell measurements. Waking up MR and again switching to LR after each measurement leads to increased power consumption. Additionally, long time to wake up MR and switching to LR may lead to UE missing paging indication.

So, we propose that if criteria for inter frequency/inter RAT frequency measurement is met, UE does not monitor LP-WUS. If UE is already monitoring LP-WUS, it stops monitoring LP-WUS and wakes up the MR and monitor PEI/PO for paging and also perform inter frequency/inter RAT frequency measurement using the MR.

Proposal 4. If criteria for inter frequency/inter RAT frequency measurement is met, UE does not monitor LPWUS. If UE is already monitoring LPWUS, it stops monitoring LPWUS and wakes up the MR and monitor PEI/PO for paging and also perform inter frequency/inter RAT frequency measurement using the MR.

Conclusion

RAN2 is requested to discuss and agree to the following proposals:

Proposal 1. For the serving cell measurement full offloading for UEs capable of LP-WUS, in addition to ‘MR greater than a certain RSRP threshold’,

    - The MR-based ‘low mobility’ criteria is supported.

Proposal 2. For neighbor cell MR measurement relaxation for UEs capable of LP-WUS, in addition to MR ‘UE not at cell edge’ criterion,

    - The MR-based ‘low mobility’ criterion is also supported which is the same as Rel16.

    - The LR-based ‘enhanced’ criteria are supported, e.g., ‘UE not at cell edge’ and ‘low mobility’.

Proposal 3. The MR neighbor cell RRM measurement relaxation should be stopped if any of the exit conditions of LP-WUS monitoring are met.

Proposal 4. If criteria for inter frequency/inter RAT frequency measurement is met, UE does not monitor LP-WUS. If UE is already monitoring LP-WUS, it stops monitoring LPWUS and wakes up the MR and monitor PEI/PO for paging and also perform inter frequency/inter RAT frequency measurement using the MR.

- References

[1] RP-234056 Low-power wake-up signal and receiver for NR (LP-WUS/WUR), CMCC

[2] 3GPP TR38.869 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on low-power wake up signal and receiver for NR (Release 18)