3GPP TSG RAN2 Meeting #128                                                                R2-2410608

18th Nov. – 22nd Nov. 2024, Orlando, USA

Agenda item:	8.4.4

Source:	Samsung

Title:	Procedures for LP-WUS in RRC Connected Mode

Document for:	Discussion & Decision

Introduction

Energy conservation has emerged as an important issue in recent mobile communication network environments. Accordingly, RAN2 starts with the new Work Item (WI), Low-power wake-up signal and receiver for NR (LP-WUS/WUR) [1][2].

During the previous meeting #127bis, with respect to Procedures for LP-WUS in RRC Connected Mode agenda, RAN2 agreed as follows:

Option 1-2 related For Option 1-2, LP-WUS monitoring is performed at least outside legacy C-DRX Active Time. FFS if the legacy drx-onDurationTimer is started or not if the new timer is configured in Option 1-2. In option 1-2, a new timer triggered by LPWUS is introduced. When this new timer is running, UE is in C-DRX active time. When UE is not in C-DRX active time, UE goes back to LPWUS monitoring. When UE is in C-DRX active time, UE PDCCH monitoring behaviors related to other legacy DRX timers (except for drx-onDurationTimer) are not affected.

And, RAN1 made the below related summary at the meeting #118bis w.r.t. various LP-WUS procedure options[3]:

Agreement For Option 1-1 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective. - LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer - UE is configured with legacy C-DRX configurations as Rel-18 - UE behaviors related to CDRX active time triggered by all legacy DRX timers are not affected unless included in this proposal - No impact on RRM/RLM/BFD measurement requirements is assumed - For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP) - Periodic CSI/L1-RSRP is not reported during the time given by the configured drx-onDurationTimer if UE is not indicated to wake-up - Periodic CSI/L1-RSRP is periodically reported during the time given by the configured drx-onDurationTimer regardless if UE is indicated to wake-up or not - Note: Periodic CSI/L1-RSRP is reported during CDRX active time as in legacy specification Agreement Confirm following working assumption Working Assumption From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE Note: Above can be revisited considering RAN2 decisions. Agreement - For LP-WUS CONNECTED mode operation, Option 1-2 can be configured separately from Option 1-1 - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE Agreement - Regarding the potential restriction for LP-WUS configuration in relation with C-DRX configuration for option 1-2 of LP-WUS CONNECTED mode operation, from RAN1 perspective, - AltOpt2: At least LP-WUS periodicity is no larger than long C-DRX cycle, FFS other restrictions - Introducing monitoring window is not precluded

In this contribution, we would like to discuss the detailed procedures and configurations to support LP-WUS in RRC IDLE/INACTIVE mode.

Discussion

Based on the discussions that have been carried out in RAN1 and RAN2, it has become apparent that the main and most significant use case for LP-WUS in the RRC Connected mode will revolve around  'PDCCH monitoring which is triggered by LP-WUS in conjunction with the C-DRX configuration'.

For a quick summary, we capture the each options examples from RAN1’s discussion [4]:

Option 1-1: Replacing DCP following legacy configurations

Figure 1: LP-WUS monitor procedure in Option 1-1

Option 1-2: UE performs LP-WUS monitoring outside at least legacy C-DRX active time, and PDCCH monitoring may be additionally triggered

Figure 2: LP-WUS monitor procedure in Option 1-2 with ‘duty-cycled’ LP-WUS

Figure 3: LP-WUS monitor procedure in Option 1-2 with ‘continuous’ LP-WUS

Based on the discussions from RAN2#127bis, RAN2 progressed and established the precise PDCCH monitoring timing as follows:

Observation 1. RAN2 defines when the UE starts monitor PDCCH as follows:

- Option 1-1: If LP-WUS addressed to UE or UE's subgroup is detected and the legacy drx-onDurationTimer is started

- Option 1-2: If LP-WUS addressed to UE or UE's subgroup is detected and the new timer is started. FFS on drx-onDurationTimer.

The remaining question is whether this timer is newly configured or utilizes an existing traditional timer such as drx-onDurationTimer. It is noteworthy that if the abovementioned timer for Option 1-2 corresponds to drx-onDurationTimer and the LP-WUS periodicity is comparable to the DRX cycle, then Option 1-2 might be equivalent to Option 1-1.

Observation 2. If the above timer for Option 1-2 is the same as drx-onDurationTimer and the LP-WUS periodicity is similar as DRX cycle, the Option 1-2 could be the same as Option 1-1

In the meantime, when the LP-WUS enabled UE is configured with both Option 1-2 and legacy DRX, then the UE operating with LP-WUS option 1-2 is not required to monitor the PDCCH during the legacy DRX active time.

Proposal 1. When the LP-WUS enabled UE is configured with both Option 1-2 and legacy DRX, then the UE operating with LP-WUS option 1-2 is not required to monitor the PDCCH during the legacy DRX active time.

Furthermore, if Proposal 1 is accepted, we need to discuss the appropriate UE behavior with respect to drx-onDurationTimer, which might be activated w.r.t. DRX cycle and whether the affected behaviours are there or not.

Proposal 2. When the LP-WUS enabled UE is configured with both Option 1-2 and legacy DRX, then the drx-onDurationTimer is activated/deactivated based on the legacy DRX configuration. And UE behaviors w.r.t drx-onDurationTimer (except for PDCCH monitoring) are not affected.

- 3 Conclusion

RAN2 is requested to discuss and agree to the following proposals:

Observation 1. RAN2 defines when the UE starts monitor PDCCH as follows:

- Option 1-1: If LP-WUS addressed to UE or UE's subgroup is detected and the legacy drx-onDurationTimer is started

- Option 1-2: If LP-WUS addressed to UE or UE's subgroup is detected and the new timer is started. FFS on drx-onDurationTimer.

Observation 2. If the above timer for Option 1-2 is the same as drx-onDurationTimer and the LP-WUS periodicity is similar as DRX cycle, the Option 1-2 could be the same as Option 1-1

Proposal 1. When the LP-WUS enabled UE is configured with both Option 1-2 and legacy DRX, then the UE operating with LP-WUS option 1-2 is not required to monitor the PDCCH during the legacy DRX active time.

Proposal 2. When the LP-WUS enabled UE is configured with both Option 1-2 and legacy DRX, then the drx-onDurationTimer is activated/deactivated based on the legacy DRX configuration. And UE behaviors w.r.t drx-onDurationTimer (except for PDCCH monitoring) are not affected.

- References

[1] RP-234056 Low-power wake-up signal and receiver for NR (LP-WUS/WUR), CMCC

[2] 3GPP TR38.869 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on low-power wake up signal and receiver for NR (Release 18)

[3] Draft_Minutes_report_RAN1#118bis_v020, Draft Report of 3GPP TSG RAN WG1 #118bis v0.2.0, RAN1

[4] R1-2407378 FL summary #2 on LP-WUS operation in CONNECTED mode, NTT DOCOMO, RAN1