3GPP TSG RAN2 Meeting #128                                                                R2-2410609

18th Nov. – 22nd Nov. 2024, Orlando, USA

Agenda item:	8.11.2

Source:	Samsung

Title:	Random access in SBFD

Document for:	Discussion & Decision

Introduction

The Rel-19 WID Evolution of Duplex operation introduces NR specification support for gNB-side SBFD within a TDD carrier, enhancements to inter-gNB and inter-UE CLI handling, and BS RF and RRM requirements [1].

In the Rel-19 WID, it is assumed that SBFD is deployed at the gNB side and that the TDD UE operates in half-duplex. Subband full-duplex (SBFD) operation on a time-domain resource is limited to FDM in non-overlapping DL and UL subbands. SBFD enhancements for SBFD-aware UEs assume SBFD operation for NR single carrier in FR1 or FR2-1. From the perspective of the SBFD-aware UE, SBFD operation is assumed within a single configured DL and UL BWP pair with aligned center frequencies. For the semi-static indication of frequency/time locations of the SBFD subbands to SBFD-aware UEs on a legacy DL or F symbol/slot, it is assumed that one UL subband is configured for SBFD operation in an SBFD symbol.

The Rel-19 WID objectives include introduction of specification support for random access in SBFD symbols at least for RRC_CONNECTED mode and a study phase for RRC_IDLE/INACTIVE modes [1].

- Specify SBFD operation to support random access in SBFD symbols by UEs in RRC CONNECTED mode [RAN1, RAN2]

- Study and specify, if justified, SBFD operation to UE in RRC_IDLE/INACTIVE mode for random access [RAN1, RAN2]

  - RAN#104 to check whether to proceed normative work

During the previous meeting #127bis, RAN2 agreed as follows:

Early indication RAN2 understand that if additional RO is selected by SBFD-aware UE, early identification via Msg1 is possible from NW point of view for this UE without specification impact. From R2 point of view, there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework. RACH resource selection Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication (FFS if there needs to be any) is from network. For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first. RACH configuration The following two RACH configuration options are considered for SBFD based random access: - Option 1: Use one single RACH configuration based on the existing parameters of the single RACH configuration. Can extend the existing parameters if needed. - Option 2: Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration For RACH configuration Option 2, RAN2 needs to specify RRC signalling for the new SBFD based RACH configuration with a new set of parameters. The RACH configuration for SBFD is transmitted via SIB1. FFS dedicated RRC signalling detail. FFS whether NW can provide both configurations

During the previous meeting #118bis, RAN1 agreed as follows [2]:

Agreement For RACH configuration Option 1 with Alt 1-1, the association period and association pattern period of additional-ROs are determined separately from legacy-ROs. - The legacy rules for determining association period and association pattern period are reused in principle for additional-ROs. Agreement For RACH configuration Option 2, an additional-RO is valid if: - It is within SBFD symbols, or network configures that it can be valid if it starts from SBFD symbol and ends in non-SBFD symbol either in the same slot or across different slots. - It starts at least Ngap symbols after a last downlink non-SBFD symbol. - It starts at least Ngap symbols after the latest SSB - It does not overlap with SSB in time domain Note1: The Ngap here is the same as the Ngap in the current specification. Note2: It is up to network to ensure an additional-RO is configured within the bandwidth of the UL usable PRBs. Agreement Confirm the following Working Assumption: Working Assumption For RACH configuration Option 2, use legacy SSB-RO mapping rule for the additional-ROs configured by the additional RACH configuration, separate from the SSB-RO mapping for the legacy-ROs configured by the legacy RACH configuration. - FFS: Whether/How to handle the case where the legacy ROs overlap with additional ROs Agreement Confirm the following working assumption. Agreement For RACH configuration Option 2, and for interpretation of the parameter prach-ConfigurationIndex provided by the additional RACH configuration, - For FR2, use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-4 in TS38.211) - FFS whether to introduce new parameter(s) to determine the slot number for ROs in SBFD symbols. - Working Assumption: For FR1, use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-3 in TS38.211) - FFS whether to introduce new parameter(s) to determine the subframe number for ROs in SBFD symbols. Agreement For RACH configuration Option 1, support separate power control parameters for PRACH transmission in SBFD symbols and non-SBFD symbols - FFS: Details of the separate power control parameters Conclusion There is no consensus in RAN1 to support preamble partitioning on legacy-ROs for early indication of SBFD-aware UEs. It’s up to RAN2 to decide whether to support preamble partitioning on legacy-ROs for early indication of SBFD-aware UEs. Working Assumption For a PRACH transmission with Npreamblerep preamble repetition within additional-ROs, support reusing current specification rules for the determination of the set of Npreamblerep valid additional-ROs and the time period of the set(s) of Npreamblerep valid additional-ROs. - the time period of the set(s) of Npreamblerep valid additional-ROs is determined separately from the time period for legacy-ROs Conclusion PRACH transmission with preamble repetitions across additional-ROs and legacy-ROs is not supported. Agreement For SBFD-aware UEs and PRACH mask indication for CFRA triggered by PDCCH order, discuss whether/how to indicate/determine additional-RO only, legacy-RO only, or both is used. Agreement For SBFD aware UEs, for Msg3 PUSCH transmission, reuse Table 8.3-1 in TS 38.213 for determination of the frequency offset for the 2nd hop, with the update that the frequency offset for 2nd hop is based on the size of UL usable PRBs. - FFS the case when the value of NUL,hop= 11 - FFS the definition of UL usable PRBs for Msg3 PUSCH transmission Agreement For RACH configuration Option 1 with Alt 1-1, for determination of lowest RO of additional-ROs in SBFD symbols, select one from the following alternatives. - Alt 1: The parameter msg1-FrequencyStart in rach-ConfigCommon is applied with no reinterpretation. - Alt 2-1: The parameter msg1-FrequencyStart in rach-ConfigCommon is reinterpreted as the frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs. - Alt 2-3: The frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs is equal to mod (msg1-FrequencyStart, bandwidth of UL usable PRBs). - FFS: Possible modifications Conclusion There is no consensus in RAN1 to support the following in Rel-19: - Enabling both RACH configuration Option 1 and Option 2 for SBFD random access operation in a cell from network side.

In this contribution, we would like to discuss the detailed random access in SBFD symbols in RRC Connected and Idle/Inactive mode.

Discussion

To increase the SBFD symbol utilization, random access in SBFD symbols are proposed and studied. Random access in SBFD symbols increases the number of available UL transmission opportunities in the TDD UL-DL frame configuration period for the PRACH/preamble transmissions and reduces the random access latency.

According to RAN1 discussion so far, there could be a RACH Occasion (RO) within a SBFD symbol. And such RO may start from SBFD symbol and ending in non-SBFD symbol either in the same slot or across different slots can be valid based on network configuration. Figure 1 illustrates an example of RO across two SBFD slots.

Utilizing such RO in SBFD symbols, RAN1 and RAN2 require to design/ extend the existing random access procedure such as RO configuration, Msg.1/preamble transmission, and UE power ramping behaviour.

Figure 1 An example of RO across SBFD slots

Early Indication

RAN2 needs to discuss and decide on the necessity of an 'Early Indication' mechanism for Release 19 SBFD. Similar to previous releases of NR that supported an early indication mechanism for REDCAP, we believe Msg 3-based early indication mechanism could be considered for Release 19 SBFD, in addition to the obvious Msg 1-based early indication which is already agreed as a RAN2 understanding.

The reason behind considering an early indication mechanism is the same as for previous NR features; namely, the inability to promptly exchange or retrieve UE radio access capabilities until later during the initial access procedure once the UE identity has been verified and NAS and AS authentication have successfully completed.

The example of NR standalone mode operation for FR1 TDD 30kHz is considered in below Figure 2. Any UE in RRC_IDLE must connect to the network and first perform the NAS initial access procedure (“with stored context”) before DL/UL data scheduling.

Figure 2 NAS initial access procedure (“with stored context”) in NR SA mode

It should be considered that about 8-10 DL/UL RRC/NAS messages must be exchanged between the network and the UE during NAS initial access procedure. If an early indication type of mechanism becomes available for SBFD-aware UEs, then any PUSCH containing RRC and NAS messages during the transition from RRC_IDLE to RRC_CONNECTED mode can already be transmitted usint the SBFD UL subband following RACH Message 5 (RRC Setup Complete + NAS Registration Request) upon entering RRC_CONNECTED mode. Hence, an early indication could reduce the initial access delay significantly.

This is because SBFD UL subbands can only be configured for the UE by the RRC re-configuration msg which itself can occur only after the UE has been identified and authenticated by gNB and AMF.

As a candidate for early indication (EI) of SBFD-aware UEs, we consider both Msg1-based EI which could rely on RACH/preamble resource and Msg3-based EI that could rely on Msg.3 (or later) transmissions.

When RACH configuration option 1 is configured or indicated through SIB1, the choice of ROs used for random access attempts by SBFD-aware UEs depends on the RO selection and validation rules. If an SBFD-aware UE utilizes a legacy RO, Early Indication may require a Layer 3 (L3)-based approach.

It is also crucial to consider that feature independence must be achievable for deploying SBFD on the network side. Operators may opt to deploy SBFD solely for operations involving UEs in RRC_CONNECTED mode, meaning they may choose not to leverage the random access in SBFD resources feature. Consequently, Early Indication upon transitioning to RRC_CONNECTED mode would still necessitate reliance on an L3-based solution.

Proposal 1. RAN2 supports the Msg3-based Early Indication for Rel-19 SBFD-aware UEs.

RACH Resource Selection

Considering a User Equipment (UE) supporting the SBFD feature, it could make use of both legacy Random Access Opportunities (RO) and SBFD RO for random access. In situations where there are two ROs configured, the UE must determine which RO to utilize for its random access.

To ensure that the SBFD-aware UE benefits from this additional SBFD RO, we should first establish whether such a UE should be capable of utilizing the legacy RO. When considering the relationship between the SBFD RO and the legacy RO, we might explore the possibility of introducing rules, conditions, or prioritization mechanisms among the ROs. Given these considerations, we recommend that RAN2 agreed as follows:

Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/ configured conditions/ prioritizations, if no additional indication (FFS if there needs to be any) is from network.

In more detail, UE could select one type of ROs, based on 1) network configured or specified conditions are met, or 2) existing(configured) prioritization rule.

Considerable selection criteria could be as follows:

Proposal 2. Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects SBFD ROs over legacy ROs if the serving cell measurement is above the network configured threshold value.

We believe that there exists an SBFD-aware UE capable of utilizing both SBFD ROs and legacy ROs, allowing it to take advantage of the fastest available RO. In order to provide low latency support for this UE, we suggest that the network should indicate such behavior. Proposal 3 suggests that upon initiating the CBRA RACH process for an SBFD-aware UE, the network can instruct the UE to use the nearest RO for its initial access. The specific details regarding FFS signaling are yet to be determined.

Proposal 3. Upon initiation of CBRA RACH procedure for a SBFD-aware UE, network can indicate the UE to utilize the nearest RO for low latency.

Type-2 Random Access

In the Type-2 random access procedure, RACH Msg.1 and Msg.3 (PUSCH) are transmitted together as MsgA. The RACH Msg.2 (RAR) and Msg.4 (for contention resolution) are transmitted together as MsgB. Type-2 is applicable for UEs in RRC_CONNECTED and in RRC_IDLE/INACTIVE modes. Both CBRA and CFRA are possible for Type-2 random access. For MsgA transmission, the gNB configures the MCS and PUSCH allocation. For MsgB reception, a new RNTI is defined. The Type-2 procedure reduces signaling overhead and decreases RA procedure delay.

In Rel-16 NR, the random access causes for Type-1 are considered as valid for Type-2 random access. No new/additional random access cause is introduced for Type-2.

When Type-2 random access is available and configured in the serving cell, the UE selects the Type-1 or Type-2 procedure at the initiation of the random access procedure based on the network configuration.

Most expected design and specification impact to support Type-1 random access procedure in SBFD symbols occur with respect to the Msg.1/preamble transmission such as RO configuration/validation, SSB-RO mapping rules, and UE power ramping behavior. Support of the Type-2 procedure for MsgA transmission shares these common design aspects. For (Type-2) PUSCH in MsgA in SBFD symbols, network configuration can be used similar to existing Rel-16 NR principles.

Therefore, we do not see significantly increased specification effort to also support Type-2 random access in SBFD symbols in addition to the Type-1 procedure.

Proposal 4: For SBFD-aware UEs, support the Type-2 random access procedure in SBFD symbols.

Conclusion

RAN2 is requested to discuss and agree to the following proposals:

Proposal 1. RAN2 supports the Msg3-based Early Indication for Rel-19 SBFD-aware UEs.

Proposal 2. Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects SBFD ROs over legacy ROs if the serving cell measurement is above the network configured threshold value.

Proposal 3. Upon initiation of CBRA RACH procedure for a SBFD-aware UE, network can indicate the UE to utilize the nearest RO for low latency.

Proposal 4: For SBFD-aware UEs, support the Type-2 random access procedure in SBFD symbols.

- References

[1] RP-240789 “New WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”; RAN#103

[2] Draft_Minutes_report_RAN1#118bis_v020, Draft Report of 3GPP TSG RAN WG1 #118bis v0.2.0, RAN1