3GPP TSG-RAN WG2 Meeting #128	R2-2410613

Orlando, USA, 18 – 22 November 2024

Agenda item:	8.18

Source:	Nokia, Nokia Shanghai Bell

Title:	PDCP SN Gap reporting at mobility

WID/SID:	TEI - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The previous RAN3 meetings discussed Rel-18 proposals of transferring, between handover source RAN node and target RAN node, uplink PDCP SN Gap information received from the UE. Since a possible need to transfer such information is specific to DRBs mapped on RLC AM, whose applicability to XR is only being optimized in Rel.19 by RAN2, nothing was agreed.

This contribution proposes that RAN2 address the issue as part of the ongoing Rel-19 work.

# 2	Discussion

Based on the current Rel-18 specifications, the following alternative courses of events may have happened with a DRB mapped on RLC AM, just before the UE receives an RRC command for an event involving network-side change of the DRB’s PDCP-termination point (handover, or bearer-type change between MN- and SN-terminated bearer):

A.	The source RAN node successfully received a PDCP SN Gap report from the UE:

-	The current specs, such as XnAP SN Status Transfer, provide no means to transfer this information from the source RAN node to the target RAN node;

-	This case was main the topic of the recent RAN3 discussions.

B.	The UE transmitted a PDCP SN Gap report, but its RLC transmissions did not successfully complete before the layer was flushed by the RRC command:

-	In this case, there is no information to be transferred from the source RAN node to the target RAN node.

Considering the case B, the only solution direction seems to involve the UE retransmitting PDCP SN Gap information to the target RAN node.

Observation:	The UE PDCP may have transmitted a PDCP SN Gap report just before a PDCP-mobility event that interrupts the report’s RLC transmission. The only way to recover the report’s information seems to be retransmission by the UE PDCP, to the target RAN node.

A simple solution seems to be that, as part of PDCP Re-establishment (which is always invoked at any PDCP termination-point change), the UE re-transmit any previously transmitted PDCP SN Gap report(s) for which no lower-layer confirmation of successful delivery has been received.

Proposal 1:	As part of PDCP entity re-establishment, for AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink, the transmitting PDCP entity shall re-transmit any previously (prior to the PDCP entity re-establishment) transmitted PDCP SN Gap report(s) for which the successful delivery has not been confirmed by lower layers.

A text proposal is provided in the Annex.

Proposal 2:		RAN2 adopt the Text proposal in Annex implementing Proposal 1.

# 3	Conclusion

This document has made the following observations:

Observation:	The UE PDCP may have transmitted a PDCP SN Gap report just before a PDCP-mobility event that interrupts the report’s RLC transmission. The only way to recover the report’s information seems to be retransmission by the UE PDCP, to the target RAN node.

And proposed the following:

Proposal 1:	As part of PDCP entity re-establishment, for AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink, the transmitting PDCP entity shall re-transmit any previously (prior to the PDCP entity re-establishment) transmitted PDCP SN Gap report(s) for which the successful delivery has not been confirmed by lower layers.

Proposal 2:		RAN2 adopt the Text proposal in Annex implementing Proposal 1.

# Annex: Text proposal to TS 38.323

### 5.1.2	PDCP entity re-establishment

When upper layers request a PDCP entity re-establishment, the UE shall additionally perform once the procedures described in this clause for Uu or PC5 interface. After performing the procedures in this clause, the UE shall follow the procedures in clause 5.2.

When upper layers request a PDCP entity re-establishment, the transmitting PDCP entity shall:

-	for UM DRBs and AM DRBs, reset the ROHC protocol for uplink and start with an IR state in U-mode (as defined in RFC 3095 [8] and RFC 4815 [9]) if drb-ContinueROHC is not configured in TS 38.331 [3];

-	for UM DRBs and AM DRBs, reset the EHC protocol for uplink if drb-ContinueEHC-UL is not configured in TS 38.331 [3];

-	for AM DRBs, reset the UDC compression buffer to all zeros and prefill the dictionary if drb-ContinueUDC is not configured in TS 38.331 [3];

-	for SRBs and UM DRBs, set TX_NEXT to the initial value;

-	for SRBs, discard all stored PDCP SDUs and PDCP PDUs;

-	apply the ciphering algorithm and key provided by upper layers during the PDCP entity re-establishment procedure;

-	apply the integrity protection algorithm and key provided by upper layers during the PDCP entity re-establishment procedure;

-	for UM DRBs, for each PDCP SDU already associated with a PDCP SN but for which a corresponding PDU has not previously been submitted to lower layers, and;

-	for AM DRBs for Uu interface whose PDCP entities were suspended, from the first PDCP SDU for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by lower layers, for each PDCP SDU already associated with a PDCP SN:

-	consider the PDCP SDUs as received from upper layer;

-	perform transmission of the PDCP SDUs in ascending order of the COUNT value associated to the PDCP SDU prior to the PDCP re-establishment without restarting the discardTimer or the discardTimerForLowImportance, as specified in clause 5.2.1;

-	for AM DRBs whose PDCP entities were not suspended, from the first PDCP SDU for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by lower layers, perform retransmission or transmission of all the PDCP SDUs already associated with PDCP SNs in ascending order of the COUNT values associated to the PDCP SDU prior to the PDCP entity re-establishment as specified below:

-	perform header compression of the PDCP SDU using ROHC as specified in the clause 5.7.4 and/or using EHC as specified in the clause 5.12.4;

-	If drb-ContinueUDC is configured and if the PDCP SDU has been compressed before:

-	submit the PDCP SDU previously compressed to integrity protection and ciphering function;

-	else:

-	perform uplink data compression of the PDCP SDU as specified in clause 5.14.4, and submit the PDCP SDU to integrity protection and ciphering function;

-	perform integrity protection and ciphering of the PDCP SDU using the COUNT value associated with this PDCP SDU as specified in the clause 5.9 and 5.8;

-	submit the resulting PDCP Data PDU to lower layer, as specified in clause 5.2.1;

-	for AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink (sn-GapReport in TS 38.331 [3]), if prior to the PDCP entity re-establishment a PDCP SN gap report was submitted to lower layers but its successful delivery has not been confirmed by lower layers:

-	re-submit the PDCP SN gap report to lower layers as specified in clause 5.2.1 for Uu interface.

When upper layers request a PDCP entity re-establishment, the receiving PDCP entity shall:

-	process the PDCP Data PDUs that are received from lower layers due to the re-establishment of the lower layers, as specified in the clause 5.2.2.1;

-	for SRBs, discard all stored PDCP SDUs and PDCP PDUs;

-	for SRBs, UM DRBs and UM MRBs, if t-Reordering is running:

-	stop and reset t-Reordering;

-	for UM DRBs and UM MRBs, deliver all stored PDCP SDUs to the upper layers in ascending order of associated COUNT values after performing header decompression;

-	for AM DRBs and AM MRBs for Uu interface, perform header decompression using ROHC for all stored PDCP SDUs if drb-ContinueROHC is not configured in TS 38.331 [3];

-	for AM DRBs for PC5 interface, perform header decompression using ROHC for all stored PDCP IP SDUs;

-	for AM DRBs and AM MRBs for Uu interface, perform header decompression using EHC for all stored PDCP SDUs if drb-ContinueEHC-DL is not configured in TS 38.331 [3];

-	for UM DRBs, AM DRBs, UM MRBs and AM MRBs, reset the ROHC protocol for downlink and start with NC state in U-mode (as defined in RFC 3095 [8] and RFC 4815 [9]) if drb-ContinueROHC is not configured in TS 38.331 [3];

-	for UM DRBs, AM DRBs, UM MRBs and AM MRBs, reset the EHC protocol for downlink if drb-ContinueEHC-DL is not configured in TS 38.331 [3];

-	for SRBs and UM DRBs, set RX_NEXT and RX_DELIV to the initial value;

-	for UM MRBs and AM MRBs, set RX_NEXT and RX_DELIV to the initial value if initialRX-DELIV is configured in TS 38.331 [3];

-	apply the ciphering algorithm and key provided by upper layers during the PDCP entity re-establishment procedure;

-	apply the integrity protection algorithm and key provided by upper layers during the PDCP entity re-establishment procedure.

NOTE 1:	After PDCP re-establishment on a sidelink ‎SRB/DRB, UE determines when to transmit and receive with the new key and discard the old key as specified in TS ‎‎33.536 [14].‎

NOTE 2:	At PDCP re-establishment, the MRB type (i.e. UM MRB or AM MRB) is determined by the target configuration.‎