3GPP TSG RAN WG2 #128                                         R2-2410619

Orlando, USA, Nov.  18th – 22nd , 2024

Agenda Item:	8.13.1

Source: 	TCL

Title: 	Relay discovery and (re)selection

Document for:	Discussion and decision

Introduction

In RAN#103 meeting, the new WID was approved [1], and the relay discovery and (re)selection objective is introduced as following:

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. - Relay discovery and (re)selection [RAN2] - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] - Impact on SRAP and QoS handling for multi-hop [RAN2] - Control plane procedures [RAN2, RAN3] .

In this paper, we discuss how to introduce multi-hop relay, and how to enable the remote UE to perform SL relay discovery and (re)selection.

Discussion

relay UE selection

When a remote UE is selecting a first relay UE, the first relay UE is working as a remote UE before any remote UE is connected to itself. So in this case, the first relay UE as a remote UE should work in RRC_CONNECTED state when it is ready to work as a relay UE before any remote UE is connected to the first relay UE. In the WID, it is assumed that the all UEs are in RRC_CONNECTED state before and after forming a multi-hop PC5 link. And candidate first relay UE is in RRC_CONNECTED state via indirect connection. Therefore, the candidate first relay UE is a remote UE in RRC_CONNECTED state.

Proposal 1. the first relay UE as a remote UE should work in RRC_CONNECTED state when it is ready to work as a relay UE before any remote UE is connected to the first relay UE.

In the Rel_17/18 SL relay discovery, the relay UE is allowed to transmit discovery messages when both a maximum Uu RSRP threshold and/or a minimum Uu RSRP threshold are met, if configured; One solution is to consider that relay UEs that falls below the minimum Uu RSRP threshold are allowed to transmit discovery as an intermediate relay UE, similar to the restriction that was applied to the legacy remote UE.

Proposal 2: RAN2 is asked to consider if RSRP threshold of Uu interface can be considered as a factor for SL relay first relay UE selection.

Likewise, when multi-hop PC5 links is introduced to the SL relay, it is straightforward that the PC5 link quality(s) of each link should be considered. With the best PC5 link quality(s), the remote UE can select the most reliable PC5 link to be connected to the gNB. The remote UE should know the best PC5 links of the PC5 link. The details of how the remote UE obtains the multi-hop PC5 link and how the remote UE considers each PC5 link quality(s) can be further discussed.

Proposal 3: the remote UE should consider the PC5 links quality of the multi-hop PC5 links for relay selection. The details of how the remote UE obtains the multi-hop PC5 link and how the remote UE considers each PC5 link quality(s) can be further discussed.

There are also some other information should be considered for triggering relay (re)selection, we should also consider whether the new information provided in the discovery message should impact relay (re)selection. In particular, SA2 has included in the discovery message both hop count and QoS information which is either Per-hop QoS information or cumulative QoS information (e.g. cumulative delay between Remote UE up to the Intermediate Relay).  It isn’t clear how such QoS information is provided to the relay UEs and how remote UEs, esp. remote UEs attempting initial access would use such QoS information. In this circumstance, we believe that both Model A discovery procedure and Model B discovery procedure should apply the information for the remote UE for relay selection.

Proposal 4: both Model A discovery procedure and Model B discovery procedure should consider the hop number for relay selection.

Therefore, for Model A discovery, we assume IDLE remote UEs should select relay UE with the least number of hops, as long as the PC5 link quality is above the (pre)configured threshold. For example, we assume the remote UE will select a U2N relay UE over any intermediate relay UE and that discovery message contents will be used to distinguish between the two types of relay UE.  However, it should be further discussed whether this should be left for UE implementation.

Proposal 5: For Model A discovery procedure for the purpose of initial relay selection, the remote UEs should select the path considering the number of hops of each relay UE .

For model B discovery procedure, we assume the remote UE firstly sends Relay discovery solicitation message to the relay UE, and then relay UE replies the Relay discovery response message to the remote UE. So in this model B discovery procedure, relay UE can include the hop number in the Relay discovery response message to the remote UE.

Proposal 6: For Model B discovery procedure for the purpose of initial relay selection, the remote UEs should select the path considering the number of hops .

We also consider QoS profile for the (potential) remote UE can be used for the gNB to select the path. This allows the gNB to select a path with low latency and/or robustness (i.e., less likely to be disconnected) depending on the QoS and radio resource management of the remote UE. According to the current SA2 discussion, the discovery message may include hop counter, ID of last relay UE and accumulated QoS for the PC5 links (i.e. Achievable PDB) in addition to the legacy contents. Therefore, the remote UE knows these metrics. Then, the gNB knows that the reported first relay UE is connecting with the same gNB or not to which the reported candidate first relay UE belongs, and the QoS profile for the (potential) remote UE. So, for selecting appropriate path (first relay UE), UE should send the information of PC5 link quality of the path the first relay UE belongs. Furthermore, the greater number of hops there are, the more the quality of each hop affects the delay and the efficiency of radio resource utilization. So, we consider that the UE needs to report to the gNB the PC5 link qualities of each hops of the path first relay UE belongs.

Proposal 7: gNB needs to know the accumulated QoS for the remote UE to select the path.

relay UE reselection

The remote UE should conduct relay UE reselection when the radio condition, hop number, and other factors is facing deteriorating. But we also need to identify that the relay UE reselection can’t be conducted too frequently, as the remote UE may frequently reselect a new relay UE, in result that the data transmission is interrupted too frequently.  So there should be a mechanism, like the gNB can configure the minimum relay UE reselection condition, if the minimum relay UE reselection condition is satisfied, then the remote UE doesn’t need to conduct a relay UE reselection.

Proposal 8: the gNB can configure the minimum relay UE reselection condition, if the minimum relay UE reselection condition is satisfied, then the remote UE doesn’t need to conduct a relay UE reselection.

Conclusion

In this contribution, we discussed the factors for relay UE (re)selection, and

Proposal 1. the first relay UE as a remote UE should work in RRC_CONNECTED state when it is ready to work as a relay UE before any remote UE is connected to the first relay UE.

Proposal 2: RAN2 is asked to consider if RSRP threshold of Uu interface can be considered as a factor for SL relay first relay UE selection.

Proposal 3: the remote UE should consider the PC5 links quality of the multi-hop PC5 links for relay selection. The details of how the remote UE obtains the multi-hop PC5 link and how the remote UE considers each PC5 link quality(s) can be further discussed.

Proposal 4: both Model A discovery procedure and Model B discovery procedure should consider the hop number for relay selection.

Proposal 5: For Model A discovery procedure for the purpose of initial relay selection, the remote UEs should select the path considering the number of hops of each relay UE .

Proposal 6: For Model B discovery procedure for the purpose of initial relay selection, the remote UEs should select the path considering the number of hops .

Proposal 7: gNB needs to know the accumulated QoS for the remote UE to select the path.

Proposal 8: the gNB can configure the minimum relay UE reselection condition, if the minimum relay UE reselection condition is satisfied, then the remote UE doesn’t need to conduct a relay UE reselection.

Reference

- RP-241609, New WID on NR sidelink multi-hop relay