3GPP TSG-RAN WG2 Meeting #128													                         R2-2410636

Orlando, USA, Nov 18th – 22th, 2024                                                           Revision of R2-2408622

Agenda item:	8.9.2

Source:	MediaTek Inc.

Title:	RAN2 impact on S&F mode

Document for:	Discussion and Decision

# Introduction

Rel-19 IoT NTN work item was revised in RAN#105 [1], one of the objectives is to support S&F operation, as below:

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore:

  - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2]

    - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3]

Note: Strive to minimise UE impact.

Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

A study report, 23.700-29 [2], on the integration of satellite components in the 5G architecture Phase 3 has been released by 3GPP SA2. The conclusion for Key Issue #2: Support of Store and Forward Satellite operation has been made. Two types of system architecture can be adopted by operation deployment: a full CN onboard the satellite or a Split MME architecture.

In this contribution, we share our view on S&F mode, incorporating input from SA2.

# Discussion

S&F time indication

In the RAN2#127, remaining time of the satellite’s current operation mode was discussed, but no agreement was reached.

Proposal 6 UE is informed of the remaining time of the satellite’s current operation mode (normal or S&F) via System Information broadcast. - Ericsson thinks this could be realized with the introduction of a t-service like parameter - QC is not sure how this would be used. Samsung agrees the UE behaviour is not clear. CATT agrees - ZTE is interested in this proposal - Sequans thinks we need an absolute time. Ericsson confirms this is the intention - IDC thinks this could be useful - Telit thinks that if the timer is shorter than t-service this would rather bring additional problems to the NW/system behaviour - Google thinks an indication could be beneficial also for R19 UUs not supporting S&F operation. Come back to this in the next meeting

It can also be beneficial in many aspects if the UE knows when the cell will gain or lose the feeder link. The device user can be notified of how long the feeder link will be available, allowing them to take the opportunity to start delay-sensitive transmissions, or start cell measurement for cell reselection before enter S&F mode. Alternatively, the device user can be aware of when the feeder link will become available, enabling them to catch the gap when the cell can provide real-time services. How to use this information is also up to the UE implementation.

Proposal 1: The times when the feeder link will become available and unavailable can be broadcast in an S&F operation-enabled cell via system information.

There was discussion on reusing the legacy t-service for cell change to S&F mode.  The reason is that for legacy UE, the UE behaviours for losing coverage and entering S&F mode are the same. But for S&F capable UEs, the UE behaviours are different. For losing coverage, S&F capable UEs should follow legacy approach (start measurement for cell reselection); for entering S&F mode, S&F capable UEs in idle should be fine with it, maybe just a mode switch notification from RRC to upper layer. So reusing legacy t-service for cell change to S&F mode does not work for S&F capable UEs. The time for cell change to S&F mode must be indicated by new IE in system information.

For legacy UE, when the cell change to S&F mode, the network can send paging message to update the system information and bar legacy UE. Without this time information is acceptable for legacy UE.

Proposal 2: New IE in system information is used for indicating the time when the cell will change to S&F mode.

Cell selection with satellite ID

In the study report [2], for the split MME architecture, a wait time and a list of satellite IDs will be provided to UE to re-attempt the Attach/TAU procedure.

23.700-29 v1.0.0 clause 8.2 b) Wait timer: Indicates to the UE the time it should wait before re-attempting the Attach/TAU procedure in the current or another satellite of the same PLMN. c) Optionally, The list of Satellite IDs over which the UE may re-attempt the Attach/TAU procedure, after wait timer expires. The Satellite IDs are based on the SIB information broadcasted by eNB.

For the full CN onboard the satellite, a S&F monitoring list of satellites IDs will be optionally provided to the UE during the attach/TAU. The UE uses the satellites in the S&F monitoring list for MO/MT data/signalling with the CN. Specifically, the S&F monitoring list may assist the UE in retrieving MT data.

23.700-29 v1.0.0 clause 8.2 - Optionally the MME provides the UE with a S&F monitoring list of satellites IDs, during attach/TAU. The UE uses the satellites in the S&F monitoring list for MO/MT data/signalling with the CN. The S&F monitoring list can be determination by the CN. How network determines the S&F monitoring list is outside the scope of 3GPP in this release of specification. NOTE 8: The S&F monitoring list may assist the UE in retrieving MT data.

To support these functions, NAS can indicate a list of satellite IDs to AS during the PLMN selection and cell selection. Currently, SIB31, SIB32, and SIB33 contain satellite IDs. SIB32 includes a frequency list for a given satellite ID, and the corresponding frequency of the satellite ID in SIB33 can be found in SIB3 or SIB5. The frequency of satellite in SIB31 is the frequency of the serving cell. The UE can determine the frequency of the cell from the satellite ID. When NAS indicates the satellite ID to AS, the UE can first find the corresponding cell frequency from the SIBs and search the cell on that frequency. If a cell is found on that frequency, the UE may check the satellite ID to see if it matches the one requested by NAS.

Alternatively, NAS can request the UE to report the satellite ID of the found cell during the PLMN selection and cell selection.

The wait time can be used in NAS to trigger the PLMN selection or cell selection.

Proposal 3: NAS can indicate a list of satellite IDs to AS during the PLMN selection and cell selection. The UE AS will attempt to camp on the cell provided by satellite IDs.

Proposal 4: NAS can request UE to report the satellite ID of found cell during the PLMN selection and cell selection.

Proposal 5: Satellite IDs in SIB31, SIB32, and SIB33 can be reused for providing the mapping between satellite ID and frequency information.

To support cell search for a specific satellite when the UE is not camping on a cell, the UE may store all the known satellite ID and frequency mapping information for each PLMN.

Proposal 6: The UE stores satellite ID and frequency mapping information for each PLMN.

# Conclusion

S&F time indication

Proposal 1: The times when the feeder link will become available and unavailable can be broadcast in an S&F operation-enabled cell via system information.

Proposal 2: New IE in system information is used for indicating the time when the cell will change to S&F mode.

Cell selection with satellite ID

Proposal 3: NAS can indicate a list of satellite IDs to AS during the PLMN selection and cell selection. The UE AS will attempt to camp on the cell provided by satellite IDs.

Proposal 4: NAS can request UE to report the satellite ID of found cell during the PLMN selection and cell selection.

Proposal 5: Satellite IDs in SIB31, SIB32, and SIB33 can be reused for providing the mapping between satellite ID and frequency information.

Proposal 6: The UE stores satellite ID and frequency mapping information for each PLMN.

# References

RP-242397, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, Rapporteur (MediaTek)

TR 23.700-29 v.1.0.0. Study on integration of satellite components in the 5G architecture Phase 3