3GPP TSG-RAN WG2 Meeting #128        											                          R2-2410643

Orlando, USA, Nov 18th – 22th, 2024                                                        Revision of R2-2408624

Agenda item:	8.9.4

Source:	MediaTek Inc.

Title:	Discussion on supporting PWS for NB-IoT

Document for:	Discussion and Decision

# Introduction

New objective was added to Rel-19 IoT NTN work item in RAN#105 [1]:

- Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2]

Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI.

Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

This work item has been discussed in RAN2#127bis meeting, and the following agreements have been reached.

Agreements:

1.	RAN2 confirms the understanding that this WID objective covers all PWS services, including ETWS and CMAS

2.	The support for PWS introduced for NB-IoT NTN can also be made applicable for NB-IoT in TN, if this does not require additional NB-IoT TN specific changes.

3.	For NB-IoT NTN PWS, introduce the following new SIBs, taking the content within the corresponding LTE SIBs as a baseline (but also checking whether we can have some optimization/ skip some unnecessary fields):

- SystemInformationBlockType10-NB for primary ETWS notification;

- SystemInformationBlockType11-NB for secondary ETWS notification;

- SystemInformationBlockType12-NB for CMAS notification.

4.	Add the following PWS indication in direct indication information for NB-IoT (Come back in the next meeting on whether this is also added in Paging-NB):

- etws-Indication;

- cmas-Indication.

5.	Upon receiving the PWS notification from NB-IoT cell, the PWS-capable NB-IoT UE acquires the corresponding PWS message immediately (can come back to clarify further what immediately means)

6.	Clarify in the spec (and at least in 36.300) that for NB-IoT the ETWS, CMAS, PWS requirement may not be met in some scenarios, e.g. when the UE is in eDRX (can come back to further clarify the specific cases)

In this contribution, we continue discussing the indication in paging message, in RRC connected mode.

# Discussion

To support the PWS message for NB-IoT, the LTE mechanism can be reused as indicated in the WID.

In the RAN2#127bis, the following agreement has been reached to adding the ETWS indication and CMAS indication in the direct indication information. Whether to add the same indications to paging is for FFS.

Add the following PWS indication in direct indication information for NB-IoT (Come back in the next meeting on whether this is also added in Paging-NB):

- etws-Indication;

- cmas-Indication.

The PWS messages can be indicated to the UE in RRC idle mode via the direct indication information. But when there are already paging messages transmitting to the UE, adding ETWS indication and CMAS indication to paging message can help reduce air message overhead and the transmission latency of PWS messages.

Similar to LTE, new bits in the paging message should be introduced to send ETWS indication and CMAS indication to the UE in RRC Idle mode.

Proposal 1: Introduce new ETWS indication and CMAS indication in the Paging-NB message.

By current specification, NB-IoT UE does not monitor paging message and direct indication information in the RRC connected state. Therefore, the PWS message does not work in RRC connected mode. We believe that the above agreement is only applicable to the RRC idle mode. To fulfill the Rel-19 work item, it is natural to enable PWS message monitoring in RRC connected mode for NB-IoT UEs.

For eMTC, receiving Direct Indication Information using SI-RNTI in connected mode is supported as in 36.331 Table 6.6-2.

Table 6.6-2: Direct Indication information using SI-RNTI

| Bit | Direct Indication information |
| --- | --- |
| 1 | etws-Indication |
| 2 | cmas-Indication |
| 3, 4, 5, 6, 7, 8 | Not used, and shall be ignored by UE if received. |

To acquire ETWS indication and CMAS indication in RRC connected mode, a new Direct Indication Information in RRC connected mode similar to that in eMTC should be introduced. This change should also be captured in RAN1 spec, and an LS for this is needed.

Proposal 2: Send an LS to RAN1 for new Direct Indication Information that includes ETWS indication and CMAS indication in RRC Connected mode.

Monitoring the Direct Indication Information in RRC connected mode will result in increased power consumption. Hence, whether a UE needs to monitor the Direct Indication Information in RRC connected mode should be configured by the network. Similar to eMTC, a new dedicated RRC configuration should be introduced.

Proposal 3: Introduce a new dedicated RRC configuration to receive new Direct Indication Information in RRC Connected mode.

The network needs to know if the UE is capable of monitoring new Direct Indication Information in RRC Connected mode, so that it can determine whether to send the configuration. Similar to eMTC, a new UE capability is needed.

Proposal 4: Introduce a new UE capability to receive new Direct Indication Information in RRC Connected mode.

# Conclusion

Proposal 1: Introduce new ETWS indication and CMAS indication in the Paging-NB message.

Proposal 2: Send an LS to RAN1 for new Direct Indication Information that includes ETWS indication and CMAS indication in RRC Connected mode.

Proposal 3: Introduce a new dedicated RRC configuration to receive new Direct Indication Information in RRC Connected mode.

Proposal 4: Introduce a new UE capability to receive new Direct Indication Information in RRC Connected mode.

# References

[1] RP-242397, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, Rapporteur (MediaTek)