3GPP TSG-RAN WG2#128		      					R2-2410661

Orlando, USA, November 18 – 22, 2024			     Revision of R2-2409001

Agenda item:	8.6.4

Source: 	Kyocera

Title: 	Discussion on Conditional Intra-CU LTM

Document for:	Discussion

Introduction

RAN#105 approved the revised WID for NR mobility enhancements Phase 4 [1].

The following was agreed on conditional intra-CU LTM in RAN2#127-bis [2]:

Agreements on C-LTM 1. Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions. 2. Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions. 3. Source cell and each candidate cell provides its own execution condition for conditional LTM. 4. It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used. 5. RACH-less Conditional intra-CU LTM is supported. 6. RACH based conditional intra-CU LTM is supported. 7. UE based TA measurement mechanism is supported for conditional intra-CU LTM. 8. PDCCH ordered early TA acquisition is supported for conditional LTM. 9. Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM. 10. For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach. 11. The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

In this contribution, the potential issues for supporting Conditional Intra-CU LTM are discussed.

Discussion

LTM preparation phase

In the LTM preparation phase of Rel-18 Intra-CU LTM, the network configures the UE with LTM Config within RRC Reconfiguration [4].

For the conditional LTM preparation phase, it’s straightforward that the conditional LTM candidate cell(s) is pre-configured by RRC Reconfiguration as well. However, it’s unclear whether the existing LTM Config can be reused for the Conditional LTM preparation. The following options would be considered:

- Option 1a: Extend the existing LTM Config for Conditional LTM.

- Option 1b: Introduce a new Conditional LTM Config.

With regard to Option 1a, it’s easy to extend by adding new IE(s) for Conditional LTM to the existing LTM Config. On the other hand, with Option 1b, introduces a new Conditional LTM Config, which can be separately from the existing LTM Config.

RAN2 should discuss whether to extend the existing LTM Config (Option 1a) or introduce a new conditional LTM Config (Option 1b).

Early sync phase

In the previous meeting, RAN2 agreed to support RACH-less Conditional intra-CU LTM with PDCCH-ordered early TA acquisition and UE-based TA measurement. In case of PDCCH-ordered early TA acquisition, it has a couple of potential issues in handling TA value.

It’s already clear that the Cell Switch Command MAC CE is not used in Conditional LTM. So, for the PDCCH-ordered early TA acquisition, there is no way for the UE to receive the TA value, even if the early TA was initiated. Therefore, it should be considered how the UE to receive the TA value, and the following options are available for transferring TA values:

- Option 1: A new TA command MAC CE

  - Pros: There is no impact to the existing procedures due to the introduction of a new MAC CE.

  - Cons: A new MAC CE structure and corresponding procedure need to be specified.

- Option 2: Existing Random Access Response

  - Pros: Existing RAR PDU can be modified and reused.

  - Cons: According to the existing specification for random access procedures for LTM (i.e., the early TA), it is not required to wait for an RAR. Therefore, the random-access procedure for LTM needs to be changed.

    - In addition, it is necessary to discuss how to avoid unnecessary information which is in the current RAR PDU, i.e., UL grant and C-RNTI. The changes could significantly impact the current random access procedure.

RAN2 should discuss whether the UE receives the TA value by a new TA command MAC CE (Option 1) or by the existing RAR (Option 2), for Conditional Intra-CU LTM.

In addition, it would be considered whether the source gNB or the candidate gNB sends the TA values. The pros and cons of these options would be analysed as follows:

- Option A: Source gNB:

  - Pros: As in the case with Rel-18 early TA, the TA values can be transmitted to UE with the minimum data interruption time. The current specification of how to transmit the Cell Switch Command MAC CE can be the baseline.

  - Cons: The TA value needs to be transferred via the network interfaces, i.e., the candidate DU  the candidate CU  the source CU  the source DU, which cause a considerable delay, e.g., tens of milli-seconds. Therefore, before the TA value is transmitted to the UE, there is a possibility that the radio quality from the serving cell may become worse, and the TA value cannot be received by the UE.

- Option B: Candidate gNB:

  - Pros: For conditional LTM, the NW doesn’t know when the UE executes the LTM cell switch, so the NW should transmit the TA value as soon as possible. Since the TA value is obtained directly from the candidate gNB, the latency for the UE’s TA value acquisition is minimized.

    - In case the PDCCH-ordered PRACH is received by the candidate gNB successfully, it means the radio condition between the candidate gNB and the UE is good enough to transfer the TA value, regardless of the radio condition between the serving cell and the UE.

  - Cons: While the UE is waiting for the TA value from the candidate gNB, the data transfer with the source gNB is interrupted.

    - If the UE is configured with many candidate cells (different gNB(s)), then each of these cells will need to send separate TA values to the UE, whereas if the source cell sends the TA value it could be done in one message, assuming all the RACH to all the candidate cells has already being sent.

When the radio condition between the UE and the source gNB worsens, it may no longer be possible for the UE to receive the TA value from the serving gNB, which results in a longer cell switch process since the UE triggers the RACH-based LTM. From a robustness perspective under such an unstable radio condition on the serving gNB link, it’s preferable that the candidate gNB transmits the TA value, rather than the serving gNB. When the candidate gNB transmits the TA value, there is the data interruption, but according to current specification for the licensed spectrum, the interruption time is as short as 1ms and at most less than 10ms [4], which can be acceptable for more robust early TA acquisition.

RAN2 should agree the candidate gNB transmits the TA value to the UE, for Conditional Intra-CU LTM.

The other issue is how to manage the validity of TA value. In Rel-18, the serving gNB handles the TA validity management and always sends the valid TA value in Cell Switch Command MAC CE. However, it’s assumed that no Cell Switch Command MAC CE is used in Conditional LTM. So, the UE needs to check whether the TA value it has is valid, before the RACH-less Conditional LTM execution.  The current UE uses the timing alignment timer (TAT) for the same purpose, so a similar timer can be introduced for the TA management in the early sync phase.

RAN2 should agree to introduce a timer like existing TAT (timing alignment timer) to ensure the validity of TA values for Conditional LTM.

Conclusion

In this contribution, the issues for supporting conditional intra-CU LTM are discussed. RAN2 is kindly asked to take into account the proposals below:

Proposal 1	RAN2 should discuss whether to extend the existing LTM Config (Option 1a) or introduce a new conditional LTM Config (Option 1b).

Proposal 2	RAN2 should discuss whether the UE receives the TA value by a new TA command MAC CE (Option 1) or by the existing RAR (Option 2), for Conditional Intra-CU LTM.

Proposal 3	RAN2 should agree the candidate gNB transmits the TA value to the UE, for Conditional Intra-CU LTM.

Proposal 4	RAN2 should agree to introduce a timer like existing TAT (timing alignment timer) to ensure the validity of TA values for Conditional LTM.

References

- RP-242356 “Revised Work Item: NR mobility enhancements Phase 4”, Apple Inc, China Telecom, RAN#105

- R2-2409211 “Report from session on V2X/SL, R18/19 MOB, and R19 NES”, Vice Chairman (Samsung), RAN2#127bis

- TS38.300, “NR and NG-RAN Overall Description; Stage 2”, 3GPP

- TS38.331, “Radio Resource Control (RRC) protocol specification”, 3GPP