3GPP TSG-RAN WG2 Meeting #128                                      	         R2-2410671

Orlando, USA, Nov 18th – 22th, 2024

Agenda item:	8.8.2

Title:	RAN2 Impact on DL coverage enhancements

Source:	CMCC

Document for:	Discussion

- Introduction

In previous RAN1 meetings, the study of downlink coverage enhancement in NR NTN had been discussed, focusing on the evaluation methodology, simulation assumptions and link budget analysis for DL coverage performance.

And in last RAN1 meeting, the following agreements has acheived:

Agreement For NR NTN, support extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is at least 160 ms. - FFS: whether 320ms can be supported as the maximum of the additional default value instead of or in addition to 160ms

This means RAN1 has agreed to extend the default SSB dectetion periodicity at least 160ms. Since current RRC signalling can support the SSB periodicity value of 160ms, there is no much impact on RRC signalling. In this contribution, we will further provide our views on on RAN2 aspects of DL coverage enhancements (e.g. cell level / beam level DTX/DRX mechanism, etc.).

- Discussion

2.1 Design of periodic cell DTX/DRX pattern for DL Coverage enhancement

According to the WID objective, Rel-18 network energy saving techniques are expected to be reused as baseline for DL system level enhancement:

- Rel-18 network energy saving techniques should be considered as baseline in the system level study

As specified in TS 38.300, current cell DTX/DRX patterns in Rel-18 NES is common for the UEs configured with this feature in the TN cell. For Rel-18 NES, a serving cell can be configured with either DTX or DTX+DRX. For cell DTX/DRX, each Serving Cell may be configured by RRC with a periodic cell pattern with only two status, i.e., Active and Inactive Periods.

For the NTN cell, according to the agreed assumption in RAN1, a satellite may have large satellite footprint and limited feeder link bandwidth, especially in Set1-2 in RAN1 agreement. For Set1-2, the number of active beams is just 16, which is constrained by satellite payload capability, it is not be able to provide NR service for all beam footprints, the total number of the beam footprints is 1058. As shown in the table, without system level coverage enhancement, the effective coverage providing communication service/common signaling can only be transmitted for 1.5% of beam footprints within a satellite footprint.

Figure 1 Two scenarios of the satellite target coverage area in NTN system

As shown in Figure1, there are mainly 2 options which can be considered for the mapping of NR beams and NR cells to one satellite beams:

- Scenario 1: Different PCIs for mutiple beam footprints in one satellite cell (1-to-1 mapping);

- Scenario 2: Single PCI for Multi-beam footprints in one satellite cell (multiple-to-1 mapping)

Regarding Scenario 1, which might be considered as a baseline consist in one-to-one mapping of one beam (similar to one Terrestrial-network cell) to Satellite cell. In option 1, the beam hopping can be modelled as cell switching. Hence, NR Beam management operation is not needed.

Moreover, as indicated in RAN1 LS as follows:

- Beam footprint status in terms of "off", "common messages only" and "active traffic"  in one of the RAN1 agreements is only defined for the sake of system-level evaluation methodology.

This means there is no term of N1, N2, N3 beam introduced in the spec. And a UE in a satellite cell is expected to be transited among only the two status for beam footprints, where beam status is to switch among ON and deactive during the beam hopping procedure. Hnece,  the current periodic cell DTX/DRX pattern (i.e. active and non-active periods) for reducing gNB downlink transmission/uplink reception active time seems can be utilized for the DL system level enhancement (i.e., beam hopping ) use case.

Proposal 1: for senario1, i.e. Different PCIs for mutiple beam footprints in one satellite cell (1-to-1 mapping), the current periodic cell DTX/DRX pattern (i.e. active and non-active periods) in Rel-18 NES for reducing gNB downlink transmission/uplink reception active time seems can be utilized for the DL system level enhancement (i.e., beam hopping ) use case.

Then for scenario 2, multiple NR beams per NR cell, with the same cell (PCI) is mapped to partial numbers of satellite beams (e.g., quarter beams in one satellite cell). This means the granulartiy of the current periodic cell DTX/DRX pattern can not be directly applied to this senario, since different beam footprints will obviously experience different DTX/DRX pattern in beam footprint level. Alternatively, there is still one periodic cell DTX/DRX pattern with more domain information, e.g. beam footprint or geographical information.

Proposal 2: for senario2, i.e. Single PCI for Multi-beam footprints in one satellite cell (multiple-to-1 mapping)

, the current periodic cell DTX/DRX pattern (i.e. active and non-active periods) in Rel-18 NES for reducing gNB downlink transmission/uplink reception active time seems can be enhanced for the DL system level enhancement (i.e., beam hopping ) use case:

- Alt 1．Extend the granularity of  periodic cell DTX/DRX pattern in NES from cell to beam footprint

- Alt 2．Reuse current periodic cell DTX/DRX pattern with more domain information, e.g. beam footprint or geographical information

2.2  Definition of UE behavior under the non-active period of DTX/DRX pattern for DL Coverage enhancement

In addition, as for NES, no impact to idle UE, e.g., RACH, paging, SIBs and SSB transmission, it is proposed to define different UE behavior is different in the non-actvie status when reusing the current periodic cell DTX/DRX pattern (i.e. active and non-active periods) in Rel-18 NES.

Table 1  Comparison of UE behavior between DTX/DRX and Beam hopping

| Status | R19 beam hopping | R18 NES | NOTE |
| --- | --- | --- | --- |
| Active status | Regular UE behaviors | Regular UE behaviors |  |
| Cell DTX non-active status | Nothing to be received/transmitted | Not monitor PDCCH in certain cases Not monitor SPS No impact to idle UE, e.g., RACH, paging, SIBs and SSB transmission | As for NES, no impact to idle UE, e.g., RACH, paging, SIBs and SSB transmission; the UE behavior is different in the non-actvie status. |
| Cell DRX non-active status | Nothing to be received/transmitted | Not transmit on CG resources No SR transmission No periodic/semi-persistent CSI reporting No periodic/semi-persistent SRS No impact to idle UE, e.g., RACH, paging, SIBs and SSB transmission |  |

Proposal 3: As for NES, no impact to idle UE, e.g., RACH, paging, SIBs and SSB transmission, it is proposed to define different UE behavior is different in the non-actvie status when reusing the current periodic cell DTX/DRX pattern (i.e. active and non-active periods) in Rel-18 NES.

- Summary

Proposal 1: for senario1, i.e. Different PCIs for mutiple beam footprints in one satellite cell (1-to-1 mapping), the current periodic cell DTX/DRX pattern (i.e. active and non-active periods) in Rel-18 NES for reducing gNB downlink transmission/uplink reception active time seems can be utilized for the DL system level enhancement (i.e., beam hopping ) use case.

- Proposal 2: for senario2, i.e. Single PCI for Multi-beam footprints in one satellite cell (multiple-to-1 mapping)

, the current periodic cell DTX/DRX pattern (i.e. active and non-active periods) in Rel-18 NES for reducing gNB downlink transmission/uplink reception active time seems can be enhanced for the DL system level enhancement (i.e., beam hopping ) use case:

- Alt 1．Extend the granularity of  periodic cell DTX/DRX pattern in NES from cell to beam footprint

- Alt 2．Reuse current periodic cell DTX/DRX pattern with more domain information, e.g. beam footprint or geographical information

Proposal 3: As for NES, no impact to idle UE, e.g., RACH, paging, SIBs and SSB transmission, it is proposed to define different UE behavior is different in the non-actvie status when reusing the current periodic cell DTX/DRX pattern (i.e. active and non-active periods) in Rel-18 NES.

- References

- Chairman's Notes RAN1#116

- R2-2407919 Reply LS on DL coverage enhancements