# 3GPP TSG-WG2 Meeting #128                                             R2-2410673

Orlando, USA, 18th – 22nd November 2024

Source: 			CMCC

Title:  		    Discussion on LCM for positioning

Agenda item:	8.1.2.3

Document for: 	Discussion

Introduction

With the RAN2#127bis meeting, RAN2 discussed general procedures for LCM for UE sided model for AI positioning case 1. The following agreements have been reached.

1: The following procedures for LCM for UE sided model for AI positioning case 1 is the baseline: Step 1: LMF may request the UE to report the supported functionalities at the UE side by LPP request capabilities message. Step 2: UE sends LPP provide capabilities message to LMF with the supported functionalities at the UE side. Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition). Step 4: UE reports the applicable functionality to the LMF by the LPP provide capabilities message. Step 5: The LMF requests the inferred location information using the LPP request location information message. Step 6: UE reports the inferred location using LPP provide location information message. 2: Whether the inference configuration is provided in step 3 or/and step 5 is FFS (to be revised based on RAN1 progress). 3: Whether network side additional condition is needed and what it contains is FFS (to be revised based on RAN1 progress). 4: FFS whether LMF controls the UE sending unsolicited LPP provide capabilities (i.e. whether step4 is sent reactively or proactively). FFS the signalling details. 5: RAN2 will decide whether AI positioning will be a new method after further details from RAN1 are received.

In the contribution, we will focus on the leftover issues on LCM for positioning use case.

Discussion

In the RAN2#125Bis meeting, it has concluded that both proactive and reactive reporting are applicable to positioning use case.

As shown in Fig.1, the UE reports the applicable functionalities in step 4. The applicable functionalities are determined based on the assistance data information from both network side additional conditions and UE supported functionalities. Step 4 can be response of step 3, as reactive reporting, if it is the initial reporting after UE receive the network side additional condition. At the same time, the step 4 can be independent, as proactive reporting, if UE size condition is changed. In our understanding, proactive applicable functionality reporting via LPP ProvideCapabilities message can be supported.

Fig.1 Reactive applicable functionality reporting        Fig.2 Proactive applicable functionality reporting

Proposal 1: Proactive applicable functionality reporting via LPP ProvideCapabilities message can be supported.

With RAN1#116bis and RAN1#117, we have the following work assumptions and agreement:

Work assumptions: For training data generation of AI/ML based positioning Case 1/2a/2b/3b, the label and its related data (e.g., time stamp) can be generated by:

- PRU

- Non-PRU UE with estimated location

- LMF

Note: transfer of label and its related data is out of RAN1 scope.

For the case that the ground truth label is generated by LMF, LMF generates the ground truth label based on some measurement transmitted by UE and TRP’s location coordinate. LMF provides the corresponding ground truth label and its related data (e.g. time stamp) to UE for model training. The existing signaling ProvideAssistanceData can be reused.

Proposal 2: At least ground truth label and its related data (e.g. time stamp) should be provided by LMF via signaling ProvideAssistanceData.

As per RAN1 progress in last meeting, for both UE initiated and LMF initiated case, the DL PRS configuration and existing assistance data IE of UE-based DL-TDOA and/or UE-based DL-AoD are needed to be provided by LMF to UE.

For training data collection of Case 1, in terms of DL PRS configuration for collecting training data, both options are feasible by using legacy mechanisms: Option A. (UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE. Option B. (LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE. Note: the UE can be a PRU and/or a Non-PRU UE. Note: as in existing specification, the DL PRS configurations in the assistance data from LMF to UE are based on DL PRS configuration coordinated between LMF and gNB.

For AI/ML positioning Case 1, regarding the assistance data provided from LMF to UE, for ensuring consistency between training and inference, - for each of the existing assistance data IE of UE-based DL-TDOA and/or UE-based DL-AoD, study whether it should be: (a) explicitly indicated, (b) implicitly indicated and/or (c) other; - Companies can provide inputs on further enhancements of existing assistance data, including new information - Note: this does not mean that training and inference phases are mentioned in assistance data. Table. Existing assistance data (supported up to Rel-18) that may be transferred from LMF to UE in UE-based DL-TDOA [1] or UE-based DL-AoD [2], as applicable.

Proposal 3: For data collection for UE sided POS model training, at least DL PRS configuration, existing assistance data IE of UE-based DL-TDOA and/or UE-based DL-AoD need to be provided by LMF to UE.

Conclusion

According to the above discussion, the following observations and proposals are given:

Proposal 1: Proactive applicable functionality reporting via LPP ProvideCapabilities message can be supported.

Proposal 2: At least ground truth label and its related data (e.g. time stamp) should be provided by LMF via signaling ProvideAssistanceData.

Proposal 3: For data collection for UE sided POS model training, at least DL PRS configuration, existing assistance data IE of UE-based DL-TDOA and/or UE-based DL-AoD need to be provided by LMF to UE.

Reference

[1] RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR

[2] Table 8.12.2.1.0-1 in 38.305, Use equipment (UE) positioning in NG-RAN (Release 18), v18.3.0

[3] Table 8.11.2.1.0-1 in 38.305, Use equipment (UE) positioning in NG-RAN (Release 18), v18.3.0