# 3GPP TSG-WG2 Meeting #128                                            R2-24010674

Orlando, USA, 18th – 22nd November 2024

Source: 			CMCC, ZTE, Media Tek Inc., vivo, CATT

Title:  		    Discussion on UE aggregation enhancement

Agenda item:	8.18

Document for: 	Discussion

Introduction

R17 SL relay is mainly focus on single path, where remote UE is connected to network via SL path. R18 SL relay enhancement extends the scenario to multi-path, one direct path (uu link) and one indirect path (SL or Non-3gpp link). The direct link and indirect path are limited to same gNB.

For TEI 19, we suggest considering some enhancements to multi-path to meet the requirements.

Discussion

3GPP RAN approved a work item “NR SL relay enhancements” in R18 to address the enhancements and solutions required for supporting the UE-to-network Relay. The inclusion of multi-path with relay in R18, where a remote UE is connected to network via direct and indirect paths, holds the potential to enhance the reliability, robustness and throughput. This multi-path relay solutions are almost be utilized to for UE aggregation, a scenario where a UE is connected to the network via direct path and via another UE using a non 3gpp connection. UE aggregation is designed to cater applications requiring high UL bitrates on 5G terminals, in cases when normal UEs are too limited by UL UE transmission power to achieve required bitrate, especially at the edge of a cell. Moreover, UE aggregation can improve the reliability, stability and reduce delay of services as well, that is, if the channel condition of a terminal is deteriorating, another terminal can be used to make up for the traffic performance unsteadiness caused by channel condition variation.

However, due to time constraints, R18 only supports a single indirect path for UE aggregation, which may limit the full potential of multi-path relay solutions in enhancing network performance and service quality. The system gain from a single indirect path in UE aggregation is also constrained by the uplink transmission capability of the relay UE, as well as by mobility and radio link conditions.

In application scenarios that demand UE aggregation, such as large-scale live broadcasting, there is a significant need for high uplink transmission rates and reliability. This makes it particularly important to increase the number of assisting relay UEs. For instance, in scenarios requiring ultra-long-range coverage, such as maritime fishing operations or offshore oil extraction platforms, terminals would ideally need to emit a transmission power of approximately 33dBm to cover the vast distances. To achieve this with typical terminals that have a transmission power of 23dBm, it would be necessary to aggregate signals from ten such terminals.

This aggregation is crucial for meeting the high demands of uplink bitrates and ensuring the reliability of services, especially in environments where the transmission power of individual UEs may not be sufficient to achieve the required performance. By increasing the number of assisting relay UEs, the system can better accommodate the needs of high-data-rate applications and improve the overall robustness of the network.

Observation 1: One indirect path in UE aggregation cannot meet the requirements of some use cases.

Proposal 1: RAN2 is suggested to support multiple indirect paths for multi-path relay via N3C.

Conclusion

According to the above discussion, the following proposal is given:

Observation 1: One indirect path in UE aggregation cannot meet the requirements of some use cases.

Proposal 1: RAN2 is suggested to support multiple indirect paths for multi-path relay via N3C.