3GPP TSG RAN WG2 #128		R2-2410678

Orlando, USA, November 18th – August 22nd, 2024

Agenda Item:	8.3.2.1

Source:	Indian Institute of Technology Madras (IITM), IIT Kanpur

Title:	 Simulation and evaluation of RRM measurement prediction

Document for:	Discussion

Introduction

At the RAN 102 meeting, a new SID [1] on “Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR” was approved. The primary objectives the SID are as follows,

- RRM measurement prediction

- Measurement event prediction

- RLF HO failure prediction

At the RAN2 #125 bis meeting it was agreed to study the following cases for RRM measurement prediction

      - Case 1: To predict beam level results, then generate cell level results based on the predicted beam results

      - Case 2: To directly predict cell level results based on cell level results

      - Case 3: To directly predict cell level results based on beam level results

Further, it was agreed in RAN2 #125 bis [2] and RAN2 #126 [3] to study the implementation of the above cases in spatial domain, temporal domain and frequency domains. In continuation to the agreements in the prior meetings, several simulation assumptions were also agreed for the development of AI/ML models for RRM predictions and the following study goals were fixed. This document focuses on case 4 highlighted in Table 1.

Table 1. RRM prediction subcases and priorities

| Case number | Prioritization | Evaluation scenario combination | Target study goal |
| --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 2nd goal |

Based on above-mentioned agreements and simulation assumptions, this document focuses on the development of AI/ML models for RRM prediction - specifically for FR2 intra frequency intra cell prediction in the temporal domain.

FR2 to FR2 intra-frequency temporal domain case A:

As shown in Figure 1, in the intra-frequency temporal domain Case A, (X times Tper) measurements (RSRPs with dimension N_Tx*N_Rx) are given as inputs and we need to predict (Y times of Tper) highest RSRP for each Tper.

Figure 1. Intra-frequency Intra-cell temporal domain Case A

Generation of dataset

This section details the process for generating mobile UE trajectories for RRM prediction as well as the simulation assumptions used in data generation for training/testing of ML methods.

The following are the characteristics of the data used in our simulations:

            - UEs move on random tracks in a two-tier layout as shown in Figure 2.

            - We define a track length, which is divided into segments. The segments are generated based on minimum length of 20, mean length of 40, standard deviation (of length) of 10, curve radius of 20 and turn probability of 0.7

            - Channel measurements are captured at regular snapshots based on the UE speed an a CSI-RS periodicity of 20ms

            - If the UE encounters the boundary it’s track is terminated and a new UE is dropped at random position.

            - At each snapshot, we measure the RSRP values [4] at the UE’s position from the serving cell and all neighbouring cells. These RSRPs form the model input.

            - All the simulation assumptions are illustrated in Table 2 and other parameters are as per the outcome of the email discussion [5]

Figure 2: Simulated hexagonal 2-tier layout

Table 2: Simulation assumptions

| Parameter | Value |  |
| --- | --- | --- |
| SCS |  | 60 kHz |
| Number of resource elements |  | 66 * 12 |
| UE track length |  | 2000 m |
| BW |  | 50 MHz |
| ISD |  | 200 m |
| Center frequency |  | 30 GHz |
| Scenario |  | UMi |
| Layout |  | 2 tier hexagonal cell |
| Number of Tx (N_Tx) |  | 32 |
| Tx antenna height |  | 10 m |
| Number of Rx (N_Rx) |  | 4 |
| Rx antenna height |  | 1.5 m |
| BS Tx power |  | 30 dBm for 20 RB bandwidth |
| UE speed |  | 60 kmph |
| UE rotation |  | No |

Model Assumptions:

We used an LSTM (Long Short-Term Memory) based architecture for RRM measurement prediction. Details are given below.

Table 2: Model architecture and other experimental parameters

| Assumptions | Description |  |
| --- | --- | --- |
| Data | Number of beams in set A | 32 * 4 |
|  | Number of beams in set B | 32 * 4 (same as Set A) |
|  | Experimental sampling period (Tper) | 80 ms |
|  | Observation window (Ot) | 240 ms, 400 ms, 400 ms |
|  | Prediction window (Pt) | 240 ms, 480 ms, 800 ms |
| AI/ML model | Model input | 3*32*4, 5*32*4, 8*32*4. (3, 5 and 8 obtained from Ot/Tper) |
|  | Model output | 1*3, 1*6, 1*10 (The RSRP values) |
|  | Model | LSTM model |
|  | Hidden size (H) | 64 |
|  | Number of LSTM layers | 2 |
|  | Output layer | Fully connected layer H * (Pt/Tper) |
| Data size | Training | 0.2 M |
|  | Testing | 0.04 M |

Figure 3: AI/ML model implementation for RRM measurement prediction for FR2 intra cell intra frequency temporal domain.

RRM prediction Results for FR2 Intra Frequency Intra Cell Temporal Case A

Once the RSRPs are predicted using the model in Figure 3, the difference between the predicted RSRP and the true RSRP is calculated. Then, % of samples within different dB ranges of difference are calculated and plotted in Figure 4. From Figure 4 it can be observed that for the same observation window, the accuracy varies for different prediction windows. For instance, for fixed Ot  of 5Tper the prediction accuracy decreases when the Pt is increased (from 6 Tper and 10 Tper). Therefore  Pt needs to be chosen carefully.

Figure 4. CDF of the difference between the predicted and true RSRPs (Predicted – True) at different dB ranges of difference. Prediction window is varied and observation window is fixed.

Figure 5. CDF of the difference between the predicted and true RSRPs (Predicted – True) at different dB ranges of difference. Observation window is varied and prediction window is fixed.

Observation 1: The accuracy of the ML model decreases for higher lengths of prediction window, for a given observation window.

From Figure 5, it can be observed that for the same prediction window size, the accuracy of the model decreases when the observation window size is decreased. Basically this follows from the fact that the accuracy of timeseries prediction with ML models changes when the history of samples to the model is reduced.

Observation 2: The accuracy of the ML model is low for small observation window sizes and high for large observation window

From Figures 4 and 5 it can be seen that the ML models are able to predict the future measurements for FR2 intra frequency temporal domain case A with decent accuracy. However the accuracy depends on the parameters such as Ot and Pt. At the same time, based on our previous experiments in the spatial domain in [6], we believe that the model complexity also changes the prediction accuracy in the temporal domain. However, it needs to tested experimentally.

Observation 3: The performance of ML-based RRM measurement prediction for mobility in FR2 intra frequency temporal domain case A depends on Ot, Pt and model complexity.

References

[1] RP-234055 Study on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR, FS_NR_AIML_Mob

[2] Chair’s Notes, RAN2 #125 bis, April 2024

[3] Chair’s Notes, RAN2 #126, May 2024

[4] 3GPP TS 38.215

[5] RAN2 #126 Post Email Discussion: R2-2405960 Draft summary of [AT126][030][AIMob] Simulation assumptions

[6] R2-2407451 RAN2 #127 August 2024 Simulation results for RRM measurement prediction