3GPP RAN WG2 Meeting #128	R2-2410680

# Orlando, USA, Nov 18th – 22nd, 2024

Agenda Item:	8.2.3

Source:	HONOR

Title:	Discussion on A-IoT Paging

Document for:	Discussion and Decision

1. Introduction

In RAN2#127bis, the following agreements were agreed [1]:

Agreements 1 The A-IoT paging message can include information to avoid duplicate response from the device to a reader. This information should be short and simple. FFS how to indicate. Wait for further information and requirements from other WGs and make this decision in normative phase. 2 Based on this information the device determines whether to skip sending the response to paging. 3 From RAN2 perspective it is feasible to support paging multiple device IDs, however depending on TB size it may not be possible to include multiple device IDs. RAN2 will determine the need for this after considering aspects related to RAN2 multiplexing and other WGs. 4 If multiple device IDs in single paging is supported, if A-IoT paging message contains multiple device IDs, reader can configure either contention free RA or contention-based RA 5 For all use cases, device determines the random access type from the paging message. FFS whether it is explicitly or implicitly. FFS whether we will down selection for all RA types (2step, 3step, CFRA) or try to unify the design for 2step, 3step

In this contribution, we focus on the remaining issues of A-IoT paging.

2 Discussion

2.1 A-IoT paging procedure

In the RFID, the select and access procedure is as following[2]:

Interrogator (reader-like entity) sends Select command to inventory, and tags initialize its slot counter when receives Query command. Tags decrement their slot counter every time they receive a QueryRep with matching session and backscatter an RN16 when their slot counter reaches 0000h. Similar to RFID procedure, we need to discuss whether the Query and QueryRep are needed in the paging and access procedure.

During the last meeting, it was agreed that reader could trigger multiple/subsequent A-IoT paging messages that are associated with the same request from the CN. The mainly reason to send subsequent A-IoT paging messages is to resolve the conflict or to give the unavailable devices other opportunities to access. Since the subsequent A-IoT paging messages could realize the functions of Round Trigger message, the Round Trigger message is needed only when we identify some specific demands. There are two methods provided by RAN1 for devices to synchronize with the reader, one is time-based method and the other is signalling-based method. For the signalling-based method, the slot-start indication is required. For the time-based method, a timer will be maintained in both the device and reader sides. However, it is up to RAN1 to decide which method to be adopted. If the signalling-based method is adopted, the entire paging and random access procedure is as following, the re-paging message following the initial paging message is associated with the same CN request:

Proposal 1: A-IoT Paging message can be used to start the round which contains multiple RA slots.

2.2 Multiple/Subsequent A-IoT Paging

In the last meeting, it was agreed that the A-IoT paging message can include information to avoid duplicate response from the device to a reader. For the same service request from CN, the reader can page multiple times to detect as many devices as possible to avoid devices not being paged due to insufficient power or random access failure. If the number of target devices is visible in the reader, the reader can perform multiple/subsequent paging based on the number of received device IDs. Otherwise, the reader needs to determine whether to send subsequent paging message based on the detection of random access failure. In addition, after receiving the device IDs from the reader, the CN may instruct the reader to page again if the number of acquired device IDs is not enough.

Proposal 2: The subsequent paging is initiated by the reader or the CN.

Proposal 3: For the case of CN initiation, the CN should indicate to reader the subsequent paging is for the same CN service request.

If the IDs of target devices are visible in the reader, the reader can provide the target ID of device that needs to respond to the subsequent paging message. In this case, the reader includes IDs of target devices which are not responded to the pervious paging message or which are unsuccessful in the RA of the pervious paging. The device is only required to respond if the paging message includes its own device ID. Therefore, the device IDs in paging message can avoid duplicate response from the device to a reader.

Observation 1: If the IDs of target devices are visible to the reader, the duplicate response from the device to a reader can be avoided.

For the case of the device IDs are not visible in the reader, e.g., group IDs or without IDs, the reader cannot page the specific device to respond. In this case, it is necessary to include an ID associated with CN service request in the paging message to help the device to avoid duplicate response. If the device has never received the same ID in a previous paging message, the device will respond to the paging message.

Proposal 4: If the IDs of target devices are not visible to the reader, the reader includes the ID associated with CN service request in the paging message to avoid duplicate response from the device.

Proposal 4.1: If the device receives a paging message that includes an ID associated with the CN service request, and this ID has not been received in the previous paging message, the device will respond to the paging message.

If the device has received the same ID associated with CN service request included in paging message, there are the following scenarios for the device to determine whether to respond subsequent paging message:

- Case 1: the device can determine whether the RA is successful.

- Case 2: the device cannot determine whether the RA is successful.

For the case 1, the reader may send an indication for the device to determine whether the random access procedure is successful. If the device receives a paging message including an ID related to the CN service request and has received the same ID in a previous paging message, the device does not need to respond to the paging message if the device is able to determine that the RA for responding to the previous paging message was successful. This means the device has completed this CN service request. If the device is able to determine that the RA of previous paging message was unsuccessful, it means the device has not yet completed this CN service request. Thus, the device needs to respond to the paging message to continue and complete the service request.

Proposal 4.2: If the device receives a paging message that includes an ID associated with the CN service request, and this ID has been received in the previous paging message, the device responds to the paging message if the RA for responding the pervious paging message was unsuccessful.

For the case 2, the reader may control the device to reply instead of letting the device decides whether to reply. In this case, the reader needs to provide some information in the paging message to help the devices that have responded to the previous paging message determine whether to reply again. The information can be the unsuccessful access occasions in the RA procedure for responding the previous paging message.

Proposal 4.3: If the device cannot decide the RA is unsuccessful, the reader includes the information in the subsequent paging message to instruct the device which has responded to the previous paging message needs to respond again.

2.3 Resources related information in paging message

For inventory or DL command procedure, the reader needs to send A-IoT paging message to find devices. After receiving A-IoT paging message, device may send random ID or device ID to the reader. Since RAN1 has agreed to use FDM for D2R message, there may be multiple frequency resources in one access slot. Unlike RFID, dedicated resources could be multiple access occasions in one slot. Thus, the A-IoT paging message can include scheduling information to indicate frequency/time resources for device(s) to respond. To support CBRA and CFRA, the resource information included in A-IoT paging message can be shared resources or dedicated resources. Since RAN1 is discussing the design of the resources, RAN2 can further discuss the parameters included in A-IoT paging message after receiving RAN1 conclusions on resources.

Proposal 5: A-IoT paging message includes dedicated resources or shared resources for device(s) to initiate RA procedure. Detailed resource-related parameters depend on RAN1.

In the last meeting, it was assumed that RAN2 considers it is feasible to support paging multiple device IDs, however depending on TB size it may not be possible to include multiple device IDs. The SA2 provides the maximum size of message is 1000 bits and the size of the device ID is about 90 bits. Excluding the control information needed to construct the paging message, it is feasible to include more than one but not many device IDs in the paging message. In some cases, the short ID may be included in the paging message to find the devices. It allows the paging message to contain even more device IDs. Including more than one device ID in the paging message can make the service more efficient and reduce the service delay. Therefore, it is possible to support including multiple IDs in the paging message.

Proposal 6: RAN2 supports that a single paging message includes multiple device IDs.

If multiple device IDs in single paging is supported, the reader can configure either contention free RA or contention-based RA. Therefore, the CFRA can be applied to single and multi-device case. For one device case, dedicated resource(s) could be included in one paging message for this device. For multiple device case, multiple dedicated resources could be included in one paging message for these devices.

Proposal 7: For CFRA, dedicated resource(s) of one or more devices can be included in single A-IoT paging message.

For the case of dedicated resources for multiple devices, device needs to determine which access occasion is specified for it. Since device uses the device ID to identify itself, device ID can be used to determine the specific access occasion. For example, the configuration of an access occasion associated with a device ID is included in A-IoT paging message; device uses the device ID to determine the specific access occasion according to the association in such configuration.

Proposal 8: For CFRA, A-IoT paging message includes a mapping between each device ID and dedicated resource.

2.4 Paging message for DO-A use case

In the last meeting, the functionality aspects discussed the Do-A scenario, and the following agreements were captured:

From RAN2 perspective, at least the A-IoT paging is an aspect/part of the current design which is not sufficient for the DO-A use case.

From RAN2 perspective, DO-A use case cannot be supported with the current design discussed in the study item.

In the TS22.369, for the sensor data collection, the Ambient IoT device is associated with a sensor. Transfer of sensor data can be initiated by the Ambient IoT device, e.g., periodically or when the Ambient IoT device has power, or can be triggered by the network. Therefore, the AF may send a command that need to acquire the data by the device periodically or the device collects the data the based on the re-defined rules. If the paging message is not supported for the DO-A use case, the AF needs to periodically send the command. And The paging message is the same for this command. This will be less efficient. By supporting paging messages for DO-A use case, the reader can collect device data regularly, thus avoiding the AF to issue command every time it needs to collect the data. And it is not complex to support paging message for DO-A use case, which needs the simple enhancements: periodic paging in reader and/or add some formation in paging message for device to send the DO-A data.

Proposal 9: RAN2 supports the paging message for the DO-A use case, which requires the enhancements: periodic paging in reader, additional information in paging message for device to send the DO-A data.

3 Conclusions

A-IoT paging procedure

Proposal 1: A-IoT Paging message can be used to start the round which contains multiple RA slots.

Multiple/Subsequent A-IoT Paging

Proposal 2: The subsequent paging is initiated by the reader or the CN.

Proposal 3: For the case of CN initiation, the CN should indicate to reader the subsequent paging is for the same CN service request.

Observation 1: If the IDs of target devices are visible to the reader, the duplicate response from the device to a reader can be avoided.

Proposal 4: If the IDs of target devices are not visible to the reader, the reader includes the ID associated with CN service request in the paging message to avoid duplicate response from the device.

Proposal 4.1: If the device receives a paging message that includes an ID associated with the CN service request, and this ID has not been received in the previous paging message, the device will respond to the paging message.

Proposal 4.2: If the device receives a paging message that includes an ID associated with the CN service request, and this ID has been received in the previous paging message, the device responds to the paging message if the RA for responding the pervious paging message was unsuccessful.

Proposal 4.3: If the device cannot decide the RA is unsuccessful, the reader includes the information in the subsequent paging message to instruct the device which has responded to the previous paging message needs to respond again.

Resources related information in paging message

Proposal 5: A-IoT paging message includes dedicated resources or shared resources for device(s) to initiate RA procedure. Detailed resource-related parameters depend on RAN1.

Proposal 6: RAN2 supports that a single paging message includes multiple device IDs.

Proposal 7: For CFRA, dedicated resource(s) of one or more devices can be included in single A-IoT paging message.

Proposal 8: For CFRA, A-IoT paging message includes a mapping between each device ID and dedicated resource.

Paging message for DO-A use case

Proposal 9: RAN2 supports the paging message for the DO-A use case, which requires the enhancements: periodic paging in reader, additional information in paging message for device to send the DO-A data.

4. References

[1]	RAN2 #127bis Chairman notes

[2]    EPC™ Radio-Frequency Identity Protocols Generation-2 UHF RFID