3GPP TSG-RAN WG2 Meeting #128	R2-2410681

Orlando, USA, Nov. 18th – 22nd , 2024

Agenda Item:	8.2.4

Source:	HONOR

Title: 	Discussion on A-IoT random access

Document for:	Discussion and decision

1	Introduction

The RAN finished the SI for Ambient IoT and approved the TR 38.848 as the outcome of the SI. During the initial discussion related to the Rel-19 Ambient IoT, RAN confirmed the general scope of this SI with some common understandings and limitations related to device type, deployment scenarios and traffic types [1].

During RAN2#127 bis meeting, RAN2 discussed the A-IoT random access and made several agreements as:

Agreements for Random Access

- In case of D2R data transmission failure, device follows the reader instruction:

	it is supported to re-access in another opportunity controlled/provided by the reader (i.e. retry the random access); and

	reader can repeat the R2D “upper layer command” to trigger the device to re-send the same D2R “response” (i.e., device just follows the received R2D to transmit D2R).

2	Support optional explicit R2D failure/success feedback indication for at least MSG3 for re-access purpose.  FFS for following D2R data.

3	For 2step CBRA, if mgs2 is not received by the device, the device is not expected to autonomously re-access.  The re-access is always controlled by reader.

4	For 2step and 3step CBRA, The A-IoT device performs re-access in another opportunity provided by the reader (i.e. retry the random access) at least in case of contention resolution failure.

5	R2D message is used to provide access occasion(s) which can be used for re-access purpose. One option to be captured in the TR is that this R2D message is AIoT paging message.   FFS additional indications to differentiate if needed in paging message.    FFS if other message can be used.

6	From RAN2 perspective, at least for TDMA the device can randomly select one access occasion for A-IoT Msg1 within the access occasions provided/assigned by the reader, as the baseline for CBRA.  FFS if this is applicable to FDMA.  Further enhancement option(s) can be considered after more RAN1 progress on TDMA/FDMA.

In this contribution, we analyze the remaining issues of the A-IoT random access based on the use cases and assumptions mentioned above and provide proposals related to A-IoT random access procedure and the failure handling.

- Discussion

Ambient IoT, which is introduced as a new 3GPP IoT technology, aims to rely on ultra-low complexity devices with ultra-low power consumption for the very-low end IoT applications and address some representative use cases. AIoT random access is not the same as the NR random access or RFID access procedure since the overall device assumption and the design target are different based on the previous discussions in 3GPP working groups.

In the current NR specification [2], different kinds of random-access procedure are supported: 4-step CBRA as shown in Figure 1(a), 2-step CBRA as shown in Figure 1(b), 4-step CFRA as shown in Figure 1(c), 2-step CFRA as shown in Figure 1(d).

Figure 1: Random Access Procedures in NR

Slotted-ALOHA is employed in RFID to reduce the access collision. The general access procedure for RFID comprises of basic operations like select, inventory, access.

\

Figure 2: General access procedure for RFID

RAN2 confirmed that slotted-ALOHA is the baseline for Ambient IoT random access in the first meeting and analysed the basic steps for 3-step random access CBRA, 2-step CBRA and contention-free access.

2.1 A-IoT random access steps

3-step CBRA, as the baseline access procedure for A-IoT, is similar to the 4 step CBRA in NR and the inventory procedure in RFID. During the RAN2#126 meeting, RAN2 agreed that the first two messages were for contention resolution and the device sends the Msg3 to complete the random-access procedure. To take an example of inventory use case, we provide the general procedure for 3-step CBRA based on the current agreements.

Figure 3: General procedure of 3-step CBRA

Based on our understanding, the device will not perform re-access in the following rounds if it is not aware of whether the Msg3 is not well-received by the reader. The reader needs to trigger another procedure to find the failed device which results in more signalling overhead and longer latency to complete the service request. In addition, if the failure is informed timely, the failure handling could be performed with less latency. Firstly, the R2D transmission after Msg3 could be used to inform and handle the Msg3 transmission failure and provide some performance improvement. For example, the R2D transmission after Msg3 could confirm the failure reception of the Msg3 and the absent of it indicates the success/failure reception of Msg3. The success indication of the Msg3 could be explicitly provided by the start of the next occasion without one failure indication ahead.

Proposal 1: Present of the subsequent R2D transmission is to confirm the failure reception of the Msg3 while the absent of it before the start of the next occasion indicates the success reception of Msg3 from device’s point of view.

For 3-step CBRA, the device considers the contention resolution as successful only when the reader ack the same ID received in Msg1. Similarly, the device considers the contention resolution as failed, if the Msg2 including the same ID in Msg1 is not received before the next occasion/slot. The explicit indication could be sent from the reader to the device to inform the success/failure of the D2R message. In addition, RAN1 has studied A-IoT processing time aspects in terms of the following timing relationships:

TR2D_min: Minimum time between a R2D transmission and the corresponding D2R transmission following it. TD2R_min: Minimum time between a D2R transmission and the corresponding R2D transmission following it. TD2R_max: Maximum time between the D2R transmission and the corresponding R2D transmission following it, so that the R2D transmission timing is expected to be within [TD2R_min, TD2R_max], when a R2D transmission in response to a D2R transmission is expected for A-IoT Msg2 response to A-IoT Msg1 for the A-IoT device. See clause 6.3 for message descriptions. TR2D_R2D_min: Minimum time between two different consecutive R2D transmissions to the same A-IoT device. TD2R_D2R_min: Minimum time between two different consecutive D2R transmissions from the same A-IoT device. For the time interval between a R2D transmission and the corresponding D2R transmission following it, there are two options studied: Option 1: Define a maximum time TR2D_max between a R2D transmission and the corresponding D2R transmission following it, so that the device transmits D2R transmission within [TR2D_min, TR2D_max]. Option 2: The corresponding D2R transmission timing TR2D following a R2D transmission is determined based on the control information in the R2D transmission, where TR2D ≥ TR2D_min.

As shown in the chart, the device expects the R2D transmission corresponding to the D2R transmission ahead would be sent before the TD2R_max.

Observation: Failure could be detected from perspective of device based on the TD2R_max (maximum time between the D2R transmission and the corresponding R2D transmission following it) defined by RAN1.

In another word, from the perspective of the device, it could regard the corresponding D2R transmission as failure if the R2D transmission is not received before TD2R_max from the D2R transmission. Thus, even though the explicit indication is missed by the device, it could determine the failure based on the TD2R_max. The TD2R_max could be used to determine the failure when there supposes to be a corresponding R2D transmission after the D2R transmission, for example, Msg2 following the Msg1 in the 3-step CBRA. Considering the failure already occurs before, it is possible that the explicit indication may be missed by the device. The A-IoT processing time TD2R_max seems to be the pre-defined timer based on the UE/NW capability. In addition, the maximum time between the D2R transmission and the corresponding R2D transmission following it could be pre-configured by the reader based on its scheduling strategy or load condition. Thus, based on pre-configured/pre-defined timer, the failure detection from the device side could be the supplementary solution for the agreed explicit indication.

Proposal 2: Support the failure detection (e.g., based on pre-configured/pre-defined timer) and failure handling (e.g., higher layer repetition) from perspective of the device.

In addition, there may be contention resolution failure from perspectives of both reader and device for the following two cases: 1) more than one device selects the same access occasion; 2) the decoding failure of the received D2R message for one device. Unlike the D2R transmission failure, it is hard to locate only one device for the contention resolution failure for the first case. The default operation is already captured in the TP as that the device only determines the success of the contention resolution failure if it receives one Msg2 contains the same random number sent in its Msg1. And the re-access is not autonomously performed if the Msg2 is not received. However, it is also agreed that it is up to the reader implementation whether A-IoT Msg2 is sent in random access for 2-step CBRA. In this case, reader could skip the Msg2 even it already receives the Msg1 from the device and the device is not aware of the success of contention resolution. It requires more work to solve this problem. Thus, we propose the failure could be indicated to the device or devices in the same slot as the contention resolution failure and the re-access occasion could be assigned along with it.

Proposal 3: Support explicit R2D failure/success feedback indication for contention resolution failure.

For both D2R transmission failure and contention resolution failure, the failure may be caused by the unavailability due to energy harvesting, coverage weakness and so on. The potential way to handle this kind of failure is to send the paging whenever the device stores enough energy and starts monitoring. Based on our understanding, higher layer repetition or multiple/subsequent paging could be used to collect all the targeted devices during one round or multiple rounds.

Proposal 4: Higher layer repetition of paging or multiple/subsequent paging could be used to handle the failure caused by un-availability.

3	Conclusions

In this contribution, we discussed the issues related to the random access procedure and failure handling aspects on Ambient IoT. Based on the  discussion, the following observation and proposals are concluded:

Proposal 1: Present of the subsequent R2D transmission is to confirm the failure reception of the Msg3 while the absent of it before the start of the next occasion indicates the success reception of Msg3 from device’s point of view.

Observation: Failure could be detected from perspective of device based on the TD2R_max (Maximum time between the D2R transmission and the corresponding R2D transmission following it) defined by RAN1.

Proposal 2: Support the failure detection (e.g., based on pre-configured/pre-defined timer) and failure handling (e.g., higher layer repetition) from perspective of the device.

Proposal 3: Support explicit R2D failure/success feedback indication for contention resolution failure.

Proposal 4: Higher layer repetition of paging or multiple/subsequent paging could be used to handle the failure caused by un-availability.

4	References

- RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR.

- TR 38.300, NR and NG-RAN Overall description; Stage-2; version 18.1.0 Release 18