3GPP RAN WG2 Meeting #128	R2-2410683

# Orlando, USA, Nov 18th – 22nd, 2024

Agenda Item:	8.4.2

Source:	HONOR

Title:	Discussion on LP-WUS in RRC_IDLE/INACTIVE

Document for:	Discussion and Decision

1. Introduction

In RAN#103 meeting, the following objectives of Rel-19 LP-WUS work item were agreed [1]:

- For IDLE/INACTIVE modes (RAN1, RAN2, RAN3, RAN4) - Specify procedure and configuration of LP-WUS indicating paging triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point.

The following agreements on LP-WUS monitoring conditions and subgrouping were made [2] in RAN2#127bis meeting:

- If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s). - The LPWUS monitoring exit condition does not include MR measurements. - For CN assigned LP-WUS subgrouping, RAN2 assumes similar procedure for PEI will be used for LP-WUS subgrouping. Final design is up to SA2/CT1/RAN3 discussion. - For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping. - RAN2 inform this conclusion to SA2/CT1/RAN3. - This LS is approved in R2-2409225

In this contribution, we focus on the discussion of the following issues for LP-WUS in RRC_IDLE/INACTIVE.

- Monitoring condition of LP-WUS

- Subgrouping

2. Discussion

2.1. Monitoring condition of LP-WUS

Frequent switching between LP-WUR and MR may happen when UE moves back and forth at the edge of LP-WUS coverage. As MR transition energy is huge (15000uint ~ 40000uint) [3], ping-pong between LP-WUR and MR may have a severe impact on UE power consumption. RAN2 needs to consider how to avoid ping-pong, e.g., add a hysteresis timer in entry condition.

Proposal1: RAN2 discusses how to avoid ping-pong between LP-WUR and MR at the edge of LP-WUS coverage, e.g., add a hysteresis timer in entry condition.

The following agreement has been achieved in RAN1 118bis meeting.

- Confirm the following working assumption for iDRX: For each UE, the periodicity of LO is the same as its iDRX cycle

According to the working assumption, the following issues has been observed which is showed in the below figure.

For example, the wake-up delay supported by UE is 800ms, the I-DRX cycle configured by gNB is 640ms. When a wake-up indication received within LO1, UE cannot monitor PO1 that LO1 associated with after UE wakes up MR. It does not make sense to monitor LP-WUS in this scenario. We propose that if the wake-up delay supported by UE is larger than I-DRX cycle, UE should not monitor LP-WUS.

Proposal 2: If the wake-up delay supported by UE is larger than I-DRX cycle, UE should not monitor LP-WUS.

2.2. Subgrouping

Some companies argue that false alarm rate and power consumption could be further reduced if LP-WUS and PEI are used together for one UE. In our view, there is no need to discuss it.

Firstly, RAN1 has been discussing the maximum number of subgroups per PO supported based on false alarm rate and power consumption and has assumed that the maximum number of subgroups per PO supported in Rel-19 is 32, which is sufficient to reduce the false alarm rate.

Secondly, RAN1 defines the offset(s) of LO with PO/PF as the reference point. If LP-WUS and PEI are used together for one UE, this implies the offset(s) for LP-WUS LO is associated to PEI, which is not consistent with RAN1 agreements.

Proposal 3: RAN2 does not discuss the joint use of PEI and LP-WUS.

RAN1 has two options for the offset value(s) between an LO and a reference PO/PF. Although the conclusion is determined by RAN1, RAN2 can discuss the effects of different options.

For the offset value(s) between an LO and a reference PO/PF, consider the following options: - Definition: The gap between an LO and a PO is considered to be no less than the wake-up delay a UE supports if the gap between the end of the last LP-WUS MO the UE monitors in the LO and the start of the PO is no less than the wake-up delay. - Option 1: gNB configures a single offset value. - If the gap between an LO and the PO associated with the offset is no less than the wake-up delay a UE supports, the UE monitors the PO associated with the offset after receiving a wake-up indication in a LP-WUS. - Otherwise, - Option 1-1: the UE follows the legacy paging monitoring procedure. - Option 1-2: the UE monitors LP-WUS. If it receives a wake-up indication in a LP-WUS, it monitors the first PO after its reported wake-up delay. - Option 2: gNB configures one or multiple offset values. - For the same PO, each offset corresponds to a LO. - This does not preclude the possibility that the same LO may correspond to different POs with different offset values. - Option 2A: A UE does not expect that the gap between the LO associated with the largest offset and the corresponding PO is less than the wake-up delay the UE supports. The UE monitors the LO associated with one offset that has a gap between the LO and the corresponding PO no less than the wake-up delay. - This implies that the gNB needs to configure at least one offset value that is no less than the largest wake-up delay supported by the UEs. - FFS exactly how to choose the offset - Option 2B: - If the gap between the LO associated with the largest offset and the corresponding PO is no less than the wake-up delay a UE supports, the UE monitors the LO associated with one offset that has a gap between the LO and the PO associated with the offset no less than the wake-up delay. - FFS exactly how to choose the offset - Otherwise, - Option 2B-1: the UE follows the legacy paging monitoring procedure. - Option 2B-2: the UE monitors LP-WUS. If it receives a wake-up indication in a LP-WUS, it monitors the first PO after its reported wake-up delay. - FFS exactly how to choose the offset - FFS the UE shall monitor the LO associated with additional offset(s)

For Option2A, gNB needs to configure at least one offset value that is no less than the largest wake-up delay supported by the UEs. However, RAN2 has agreed LP-WUS related configuration for IDLE/INACTIVE state is provided via system information. The gNB cannot get the wake-up delay supported by the UEs before providing LP-WUS related configuration via system information. Therefore, Option2A seems not feasible.

For Option1-2 and Option2B-2, if the gap between the LO associated with the largest offset and the corresponding PO is less than the wake-up delay a UE supports, if UE receives a wake-up indication in a LP-WUS, it monitors the first PO after its reported wake-up delay. However, gNB cannot be aware of whether the UE is monitoring LP-WUS or not in RRC_IDLE/INACTIVE state. If dynamic PO is introduced, gNB must send two paging messages on the dynamic PO and legacy PO, then paging message load will be doubled, which is a waste of paging resource.

Proposal 4: From RAN2 point of view, Option2A Option1-2 and Option2B-2 are not feasible.

3. Conclusions

In this contribution, we discussed monitor condition of LP-WUS and Subgrouping, and the proposals are summarised as below.

Proposal1: RAN2 discusses how to avoid ping-pong between LP-WUR and MR at the edge of LP-WUS coverage, e.g., add a hysteresis timer in entry condition.

Proposal 2: If the wake-up delay supported by UE is larger than I-DRX cycle, UE should not monitor LP-WUS.

Proposal 3: RAN2 does not discuss the joint use of PEI and LP-WUS.

Proposal 4: From RAN2 point of view, Option2A Option1-2 and Option2B-2 are not feasible.

4. References

[1] RP-240801 Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

[2] Draft_RAN2_127b_Meeting_Report_v1

[3] TR 38.869 Study on low-power wake up signal and receiver for NR