3GPP RAN WG2 Meeting #128	R2-2410684

# Orlando, USA, Nov 18th – 22nd, 2024

Agenda Item:	8.7.4.2

Source:	HONOR

Title:	Discussion on DSR enhancements

Document for:	Discussion and Decision

1. Introduction

In RAN2#127, the following agreements were agreed [1]:

We do not change the definition of delay-critical data For the sake of RAN2 discussions, we use the following terms: triggering threshold, reporting threshold(s) Companies should analyse the impact of setting the triggering threshold to value lower than largest reporting threshold on DSR procedure, e.g. triggering, cancellation etc. For Rel-19 DSR, the buffered data is divided into multiple portions based on the multiple reporting time threshold levels configured for an LCG. The Rel-19 DSR indicates the following information for each portion for which BS>0: - Buffer size of data volume in each portion - Shortest remaining time among PDCP SDUs buffered in each portion. There is no need to include PSI in the enhanced DSR MAC CE. A one-bit indication may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG. FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds

In this contribution, we will focus on discussion of DSR enhancements.

2 Discussion

2.1 Triggering threshold configuration for EDSR

In the last meeting, it was discussed the following options to configure the triggering threshold:

- Option 1: the triggering threshold is larger than all reporting threshold(s);

- Option 2: the triggering threshold is configured by the NW, which can be lower than the largest reporting threshold.

Since the network can configure the triggering threshold and reporting threshold(s) according to which portions are useful, the scheduling optimization achieved in both options is the same. However, the option 2 provides more flexibility in network configuration. For example, there are the data with delay information within (0ms, 5ms], (5ms, 10ms], (10ms, 15ms], (15ms, 20ms]. However, the data with remaining time in the portion of (15ms, 20ms] is not very urgent. The value of 15ms should be used to define the delay-critical data. For the option 1 and the option 2, the network needs to limit the triggering threshold to 15ms. The option 1 can obtain the delay information within (0ms, 5ms], (5ms, 10ms], (10ms, 15ms]. Compare with the option 1, option 2 can further obtain the delay information within (15ms, 20ms] when the reporting threshold is set to 20ms. Therefore, the option 2 provides the possibility for the network to obtain more delay information of the data which needs to schedule immediately or in the future.

One concern for the option 2 is the cancelling and triggering of DSR may be affected. Since the definition of delay-critical data is not changed, the current cancelling of the pending DSR can be reused, i.e. all delay-critical data associated with the DSR has been discarded. The current triggering for the Rel-18 DSR is as following:

If an LCG is configured for delay status reporting, the MAC entity shall for each logical channel within the LCG: 1> if the smallest remaining value of the running PDCP discardTimers among all the PDCP SDUs buffered for the logical channel that have not been transmitted in any MAC PDU and have not been reported as data volume in a DSR MAC CE becomes below remainingTimeThreshold of the LCG; and - if there is no DSR pending for the logical channel: 2> trigger a DSR for the logical channel.

Since it was agreed that there is only one triggering threshold, the current triggering, e.g. smallest remaining value of the LCH becomes below remainingTimeThreshold and there is no pending DSR, can be reused for the enhanced DSR. Therefore, adopting the option2 has no effect on the triggering and cancellation of DSR.

Proposal 1: The triggering threshold for the enhanced DSR should be configured by the network, which can be lower than largest reporting threshold.

Proposal 2: The existing cancelling and triggering of Rel-18 DSR is reused for the enhanced DSR.

In the Rel-18 DSR, the triggering threshold is configured per LCG. For the same considering of the scheduling policy, such as different QoS requirements for LCGs, the triggering threshold and reporting threshold should also be configured per LCG. And with such configuration, the NW can configure which LCG is needed to be configured with reporting thresholds. It is suggested not to restrict the NW to configure reporting thresholds for all LCGs that need to report DSR.

Proposal 3: The triggering threshold and reporting threshold(s) are configured per LCG for the enhanced DSR.

Proposal 4: It is up to the NW implementation to configure which LCGs are associated with the triggering threshold and reporting threshold(s).

2.2 The triggering of EDSR and DSR

There will be LCGs associated with reporting thresholds and LCGs associated only with triggering threshold. For the subsequent DSR triggering, the scenarios can be divided into the following cases:

- Case 1: only LCG with TT (triggering threshold) meets the triggering conditions of DSR.

- Case 2: only LCG with TT and RT (reporting threshold) meets the triggering conditions of DSR.

- Case 3: LCG with TT and RT, and LCG with TT meet the triggering conditions of DSR.

There is a unified solution for the above cases that enhanced DSR is always triggered and reported. However, the LCG configured with only TT is not needed to use the enhanced DSR to report delay information. Therefore, in the triggering phase, it is not reasonable to restrict the enhanced DSR should be triggered regardless of which LCG meets the triggering condition of DSR. For the case 1 and case 2, the straightforward way is to trigger DSR for case 1 and to trigger enhanced DSR for the case 2.

Observation 1: It is straightforward to trigger the Rel-18 DSR for the case 1 and to trigger the enhanced DSR for the case 2.

For the case 3, there will be two options to trigger DSR:

Option 1: only trigger the enhanced DSR for LCGs;

Option 2: trigger the enhanced DSR for LCG with TT and RT, trigger the DSR for LCG with TT.

In the above analysis, the current cancelling of pending DSR can be reused for the enhanced DSR. That is, the enhanced DSR should be cancelled if all the delay-critical data associated with the enhanced DSR have been discarded. If the option1 is adopted, the pending enhanced DSR is not cancelled when all the delay-critical data of LCG with TT and RT have been discarded. The enhanced DSR will be used to report the delay information of LCG only with TT. However, it is not needed to use the enhanced DSR MAC CE to report delay information when only the LCG with TT has available data. For the option 2, if all the delay-critical data of LCG have been discarded, the associated pending DSR or pending enhanced DSR will be cancelled. Therefore, the option 2 is preferred because the triggering and cancelling of DSR and enhanced DSR is decoupled.

Observation 2: Triggering the enhanced DSR for the LCG with TT and RT, and triggering Rel-18 DSR for the LCG with TT for the case 3 can decouple the triggering and cancelling of Rel-18 DSR and enhanced DSR.

Proposal 5: For the case of at least one LCG configured with multiple reporting thresholds, the following triggering policies of DSR are adopted:

- Triggering the Rel-18 DSR for a LCG configured only with triggering threshold when the LCG meets the triggering condition(s) of Rel-18 DSR.

- Triggering the enhanced DSR for a LCG configured with triggering threshold and reporting threshold(s) when the LCG meets the triggering condition(s) of enhanced DSR.

2.3 The reporting of EDSR and DSR

After triggering DSR, there may pending DSR and pending enhanced DSR if the resources are available to construct the DSR. Using two DSR MAC CEs to report delay information of LCGs needs the MAC entity to construct MAC CE twice. Thus, the delay information of LCGs should be reported by one DSR MAC CE for resources saving and simple. The agreed one-bit indication allows to report one pair of delay information for a certain LCG in the enhanced DSR. And there may be some LCGs have multiple pair of delay information. Therefore, if there are a pending EDSR and a pending DSR, the enhanced DSR should be used to report the delay information of all the LCGs.

Proposal 6: When to construct the DSR, if there is a pending enhanced DSR, the enhanced DSR MAC CE will be used to report the delay information; otherwise, if there is a pending Rel-18 DSR, the DSR MAC CE will be used to report the delay information.

2.4 The format of EDSR

The one-bit indication can be used to finish reporting delay information for the associated LCG. Therefore, it is simple that the one-bit indication indicates whether a further pair of delay information is present in the enhanced DSR MAC CE for the associated LCG. The new DSR MAC CE may be as following:

The new F field indicates the presence of a further pair of delay information (i.e. the Remaining Time and Buffer Size fields) for an LCG. The F field set to 1 indicates that there is a further pair of delay information for the associated LCG. The F field set to 0 indicates that the last pair of delay information for the associated LCG is reported.

Proposal 7: The one-bit indication indicates whether a further pair of delay information (i.e. remaining time and buffer size) is present in the enhanced DSR MAC CE for the associated LCG.

Proposal 8: The one-bit indication set to 0 indicates the last pair of delay information for the associated LCG is reported.

3 Conclusions

Triggering threshold configuration

Proposal 1: The triggering threshold for the enhanced DSR should be configured by the network, which can be lower than largest reporting threshold.

Proposal 2: The existing cancelling and triggering of Rel-18 DSR is reused for the enhanced DSR.

Proposal 3: The triggering threshold and reporting threshold(s) are configured per LCG for the enhanced DSR.

Proposal 4: It is up to the NW implementation to configure which LCGs are associated with the triggering threshold and reporting threshold(s).

The triggering of EDSR and DSR

For the DSR triggering, the scenarios can be divided into the following cases:

- Case 1: only LCG with TT (triggering threshold) meets the triggering conditions of DSR.

- Case 2: only LCG with TT and RT (reporting threshold) meets the triggering conditions of DSR.

- Case 3: LCG with TT and RT, and LCG with TT meet the triggering conditions of DSR.

Observation 1: It is straightforward to trigger the Rel-18 DSR for the case 1 and to trigger the enhanced DSR for the case 2.

Observation 2: Triggering the enhanced DSR for the LCG with TT and RT, and triggering Rel-18 DSR for the LCG with TT for the case 3 can decouple the triggering and cancelling of Rel-18 DSR and enhanced DSR.

Proposal 5: For the case of at least one LCG configured with multiple reporting thresholds, the following triggering policies of DSR are adopted:

- Triggering the Rel-18 DSR for a LCG configured only with triggering threshold when the LCG meets the triggering condition(s) of Rel-18 DSR.

- Triggering the enhanced DSR for a LCG configured with triggering threshold and reporting threshold(s) when the LCG meets the triggering condition(s) of enhanced DSR.

The reporting of EDSR and DSR

Proposal 6: When to construct the DSR, if there is a pending enhanced DSR, the enhanced DSR MAC CE will be used to report the delay information; otherwise, if there is a pending Rel-18 DSR, the DSR MAC CE will be used to report the delay information.

The format of EDSR

Proposal 7: The one-bit indication indicates whether a further pair of delay information (i.e. remaining time and buffer size) is present in the enhanced DSR MAC CE for the associated LCG.

Proposal 8: The one-bit indication set to 0 indicates the last pair of delay information for the associated LCG is reported.

4. References

[1]	RAN2 #127bis Chairman notes