3GPP TSG-RAN WG2 Meeting #128	R2-2410688

Orlando, USA, Nov. 18th – 22nd , 2024

Agenda Item:	8.6.3

Source:	HONOR

Title: 	Discussion on measurement event evaluation and report

Document for:	Discussion and decision

1	Introduction

The following agreements related to measurement event evaluation and measurement report for purpose of supporting LTM were made during RAN2#127-bis meeting.

Agreements on L1 event triggered MR

- MR can be sent when the leaving condition is met, based on NW configuration.

- Event triggered periodic MR can be supported, based on NW configuration.

- For measurement resource configuration, R18 LTM CSI resource configuration is reused if possible. If CSI-RS resource only IE needs to be defined, we can revisit it in the stage 3.

- For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused if possible. We can revisit it in the stage 3 if needed.

- For association between measurement resource configuration and measurement reporting configuration, R18 LTM way is reused if possible. We can revisit it in the stage 3 if needed.

- The entire event evaluation procedure is handled by MAC based on the latest L1 measured results reported by L1.

- TTT operates only based on a timer (like TTT used in L3 event triggered MR).

- Confirms WA (Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5).

- Basic information included in MR MAC CE:

- Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id)

- Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams

- Triggered event information (e.g., ReportConfigID)

MR MAC CE can include up to N beams (FFS whether the beam should satisfy the event or not).

N is configurable by NW.

- Additional information included in MR MAC CE:

- The information and quantity of current beam, based on NW configuration.

11.  The legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE.

12.	NW can configure a dedicated SR configuration for MR MAC CE transmission.

In this contribution, we further discuss the L1 measurement event evaluation and report and provide proposals accordingly.

- Discussion

During the previous meeting, multiple events for L1 measurement like events LTM2~4 were agreed, since reports triggered by different events could provide different kind of information. Note that the LTM is to trigger the switch to the specific beam, different event could be configured to the beam considering the characteristic of the beam. Thus, the L1 measurement event is configured per beam/RS, but not per cell. In addition, to achieve different effects via different events, multiple events may be configured to one beam. Furthermore, in CHO, two conditions associated with one event could be configure to UE and the event is considered as fulfilled only when both conditions are met, which improve the robustness for the handover. However, take the latency caused by the event evaluation into account, it still needs some discussions about whether one event could be associated with two conditions could be introduced into L1 measurement event evaluation.

Proposal 1: Multiple L1 measurement events may be configured for one beam. FFS whether execution event with two conditions is introduced for conditional LTM.

RAN1 had down selected the signalling content of the UE-initiated/event-driven beam reporting with variable or fixed number of beam(s) satisfying the event or at least one of the beams satisfying the event:

Agreement On UE-initiated/event-driven beam reporting, regarding UL signaling content(s) of L1-RSRP report depending on Event-2, in a report instance, at least Option-3 is supported - Option-3: N ≥ 1 beam(s) are reported in the report instance, - At least one of N reported beam(s) should satisfy the condition of Event-2 - N is configured by gNB - FFS: candidate value of ‘N’. - FFS: RRC can enable or disable whether current beam is always reported in addition to the N beams - FFS: Option-1/1a/1b/2. - Above applies at least for the single CC case

Similarly, for UE-triggered L1 measurement report for the LTM, RAN2 needs to discuss which beam is included in the report. Beam(s) of neighbour satisfying the event is the basic option since it is useful to help the NW determine the target beam. Secondly, during the last meeting, it was agreed that the current beam could be reported based on the NW configuration. In addition, it may provide more information of the current cell if the candidate beams of the serving cell could be reported. Last but not least, beams of neighbour with at least one beam satisfying the event is another option for the signalling content. As the current L3 measurement and report mechanism, the UE measures several beams to derive the cell quality to ensure that UE could receive good service under the cell with more good beams. Even the LTM is based on beam measurement, it provides more robust if the target has multiple good beams within the cell or CU, even the good beams have not met the L1 measurement event.

Proposal 2: RAN2 to discuss the following beams to be included in the report: the beam(s) of neighbour satisfying the event, candidate beams of the serving cell, beams of neighbour with at least one beam satisfying the event.

During RAN1#118-bis meeting, they further clarified the agreement made in RAN1#118 meeting and made the following agreement.

Agreement The agreement “Rel-18 LTM CSI reporting framework is the baseline for CSI-RS based L1-measurement report by gNB scheduled measurement reporting” made in RAN#118 is further clarified for L1-RSRP as follows: - UCI format defined in Table 6.3.1.1.2-8C of TS38.212 can be used by replacing SSBRI with CRI. - Whether the L1-RSRP(s) of serving cell is always included is configurable (in line with Rel-18) - The quantization method defined in clause 5.2.1.4.3 of TS38.214 and bit width defined in Table 6.3.1.1.2-6 of TS38.212 can be used - No L1 specified filtering for time and spatial domain is introduced - No enhancement on how to report L cells x M beams - Periodic reporting on PUCCH is supported - FFS: semi-persistent reporting on PUCCH/PUSCH, and aperiodic reporting on PUSCH

Based on RAN1 agreement, it is observed that L1 filter may not be introduced. To take the robust into account, RAN2 assumed filtering was needed during RAN2#126 meeting.

Agreement

RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

There may be rapid variation of L1 measurement. It may lead to misunderstanding of the beam quality if the latest measurement result of the beam is reported when the event is met. Thus, if L1 specified filter is not introduced, RAN2 needs to further discuss what measurement results to report to further improve the robust.

Proposal 3: RAN2 to discuss whether the reported L1 result needs to be filtered/averaged.

3	Conclusions

In this contribution, we discussed the L1 measurement enhancement for Rel-19 mobility WI. Based on the discussion, the following observation and proposals are concluded:

Proposal 1: Multiple L1 measurement events may be configured for one beam. FFS whether execution event with two conditions is introduced for conditional LTM.

Proposal 2: RAN2 to discuss the following beams to be included in the report: the beam(s) of neighbour satisfying the event, candidate beams of the serving cell, beams of neighbour with at least one beam satisfying the event.

Proposal 3: RAN2 to discuss whether the reported L1 result needs to be filtered/averaged.

4	References

- RP-240299, Revised Work Item: NR mobility enhancements Phase 4, Rel-19,