3GPP TSG-RAN WG2 Meeting #128	R2-2410690

Orlando, USA, Nov 18th–22nd, 2024

Agenda Item:	8.6.2

Source:	HONOR

Title: 	Further discussion on inter-CU LTM

Document for:	Discussion and Decision

1	Introduction

In Rel-19 Mobility enhancements WI, the following objective was achieved:

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured. - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged. - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released. - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling. - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support

In RAN2#127bis, the following agreements were made:

- The Rel18 handling on failure is reused in R19 if the UE selects an intra-CU LTM candidate cell after intra-CU LTM failure; for other cases, e.g. inter-CU LTM failure, the failure handling is FFS (related to SA3’s inputs). - For non-DC case, if the new Rel-19 IDs of the serving cell and the target cell have same values, the UE compares the ltm-ServingCellNoResetID and ltm-NoResetID and performs the corresponding L2 reset operation as defined in Rel-18. - The SCPAC-similar security update configuration is introduced for inter-CU SCG LTM, i.e. similar to IEs sk-CounterConfiguration, servingSecurityCellSetId and securityCellSetId. The names of the new IEs are to be discussed in stage3. - Regarding the candidate and reference configuration generation and signaling design, the following SCPAC-similar principles can be applied for inter-CU SCG LTM as baseline: - The reference configuration for inter-CU SCG LTM at least include SCG part, FFS on MCG part. - FFS: Network ensures that when UE combines the reference and candidate configuration for inter-CU SCG LTM, the configuration generated by UE must contain both MCG and SCG part configurations. - The candidate configuration and reference configuration are modeled as an MN RRCReconfiguration message. - Upon inter-CU SCG LTM, the UE performs reconfiguration with sync towards SCG, but the reconfiguration with sync in MCG is not allowed. - The MN generates the MCG part of the reference configuration (if any), while the SN (source or candidate) generates the SCG part of the reference configuration. - The MN is responsible for the reference configuration generation for SN initiated inter-CU SCG LTM. It can be up to the NW implementation whether to include the MCG part. - The MN can request an SCG reference configuration from any of the involved SNs. - For SN initiated inter-CU SCG LTM, the candidate SN provides the SCG part configuration of each candidate PSCell, and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN. - The source SN is responsible to generate the common CSI resource configuration for L1 measurement on candidate SCG cells. - The MN sends the received L1 RS configuration, early UL sync configuration, or TCI-state configuration of candidate cells to the source SN. And the source SN responds with the common CSI resource configuration to the MN. - In order to support subsequent inter-CU SCG LTM, the MN needs to transfer, during the LTM preparation phase, the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s). Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN. - Upon execution of inter-SN SCG LTM, the UE sends an MN RRCReconfigurationComplete message to the MN, which includes an SN RRCReconfigurationComplete message. - Re-use legacy LTM Cell Switch Command MAC CE for inter-SN LTM. - RAN2 confirms to support coexistence of following cases, it is up to network implementation to ensure simultaneous execution for both MCG and SCG will not happen: - Inter-MN LTM and intra-SN LTM - Inter-SN LTM and intra-MN LTM

In this contribution, we discuss the remaining issues in inter-CU LTM with non-DC and DC scenarios.

2. Discussion

2.1 Inter-CU LTM with non-DC

2.1.1 Early UL sync

During UL synchronization with LTM candidate cell(s), after performing TA acquisition based on CFRA, the source-DU acquires TA via the interaction between CU-CU and CU-DU nodes. In detail, the candidate DU notifies the associated CU of the acquired TA. The candidate CU forwards the TA to the source CU. Finally, the source DU acquires the TA from the source CU. Therefore, it is naturally for source DU to manage TA validity. If not, while the TA is about to be used, the source DU has to acquire TA from source CU, which will have negative impacts on cell switch latency.

Upon cell switch, if the TA associated with candidate cell can continue to be maintained by NW side, the signalling overhead might be reduced, since it might avoid re-acquire TA in the following period of time. To achieve this purpose, the source CU forwards TA information to the target CU, then, the target DU acquires the TA from the target CU.

Proposal 1: After cell switch, the TA associated with candidate cell(s) can continue to be kept in the following serving DU.

2.1.2 Fast LTM recovery

In the current specification, fast LTM recovery is applied in intra-CU LTM and described as follows:

When LTM execution attempt triggered by LTM cell switch command MAC CE fails or HO fails, the UE performs cell selection and if the selected cell is an LTM candidate cell and if network configured the UE to try LTM after handover/LTM execution failure, then the UE attempts RACH-based LTM execution once, otherwise re-establishment is performed.

But for inter-CU LTM, since security update will be involved, whether the legacy fast LTM recovery mechanism can be applied for inter-CU LTM depends on the security update approach. If the approach that security update automatedly by UE is adopted, there is no problem in supporting fast LTM recovery for inter-CU LTM. If not, it is not certain on whether it can be supported. In addition, security update approach shall be concluded by SA3. We can wait for SA3 input on security update.

Proposal 2: RAN2 to postpone the discussion on fast LTM recovery discussion until receiving input from SA3.

2.2 Inter-CU LTM with DC

2.2.1 Inter-CU LTM of MN with SN released or unchanged

In 37.340, the procedure of inter-Master Node handover with/without Secondary Node change is specified. Since it is similar to the scenario that inter-CU LTM of MN with SN released or unchanged, the procedure of inter-Master Node handover with/without Secondary Node change can be taken as a baseline.

Proposal 3: For inter-CU LTM of MN with SN released or unchanged, take the procedure of inter-Master Node handover with/without Secondary Node change as a baseline.

In the previous meeting, it was agreed that SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration). That is, SCG configuration can be kept or released based on NW side decision. In addition, we also need to consider how to indicate UE on this decision. For the handling on SCG configuration, there are two options as follows:

Option 1: RRC pre-configuration

In the LTM preparation phase, the SCG handling indication can be configured in the process of generating candidate cell configuration. It is straightforward that the SCG handling indication is determined by candidate MN.

Observation 1: The candidate MN is responsible to generate SCG handling indication.

Option 2: cell switch MAC CE

From the perspective of the SCG indication transmission, a cell switch MAC CE can be used to indicate the operation on SCG. However, if so, either the source MN has to be responsible to determine SCG handling (e.g., released or unchanged) or the source MN needs to recognize the indication provided by candidate MN, which might be inconsistent with current considerations.

Proposal 4: It is suggested to provide the indication of SCG unchanged or released via RRC pre-configuration.

In subsequent MCG LTM, if candidate MNs with SCG released and with SCG unchanged are configured simultaneously, the issue that SCG has been released prior to MCG LTM with SCG unchanged, should be taken into consideration. For instance, the UE switches to a target MN1 with SCG released, then the UE switches to a target MN2 with SCG unchanged.

Regarding the identified issue, there are probably two main solutions. One solution is to configure a consistent SCG handling between MNs, which will have small impact on UE. However, this is based on the restriction. Alternatively, it is allowed to configure candidate MNs with SCG released and with SCG unchanged simultaneously. If SCG has been released and the target MN has configured with SCG unchanged, an SCG addition procedure needs to be triggered. After completing SCG addition procedure, the UE is able to apply SCG configuration. Both solutions can be taken into account in further study.

Proposal 5:  RAN2 to consider on the issue that SCG has been released prior to MCG LTM with SCG unchanged.

2.2.2 Inter-CU LTM of SN with MN unchanged

In the signalling procedure of L3 SN initiated SN Change, source SN initiates the SN change procedure and performs inter-CU SCG configuration. The MN includes SCG configuration generated by the target SN into its MN RRC configuration message. The UE applies the new configuration after completing SN change. It is straightforward to take L3 SN initiated SN Change as baseline.

Proposal 6: For the procedure of inter-CU LTM of SN with MN unchanged, take L3 SN initiated SN Change as a baseline.

In SN-initiated inter-SN LTM, source SN is responsible to initial early DL/UL synch and LTM execution. During UL synchronization with LTM candidate SN, after performing TA acquisition, the candidate SN transfers TA to source SN and source SN starts to maintain TA. For CU-DU split architecture, it was a straightforward decision for source SN-DU to maintain the TA validity.

Proposal 7: In inter-CU LTM of SN with MN unchanged, source SN(SN-DU) is responsible to maintain the TA validity.

3 Conclusions

In this contribution, we discussed the issues related to inter-CU LTM. Based on the discussion, the following observation and proposals are concluded:

Observation1 : The candidate MN is responsible to generate SCG handling indication.

Proposal 1: After cell switch, the TA associated with candidate cell(s) can continue to be kept in the following serving DU.

Proposal 2: RAN2 to postpone the discussion on fast LTM recovery discussion until receiving input from SA3.

Proposal 3: For inter-CU LTM of MN with SN released or unchanged, take the procedure of inter-Master Node handover with/without Secondary Node change as a baseline.

Proposal 4: It is suggested to provide the indication of SCG unchanged or released via RRC pre-configuration.

Proposal 5:  RAN2 to consider on the issue that SCG has been released prior to MCG LTM with SCG unchanged.

Proposal 6: For the procedure of inter-CU LTM of SN with MN unchanged, take L3 SN initiated SN Change as a baseline.

Proposal 7: In inter-CU LTM of SN with MN unchanged, source SN(SN-DU) is responsible to maintain the TA validity.

4 References

- RP-241515, Revised Work Item: NR mobility enhancements Phase 4.

- TS 37.340, Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2

- TS 38.401, Technical Specification Group Radio Access Network; NG-RAN; Architecture description

- S3-241898, Draft reply LS on security handling for inter-CU LTM in non-DC cases