3GPP TSG-RAN WG2 Meeting #128		                               R2-2410692

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda Item:		8.7.4.1

Source:			MediaTek Inc.

Title:				Remaining issue for LCP enhancement

Document:			Discussion and decision.

### 1	Introduction

This document discusses the LCP enhancement according to the revised objective in R19 XR WID [1].

-	Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]:

-	Specify additional Logical Channel priority handling using delay/deadline information of packets;

From last meeting, there is still one FFS point need to be discussed:

- FFS if we can still change the priority for the 2nd round to ensure fairness, but we need to consider tight timeline of LCP procedure and UE complexity. Companies can also check whether we can leave this to UE implementation

We will express our view about this FFS point.

### 2	Discussion

A quick summary from last meeting [2], we have two options to choose when LCP procedure in 2nd Round if no residual delay-critical data in the LCH.

- Option 1: Do not change priority in 2nd Round, no matter whether there is still delay-critical data in LCH or not.

- Option 2: Fallback priority in 2nd Round if no residual delay-critical data in LCH.

|  | Pros | Cons | Supporter |
| --- | --- | --- | --- |
| Option 1 | Friendly and simple for UE/Chip vendor and spec writing. | Fairness problem | Vivo, MTK, OPPO, LGE, Apple, TCL, Nokia, Spreadtrum |
| Option 2 | Better fairness | Processing delay issues (Depends on how LCP is implementation) | Lenovo, Fujitsu, QCM, Huawei, Ericsson, Nokia, CATT, Xiaomi |

Based on above table, we can see that almost half and half on both options. The argument from some supporters of option 2 is that option 2 can achieve better fairness between LCHs and the spec change is simple [3]. However, the spec change for option 1 could be much simpler than option 2, as shown below:

-- MAC spec --

##### 5.4.3.1.1	General

The Logical Channel Prioritization (LCP) procedure is applied whenever a new transmission is performed.

RRC controls the scheduling of uplink data by signalling for each logical channel per MAC entity:

-	priority where an increasing priority value indicates a lower priority level;

-	prioritisedBitRate which sets the Prioritized Bit Rate (PBR);

-	bucketSizeDuration which sets the Bucket Size Duration (BSD).

RRC additionally controls the LCP procedure by configuring mapping restrictions for each logical channel:

-	allowedSCS-List which sets the allowed Subcarrier Spacing(s) for transmission;

-	maxPUSCH-Duration which sets the maximum PUSCH duration allowed for transmission;

-	configuredGrantType1Allowed which sets whether a configured grant Type 1 can be used for transmission;

-	allowedServingCells which sets the allowed cell(s) for transmission;

-	allowedCG-List which sets the allowed configured grant(s) for transmission;

-	allowedPHY-PriorityIndex which sets the allowed PHY priority index(es) of a dynamic grant for transmission;

-	allowedHARQ-mode which sets the allowed UL HARQ mode for transmission.

The following UE variable is used for the Logical channel prioritization procedure:

-	Bj which is maintained for each logical channel j.

The MAC entity shall initialize Bj of the logical channel to zero when the logical channel is established.

For each logical channel j, the MAC entity shall:

1>	increment Bj by the product PBR × T before every instance of the LCP procedure, where T is the time elapsed since Bj was last incremented;

1>	if the value of Bj is greater than the bucket size (i.e. PBR × BSD):

2>	set Bj to the bucket size.

1>	if logical channel j has delay critical data:

2>	set priority to delayCriticalPriority.

1>	else:

2>	set priority to priority.

NOTE:	The exact moment(s) when the UE updates Bj between LCP procedures is up to UE implementation, as long as Bj is up to date at the time when a grant is processed by LCP.

--

The fairness should be achieved in the 1st round of LCP. As we can see in the MAC spec, in the 2nd round, there is wording “regardless of the value of Bj” , which means that in 2nd round operation, fairness is not the main concern, the main reason of 2nd is not to waste the radio resource, it is more like a gift from the gods.

-- MAC spec --

The MAC entity shall, when a new transmission is performed:

1>	allocate resources to the logical channels as follows:

2>	logical channels selected in clause 5.4.3.1.2 for the UL grant with Bj > 0 are allocated resources in a decreasing priority order. If the PBR of a logical channel is set to infinity, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s);

2>	decrement Bj by the total size of MAC SDUs served to logical channel j above;

2>	if any resources remain, all the logical channels selected in clause 5.4.3.1.2 are served in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first. Logical channels configured with equal priority should be served equally.

NOTE 1:	The value of Bj can be negative.

-- MAC spec --

Proposal 1: There is no need to change priority for the 2nd round of LCP procedure.

### 3	Conclusion

This document promulgated the following proposal:

Proposal 1: There is no need to change priority for the 2nd round of LCP procedure.

### 4	Reference

[1]	RP-241771, Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia (Rapporteur).

[2]	R2-2409215, Report from session on R18 MBS, R18 QoE and R19 XR, Session chair (Huawei).

[3]	R2-2408916, LCP enhancements, Ericsson.