3GPP TSG-RAN WG2 Meeting #128	R2-2410694

Orlando, USA, Nov.  18th – 22nd , 2024

Agenda item:	8.4.3

Source:	China Telecom

Title:	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE

Document for:	Discussion

- Introduction

In the RAN #102 meeting, a new work item on Low-power wake-up signal and receiver for NR (LP-WUS/WUR) was approved. According to the above objective in WID, the following was agreed in RAN2#127bis meeting:

- Working assumption

For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’.

FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available.

This paper focus on the leftover issues on the RRM measurement relaxation and offloading of RRC_IDLE/INACTIVE UE MR for serving and neighbour cells.

- Discussion

2.1 Serving cell measurement offloading

In RAN4#111 meeting, the following cases and corresponding device states were discussed.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off: FFS the condition and the details | ON |
| #2 Relaxed case a | On with relaxation measurement | Off | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |
| #4 Relaxed case c | Off | On, FFS the condition and the details | ON |
| #5 Exit case | On | On | Off |

From RAN4 point of view, case #1 is confirmed to be clearly listed in the WID. RAN4 will focus on the requirements in case 1 at first, then further discuss case #2 to #4. That means full offloading of serving cell measurement from MR to LR is supported. And RAN4 will define the measurement offloading requirements for serving cell as well as the relaxation measurement requirements for neighboring cell measurement.

In general, the serving cell relaxed measurement can be achieved in the following ways:

- Option 1: MR performs serving cell measurement with relaxed periodicity, while LR performs partial serving cell measurement

- Option 2:  MR does not perform measurement, and LR performs all the serving cell measurement

In RAN1#116 meeting, it had concluded that serving cell measurements can be offloaded to LP-WUR based on LP-SS. Hence, from the perspective of power consumption, partial or all UE serving cell RRM measurements are offloaded from MR to LP-WUR after fulfilling the RRM relaxation criteria.

Observation 1: Partial or all UE serving cell RRM measurements can be offloaded from MR to LP-WUR, when RRM relaxation condition is met.

As outlined in TR 38.869, serving cell measurements based on MR has a significant impact on UE energy consumption. This conclusion implies that the MR may cause higher power consumption than LP-WUR to perform RRM measurement in ultra-deep sleep. If a UE is operating with LR, the number of transitions from LR to MR should be minimized to obtain the power saving gain. Thus, LP-WUR performs all RRM measurements of serving cells via LP-SS (fully offloading case) can be considered as a lower power consumption solution.

Furthermore, from the perspective of standard impact, it needs to introduce an additional RRM measurement relaxation mechanism if MR-based measurement is applied. In contrast, LP-WUR as an independent radio module has less impact on the legacy cell reselection and measurement framework. Therefore, we propose that LP-WUR performs the serving cell’s RRM measurement without waking up MR if the RRM relaxation/offloading criterion is fulfilled.

Proposal 1: It is proposed that LP-WUR performs the serving cell’s RRM measurement without waking up MR if the RRM relaxation/offloading criterion is fulfilled.

2.2 MR Neighbour Cell Measurement Relaxation

In RAN2#126 meeting, it was agreed that the RRM measurement of the neighbouring cell can only be performed by MR. Currently, one issue needs to be clarified is the situation the MR performs measurement relaxation upon the LR is active. There are two cases as below:

- If LR is active, MR only needs to be in sleep mode. (Fully offloading case) If the signal quality of the serving cell is good enough and neighboring cell measurement does not need to be performed, there is no need to wake up MR.

- If LR performs service cell measurement, MR performs relaxed neighboring cell measurement.

If the entry conditions for LP-WUS monitoring are fulfilled, before performing the serving cell measurement of MR, it should determine whether the criteria for performing neighbouring cell measurement are met. Therefore, from the above cases, the RRM relaxation and RRM fully offloading should have different entry conditions when configured to operate with LP-WUS. Moreover, RRM fully offloading means stopping MR's serving cell and neighbour cell measurements, so the criteria to be met for full offloading should be more stringent than for the RRM relaxation.

Proposal 2: The RRM relaxation and RRM fully offloading should have different entry conditions when configured to operate with LP-WUS.

In the last meeting, RAN2 agreed that for neighbor cell measurement relaxation, no additional MR-based criterion will be defined over the R16 criteria. However, it is still FFS whether to reuse “low mobility” for neighbour cell measurement relaxation for UEs capable of LP-WUS. The WID does not explicitly mention mobility requirements. But it is assumed that WUR is not limited to low mobility scenarios like NB-IoT, but that WUR is supported for a normal broadband UE. Therefore, for UEs capable of LP-WUS, the mobility issues still has to be considered. In addition, “UE with low mobility” is associated with the possibility of cell reselection. When “UE with low mobility” criterion is satisfied, the UE may not need to perform cell reselection, and therefore neighbour cell measurement can be relaxed from the MR side. Thus, the legacy Rel-16 criteria for “UE with low mobility” can be reused for neighbour cell measurement relaxation.

Proposal 3: It is need to introduce of low mobility criteria for neighbour cell measurement relaxation of MR.

- Conclusions

Based on the discussion in the previous sections we propose the following:

Observation 1: Partial or all UE serving cell RRM measurements can be offloaded from MR to LP-WUR, when RRM relaxation condition is met.

Proposal 1: It is proposed that LP-WUR performs the serving cell’s RRM measurement without waking up MR if the RRM relaxation/offloading criterion is fulfilled.

Proposal 2: The RRM relaxation and RRM fully offloading should have different entry conditions when configured to operate with LP-WUS.

Proposal 3: It is need to introduce of low mobility criteria for neighbour cell measurement relaxation of MR.

- References

- RP-240801 Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

- R2-24095xx, Draft Report of 3GPP TSG RAN WG2 meeting #127bis, Hefei, China, 14 - 18 October, 2024

- R2-2313939 Report of [AT124][505][LPWUS] Update of TR 38.869 for LP-WUS WUR (vivo)